### Physics Department

### Physics 112

### Experiment No.3

### Network Analysis 1

### The Superposition Principle And Kirchoff’s Laws

### Student’s Name:  Mudar Abed Rabbo

### Student’s No.:  1041433

### Partner’s Name:  Khairallah Amous

### Partner’s No.:  1040132

### Section:  6

### Date:  2/10/2005

### Instructor:  Dr. Ziyad Izzat

Abstract:

The aim of the experiment: is to prove experimentally the superposition principle (SPP) and Kirchoff’s laws (loop theorem and junction theorem).

The method used: is by directly measuring the currents and the voltage differences through the carbon resistors using digital multi-meter.

The main result:

Kirchoff’s Results:

I1 = 3.14 mA				V1 = 3.12 volt

I2 = 1.25 mA				V2 = 2.66 volt

I3 = 1.87 mA				V3 = 8.64 volt

SPP’s Results:

I31 = 1.52 mA

I32 = 0.35 mA

Theoretically:  I1 =3.17 mA

I2 =1.29 mA

I3 =1.53 mA

I31 =1.53 mA

I32 =0.35 mA

I3 = I31 + I32

I3 = 1.88 mA

Theory:

Applying Ohm’s law and the simple parallel and series connection rules on electric networks is of no particular help, because electric networks consists of many circuit components connected in a complicated way.

Some of the laws that we can use in such cases are; Kirchoff’s law and the superposition principle.

Kirchoff’s law:

- Loop theorem: it stats that: “The algebraic sum of the voltage drops and electromotive forces (emf’s) in a closed electric circuit is always zero.” And that means that the power generated by voltage sources is totally consumed through the closed circuit.
iVi=0, 
orkεk=jIjRj, 
where we have accounted for the opposite signs of voltage drops and emf’s.

- Junction theorem: it stats that: “The algebraic sum of currents passing through any circuit junction is always zero.” Symbolically,
jIj=0 
where the currents entering a junction have opposite signs to those leaving it.

If we took the circuit shown in fig.1 as an example, we will find that applying the previous laws on it gives the following:

- There is two junctions in the circuit and applying the junction theorem both will give us the same equation:
I1+I2−I3=0

- Three circuit loops exist, but only two independent equations could be formed: 
ε1=I1R1+I3R3
ε2=I2R2+I3R3
the third equation which is from the large loop is the sum of the previous two equations. Solving these three linear equations with three unknowns is straight forward and gives the values of the currents passing through the three resistors.

The Superposition Principle (SPP):

If circuit equations are linear, then the mathematical superposition principle is applicable. And it stats that: “The response (a desired current or voltage) at any point in a linear circuit having more than one source can be obtained as the sum of the responses caused by each of the independent sources acting alone.”

For example if we want to find the current passing through the third resistor we can follow the following steps:

- Keep ε1  and replace ε2  with a short circuit as in fig.2.

- Find the current passing through R3  as a result of the presence of ε1 alone, as follows:
I1=ε1R1+R2||R3, 
and I31R3=I1−I31R2.
Thus, I31=ε1R2R1R2+R1R3+R2R3

- Keep ε2  and replace ε1  by a short as shown in fig.3 .

- Find the current passing through R3  as a result of the presence of ε2  alone as follows:
I2=ε2R2+R1||R3, 
and I32R3=I2−I32R1.
Those give: I32=ε2R1R1R2+R2R3+R1R3

- Add both currents to find the total current passing through R3.

I3=I31+I32

Data:

R1= 1      ,     R2= 2.2       ,      R3=4.7

The measurements which we took from the multi-meter (1=12 v , 2=6 v):

I1 = 3.14 mA

V1 = 3.12 volt

I2 = 1.25 mA

V2 = 2.66 volt

I3 = 1.87 mA

V3 = 8.64 volt

(SSP) :

I31 = 1.52 mA     ,     I32 = 0.35 mA

Calculations:

Following Kirchoff’s junction theorem on the circuit shown in fig.1  we get:

I1=I2+I3 …………  eq. 1-1

Using the loop theorem we get the following equations:

12=I1+4.7I3

6=4.7I3−2.2I2

Substituting by eq. 1-1 and following simple mathematical rules we get:

12=I2+I3+4.7I3…………eq. 1-2

⇓

5.7I3+I2=12×2.2

4.7I3−2.2I2=6

⇓

26.4=2.2I2+12.54I3

6=4.7I3−2.2I2

⇓

32.4=17.24I3

I3=1.88  mA

Substituting the value of I3 in eq. 1-2 we can find the value of I2:

5.7I3+I2=12

I2=1.29  mA

Using the values of currents that we found and substituting them in eq. 1-1 we can find the value of I1:

I1=3.17  mA

Now applying ohm’s law for each resistor independently we will get:

V1=R1I1			V2=R2I2			V3=R3I3

V1 = 3.17 volt		V2 = 2.8 volt 		V3 = 8.8 volt

The values for I1 , I2 , I3 (Theoretically):

I1 = 3.17 mA				V1 = 3.17 volt

I2 = 1.29 mA				V2 = 2.8 volt

I3 = 1.88 mA				V3 = 8.8 volt

Now if we look at the circuit shown in fig.2 we will find that it is a simple circuit on which we can use ohm’s law.( as shown in the theory ( SuperPostion Principle)).

ε1 = 12 volt		R1 = 1 kΩ         R3 = 4.7 kΩ

ε2 = 6 volt           	R2 = 2.2 kΩ

we get the following values:

I31=ε1R2R1R2+R1R3+R2R3=1.53 mA

I32=ε2R1R1R2+R2R3+R1R3=0.35 mA

The values for I31 , I32 (Theoretically):

I31 = 1.53 mA

I32 = 0.35 mA

Theoretically: I3 = I31 + I32

I3 = 1.88 mA

|  | Experimentally | Theoretically |
| --- | --- | --- |
| I1 | 3.14 | 3.17 |
| I2 | 1.25 | 1.29 |
| I3 | 1.87 | 1.88 |
| V1 | 3.12 | 3.17 |
| V2 | 2.66 | 2.8 |
| V3 | 8.64 | 8.8 |
| I31 | 1.52 | 1.53 |
| I32 | 0.35 | 0.35 |

Analysis of results:

As we saw previously, using SPP we found that the sum of the current passing through R3 when each source is acting alone equals the value of the current that passes through R3 when the two sources act together. And the experimental values are around the theoretical ones.

And also as we found using Kirchoff’s laws, the values that we’ve got theoretically are around the experimental values.

The two methods which we used to get I3 , have given us the same values for it.

And finally there were some errors, which were occurred because we ignored the resistance of the equipments (the wires and the multi-meter), and this affect our result.