People often find it easier to be a result of the past than a cause of

the future.

Birzeit University

Physics Department

Physics 112

Experiment No.5

CAPACITORS and INDUCTORS

Student’s Name: Ammar T. Ammar

Student’s No.: 1040385

Section: 7

Date: 18/3/2005

Instructor: Dr. Ziyad Izzat

2005
Copy left reversed

Abstract:

The aim of the experiment: is to study the behavior of capacitors and inductors in electric circuits , and know the main features of such circuits and how they function.

The method used: by using the oscilloscope to see how the potential difference changes as a function of time.

Main results:

Theory:

I- RC circuits

Charging a capacitor:

During the positive half period of the square wave, the charge in the simple RC circuit shown in fig(l) builds up on the capacitor plates according to the following formula;

Q(t)=Cε(1−e−tRC)

The voltage across the capacitor plates is defined by:

VC=QC

and therefore ;

Vc=ε(1−e−tRC)

RC is usually called the time constant (τ) of the RC circuit. τ has the unit of time (sec ) and is a measure of how fast the voltage across the capacitor rises. When t =τ,

V=0.63ε

or, the voltage across the capacitor rises to 0.63 of its maximum value.

The current passing through the circuit is given by:

I(t)=dQdt=εRe−tRC

Therefore, the voltage across the resistor is

VR=I(t)R=εe−tRC

Discharging a capacitor:

During the negative half period of the square wave, the capacitor, in the RC circuit of fig (1), discharges according to the following formula:

Q(t)=Cε−tRC

Hence, the voltage across the capacitor plates is given by:

VC=0.37ε

Or the voltage across the capacitor plates decays to 0.37 of its maximum value within a timeτ.

The current passing through the circuit is :

I(t)=dQdt=−εRe−tRC

Thus, the voltage across the resistor is given by :

VR=I(t)R=−εe−tRC

The graphs in fig(2) show the functional relation between both the voltage across the capacitor and the resistor and the time for both charging and discharging.

II- RL circuits

In fig(3), the current passing through the circuit rises with time according to the following equation;

I=εR(1−e−RtL)

The voltage across the resistor is:

VR=IR=ε(1−e−RtL)

and the voltage across the inductor is:

VL=LdIdt=εe−RtL

The quantity L/R is called the time constant (τ) of the circuit; it is a measure of how fast the current rises in the circuit. When t = τ,

VR=063εVL=0.37ε

The graphs of fig(4) show both VR and VL as a function of time

III- LC circuits

In the circuit of fig(5), the voltage across the capacitor plates is  described through the following equation:

VC=VC0Cos(ωt+Φ)

where VC0 is the amplitude ( constant) and

ω=1LC

Fig(6) shows the voltage across the capacitor as a function of time.

Data:

After performing the experiment the following data were obtained:

Table 1: RC circuit

| C (μF) | R (Ω) | τ1(Sec) | τ2(Sec) |
| --- | --- | --- | --- |
| 0.01 | 1000 | 10×10−6 | 10×10−6 |

Table 2: RL circuit

| L (mH) | R (Ω) | τ1(Sec) | τ2(Sec) |
| --- | --- | --- | --- |
| 10 | 1000 | 0.8×10−6 | 0.8×10−6 |

Table 3 :LC circuit

| L (mH) | C (μF) | f(Hz) | ω (rad/sec) |
| --- | --- | --- | --- |
| 10 | 0.1 | 5000 | 31416 |

From Thevenin's circuit:

I3=1.86mA

From Norton's circuit:

I3=1.89mA

Data Analysis:

The theoretical values of :

From Thevenin's circuit:

From Norton's circuit:

Results and conclusions:

Theoretically:

Thevenin's and Norton’s   : I3=1.88mA

Experimentally

From Thevenin's circuit   : I3=1.86mA

From Norton's circuit:       I3=1.89mA

After performing the experiment we come to the following conclusions:

- By comparing the values of the electric current obtained from the theoretically by solving the current Thevinin’s and Norton’s techniques,  and the values obtained from the experiment we find that these techniques  fit almost perfectly with a slight discrepancy that fall within the experimental error range. Which is not something strange since they are based on the natural laws of electricity.

- Results seem to be very close to the values in experiment #3 (we have used the same circuit). And that shows the application for Thevenin and Norton techniques.

- Thevinin’s and Norton’s techniques are among the circuit analyzing methods used with complicated circuits, when we have voltage sources connected in parallel, that is to say when Ohm’s law is not applicable anymore.

- The difference in the values obtained and the theoretical ones are due to some kind of errors concerning the precision of the used apparatus, and maybe some systematic errors while setting the circuit. However, such errors don’t affect our conclusion that our verification of the circuit  has worked.