Physics Lab 112

Experiment No. 10

Resonance

Student’s Name: Mohannad Saidi

Student’s No.: 1110689

Partner’s Name: Hala Nahhal

Partner’s No: 1111512

Section: 4.

Instructor: Dr. Ziad Izzat

- Abstract:

This experiment aims to identify the natural frequency (f0) and the corresponding angular frequency(ω0) for an RLC circuit by plotting a graph of I vs. ω for different frequencies and finding the value of ω where the graph’s maximum occurs. It also aims to find the quality factor at resonance for two different values of R by using specific equations whose parameters we get using the graph mentioned above (this is explained better in II Theory). The main results we obtained are:

fo(th)=5.035KHz

fo(exp) = 8 KHz

W(th) = 31.62

W(exp) =50.42

When R = 1 kΩ:

Q=ωoΔω=50.42×10318∗103==2.8

When R = 2 kΩ:

Q=ωoΔω=50.42∗10329∗103=1.74

- Theory:

For the RLC circuit shown in Fig(1), the current in the circuit is given by: I0=V0R2+(ωL−1ωC)2 -- eq(1), At a certain value of R, the current assumes a maximum when ωL=1ωC -- eq(2). At this point, the driving voltage frequency is equal to the natural or resonant frequency, the corresponding angular frequency (ω0) is given by the relation:ω0=1LC--eq(3).

When plotting a graph of I vs.ω, it assumes a max atω0.

The quality factor (Q) is a measure of the sharpness of such a graph, where: Q=ωLR--eq(4). At resonance we get Q=1RLC--eq(5). The quality factor can be also obtained experimentally from the graph of I vs.ω: we can measure the bandwidth (Δω) of the curve which is the difference between ω that correspond to the points at which the horizontal line I=Imax2 crosses the graph of I vs. ω.

This way, Q=ω0Δω--eq(6).

Y

R2>R1

L

C

R

ε(t)

Fig(1)

Fig(2

- Procedure:

- We connected an RLC circuit like the one in Fig(1).

- With the CRO in its internal mode, we connected its Y channel in parallel to the variable resistance.

- We read the peak to peak voltage (Vp-p) that corresponds to each of the 17 different frequencies coming from the signal generator ranging from 0.2 – 40 KHz. at each frequency we first took the variable resistance to be 1KΩ then 2KΩ. So we ended with 34 voltage readings.

- Next, we plotted a graph for I (where I=Vp-p/2R) vs. ω (whereω=2πf) for each of the two values of the resistance. Then, as described is II Theory, we found the quality factor for each value of R.

  - Data:

| f kHz |  |  | VR1 | IR1 |  | VR2 |  | IR2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.2 |  |  | 0.126 | 0.126 |  | 0.239 |  | 0.1195 |
| 0.3 |  |  | 0.185 | 0.185 |  | 0.290 |  | 0.145 |
| 0.5 |  |  | 0.33 | 0.33 |  | 0.6 |  | 0.3 |
| 0.6 |  |  | 0.39 | 0.39 |  | 0.7 |  | 0.35 |
| 0.8 |  |  | 0.5 | 0.5 |  | 0.88 |  | 0.44 |
| 1.0 |  |  | 0.62 | 0.62 |  | 1.06 |  | 0.53 |
| 2.0 |  |  | 1.08 | 1.08 |  | 1.54 |  | 0.77 |
| 3.0 |  |  | 1.42 | 1.42 |  | 1.72 |  | 0.86 |
| 4.0 |  |  | 1.65 | 1.65 |  | 1.82 |  | 0.91 |
| 4.5 |  |  | 1.64 | 1.64 |  | 1.85 |  | 0.925 |
| 5.0 |  |  | 1.7 | 1.7 |  | 1.86 |  | 0.93 |
| 5.5 |  |  | 1.74 | 1.74 | 1.88 |  | 0.94 |  |
| 6.0 |  |  | 1.76 | 1.76 | 1.9 |  | 0.95 |  |
| 8.0 |  |  | 1.8 | 1.8 | 1.96 |  | 0.98 |  |
| 10.0 |  |  | 1.77 | 1.77 | 1.95 |  | 0.975 |  |
| 20.0 |  |  | 1.42 | 1.42 | 1.72 |  | 0.86 |  |
| 50.0 |  |  | 0.76 | 0.76 | 1.13 |  | 0.565 |  |

  - Calculations:

The resonance frequency:

fo=8kHz (experimentally)

The angular resonance frequency:

ωo=1LC=31.62 K rad/sec (Theoretically)

ωo=2πfo=2×3.14×8=50.42Krad/sec (experimentally)

First case R=1kΩ:

The quality factor:

Q=1RLC=11×10310×10−30.1×10−6=0.32 (Theoretically)

Imax= 1.8 Ma

0.7 07* Imax = 1.27

This value crosses graph at f=2.5 and f=20.5

Δf = 18

Q=ωoΔω=50.42×10318∗103==2.8 (experimentally)

second case R=2kΩ

The quality factor:

Q=1RLC=12×10310×10−30.1×10−6=0.158 (Theoretically)

Imax= 0.98

0.707* Imax= 0.693

This value crosses graph at f= 1.5  and f= 30.5

Δf= 29

Q=ωoΔω=50.42∗10329∗103=1.74 (experimentally)

  - Analysis:

By analyzing the graphs, data and the numerical results we can see that:

- The results obtained experimentally for the natural frequency (f0) and the corresponding angular frequency (ω0) closely matched what we predicted by theory.

- As predicted, the graph of I vs. ω has a maximum atω0.

- The graph of I vs. ω is symmetric about the line X=ω0. (We used this fact to find Δω in a manner different than that described in the lab manual because the distance between any two points on the graph lying on the same horizontal line is double the distance between one of these two points and the line X=ω0)

- For each value of R, the value we got for the quality factor experimentally and by direct calculation closely matched.

- The sharpness of the graph of I vs. ω increases as the resistance deceases: this can be seen by looking at equation (5) in II Theory, where Q is inversely proportional to R. This is also confirmed by the graphs we obtained.

  - Conclusion:

We can conclude that current in an RLC circuit has reaches a maximum when the driving voltage frequency is equal to the resonant frequency. We can also conclude that the greater the resistance in an RLC circuit, the small the quality factor.

The results we obtained experimentally in this experiment closely matched those obtained by theory; the main factor that limited our precision, and therefore gave us a slight difference between practical and theoretical results, is the limitation imposed by the instruments and graphs when taking our data and measurements.

For example: when we wanted to find the quality factor using the I vs. ω graph, we had to determine the maximum and then take ω that corresponded to Imax2… during all this much estimation had to be made, especially because we were dealing with a logarithmic scale. Additionally, for some values of f we had to set the output frequency of the signal generator to a precision that is not given by the generator’s dial so it was only a matter of human judgment.