### Physics Department

### Physics 112

### Experiment No.10

### Resonance

### Student’s Name: Almutasem bellah Omar

### Student’s No.: 1090349

### Partner’s Name: Jihad Abdullah

### Partner’s No.: 1090349

### Section: 5

### Date: 29\3\2010

### Instructor: Dr. Ghassan Andoni

Abstract:

1)The aim of the experiment :is to find the resonance frequency and the quality factor.

2)The method used :  by measuring the voltage as a function of frequency, and using it to plot the current as a function of the frequency.

3)The main results are:

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

Theory:

The amplitude of the current passing through the circuit shown in is given by, Io=VoR2+ωL−1ωC2.

As we can see, we can get a maximum value of Io when

ωL=1ωC

⇒ωo=1LC Under such a condition ω is the natural angular frequency of the circuit.

The resonance is that the current assumes its maximum value when the deriving voltage frequency equals the natural frequency of the RLC circuit.

Fig.2 shows a plot of the value of Ioas a function of ω. At resonance Io=VoR and the value of the current is only limited by the resistance of the circuit.

The quality factor:

A measure of the sharpness of the resonance curve is a quantity called the quality factor (Q), which is defined as Q=ωLR

At resonance Q=1RLC

Fig.3 shows a plot of resonance curve for different combinations of R, L, and C.

The band width (∆ω) is a practical value that measures the sharpness of the resonance curve. The band width (∆ω) is the frequency range between the maximum value of Ioand the value Io2, see fig.2.

The quality factor is related to the band width as follows:

Q=ωo2Δω.

Procedure:

1-we connected the circuit of figure 4.

2- For R=1 kΩ ,we measured the current as a function of the input voltage frequency.

3-we measured the phase shift between the current and the voltage in both cases.

4-we repeated the previous step for R=2 kΩ.

Data:

L=10mH;  C=0.1µF; R=1kΩ

| f (kHz) | ω (rad/s) | R = 1 kΩ |  | R = 2 kΩ |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | V0(volt) | I0(mA) | V0(volt) |  | I0(mA) |
| 0.2 |  |  |  |  |  |  |
| 0.3 |  |  |  |  |  |  |
| 0.5 |  |  |  |  |  |  |
| 0.6 |  |  |  |  |  |  |
| 0.8 |  |  |  |  |  |  |
| 1.0 |  |  |  |  |  |  |
| 2.0 |  |  |  |  |  |  |
| 3.0 |  |  |  |  |  |  |
| 5.0 |  |  |  |  |  |  |
| 5.5 |  |  |  |  |  |  |
| 6.0 |  |  |  |  |  |  |
| 8.0 |  |  |  |  |  |  |

Calculations:

## Theoretically:

ω0=1LC=

For R = 1kΩ

Q=ω LR=1RLC=

For R = 2kΩ

Q=ω LR=1RLC=

## Experimentally:

## From the graphs

fo=.............kHz

ωo=2πfo=.............................................

For R = 1kΩ

I0 =

I02	=

2Δω=

Q=ωo2Δω=

For R = 2kΩ

I0 =

I02	=

2Δω=

Q=ωo2Δω=

Results and conclusions:

R = 1 KΩ                               R = 2 KΩ

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

Question :