BIRZEIT UNIVERSITY

Physics Department

## Physics 111

Experiment No. 4

D.C. Circuit

………………………………………………………………………………………

Student’s Name: Lara Sami .        Student’s No.: 1120139

Partner’s Name: Weam Mikkawi     Partners’ No. : 1120442

Instructor: Ziad Sa'eed                    Section No.:  4 M

Date: 22/10/2012

- Abstract:

- The aim of the experiment:

- To test the material and find out if the material is Ohmic or  non-Ohmic.

- To find the value of Rs , Rp practically 
where:

Rs is  equivalent resistance for two resistors connected in series

Rp is the equivalent resistance for two resistors connected in parallel

- The method used: 
A)By plotting a graph of the potential difference V across the material against the current through it and keeping the temperature of the material constant. If the graph is linear then the material is Ohmic.
 B) by connecting two resistors at first In series and one more time   in parallel and measure( Vs, Is),( Vp, Ip)

- The main results are:

R = 75  ± 8 Ω

Rs = 214 ± 20Ω

Rp =49 ±4Ω

- Theory:

(*) One of the fundamental laws describing how electrical circuits behave is Ohm’s law. According to Ohm’s law, there is a linear relationship between the voltage drop across a circuit element and the current flowing through it. Therefore the resistance R is viewed as a constant independent of the voltage and the current. In equation form, Ohm’s law is:

V = IR.

Here, V is the voltage applied across the circuit in volts (V), I is the current flowing through the circuit in units of amperes (A), and R is the resistance of the circuit with units of ohms (Ω).

R=V(Voltage)I(Current)R : resistance. R doesn't depend on (I) or (V).

(*) Materials that obey Ohm's law are called Ohmic materials. Materials that don't obey Ohm's law are called non-Ohmic materials. Testing the material to decide if it's Ohmic or non-Ohmic could be done by plotting the graph of the voltage (V) on the x-axis vs. the current (I) on the y-axis and if the graph is linear then the material is Ohmic, and if it's not linear then the material is non-Ohmic.

(*)Two or more resistors can be connected in series, connected one after another (Fig. (a)), or in parallel, typically shown connected so that they are parallel to one another (Fig. (b)). As long as the current must split, go through the resistors and then coalesce, they are in parallel.

When two resistors R1 and R2 are connected in series, the equivalent

resistance RS is given by RS = R1 + R2. Thus, the circuit in Fig. (a)

behaves as if it contained a single resistor with resistance RS — that is,

it draws current from a given applied voltage like such a resistor. When

those resistors are connected in parallel instead, we use a different formula for finding the equivalent resistance.

Fig.(a)                                                                  Fig.(b)

R1,R2 on Series :                                              R1,R2 on Parallel :

- Procedure:

At first, a resistance was connected with another variable resistance  to a circuit with a 3 volt power supply and. Then the uncertainty in the current I and the uncertainty in the voltage V was estimated. Then we measured the current (I) and the potential difference (V) across the resistance. After that the current and the potential difference were measured, but after adjusting the variable resistance and this was repeated six times. Then we disconnected the variable resistance and connected another resistance (R2) in series instead. And after estimating I and V we wrote down the reading of Is and Vs. Then the second resistance was connected in parallel and I and V were estimated and the Ip and Vp were written down. Finally the values of R1 and R2 were written down using the color codes on the back of the resistances.

- Data:

Part A (one resistance used in the circuit, either R1 or R2):

V =0.1				I= 1mA

| No | 1 | 2 | 3 | 4 | 5 | 6 | Average |
| --- | --- | --- | --- | --- | --- | --- | --- |
| I(mA) | 7 | 13 | 20 | 26 | 34 | 41 | 23.5 |
| V(volt) | 0.5 | 1 | 1.5 | 2 | 2.5 | 3 | 1.75 |

Part B (R1 and R2 in series):

Vs = 3 volt                                 Vs =  0.1 volt

Is = 14 mA                                 Is =1mA

Part C (R1 and R2 in parallel):

Vp =  2 volt                             Vp = 0.1 volt

Ip = 41 mA                             Ip= 1mA

- Calculations:

R  = from the slope = ΔVΔI=0.75−010−0=0.075KΩ=75Ω

R =  R(ΔVV+ΔII)=75(0.11.75+1×10−323.5×10−3)=8Ω

Resistance R1 from color code =   75    ± 4 Ω

Resistance R2 from color code =  150 ± 8 Ω

R1 and R2 in series:

A)Practically(By experiment):

Rs = VSIS=314×10−3=214Ω

Rs = RS(ΔVSVS+ΔISIS)=214(0.13+1×10−314×10−3)=20Ω

B) theoretically:
R1 from color code =   75    ± 4 Ω
R2 from color code =  150 ± 8 Ω
 RS = R1+R2= 75+150=225

ΔRs=ΔR1+ΔR2=4+8=12

Rs= 225 ± 12 Ω

R1 and R2 in parallel:

A)Practically(By experiment):

Rp = VPIP=241×10−3=49Ω

Rp = RP(ΔVPVP+ΔIPIP)=49(0.12+1×10−341×10−3)=4Ω

B) theoretically:
R1 from color code =   75    ± 4 Ω
R2 from color code =  150 ± 8 Ω
 Rp=R1R2R1+R2=75Χ15075+150=50Ω

To find Rp Assume that

Rp=AB→then→ΔRp=(ΔAA+ΔBB)Rp

Where A= R1R2= 75× 150=11250
ΔΑ=ΔR1R1+ΔR2R2=475+8150=0.11Ω
B= R1+R2=75+150=225
ΔB=ΔR1+ΔR2=12

So

ΔRp=(ΔAA+ΔBB)Rp=(0.1111250+12225)50=3Ω
Rp =3Ω

- Results and Conclusion:

- Part A (one resistance used in the circuit, either R1 or R2):

- My result from the slope of the graph is
 R1= 75 ± 8 Ω

- The true value of the resistor R1 from color code is 
R1= 75 ± 4 Ω

Discrepancy : | true value – result| = | 75 – 75 | = 0

0 < 28 → 0<16

My result is accepted because D < 2ΔR.

- Part B(R1 and R2 in series): 
My result is
 Rs= 214 ± 20 Ω

The true value of the equivalent resistance for two resistors  
   connected in series theoretical is :
    Rs= 225 ± 12 Ω

- Discrepancy : | true value – result| = | 225– 214 | = 11

11 < 220 → 11<40

My result is accepted because D < 2ΔRs.

Part C(R1 and R2 in parallel): 
My result is
 Rp= 49 ± 4 Ω

The true value of the equivalent resistance for two resistors  
   connected in parallel theoretical is :
    Rp= 50± 3Ω

- Discrepancy : | true value – result| = | 50– 49 | = 1

1 < 24 → 1<8

My result is accepted because D < 2ΔRp.

systematic errors 
A wire is actually a resistor with very low resistance compared to the resistors we typically use in class. Therefore, we neglected (ignored) any resistance that it has. Also the ammeter and the voltmeter have a resistance that we neglected.

When we take the measurements we must look straight at the pointer to have a correct reading and if we don't our reading will not be accurate.

The answer of the Question:

My measured values of R1, Rs , and Rp are consistent with the values that I obtained from color codes.