Student’s Name: Mohammad Na’em Mohammad Al Khatib.

Student’s No.: 1051104.

Partner’s Name: Ra’fat Nihad yousef

Partner’s No.: 1050675

Instructor: Tayseer Arouri

Section number: (8)

Date: 1/12/2005

- The aim of the experiment is:

  - To measure the decay constant and the half-life of a draining column of water.

- The method used is: the Burette method.

- The main result is :

(h vs. t graph)

t12=23±2sec		(Ln(h) vs. t graph)

We consider a tube having water in it of a height () at time (). When we open the valve water drains from it at some rate.

The rate of the decay of the water column is proportional to its height (h) that is:

−dhdt∝h(t)

−dhdt=λh(t)

Whereλis the a constant,

dhdt=−λh(t)

dhh=−λdt

lnh−lnh0=−λt

lnhh0=−λt

h=h0e−λt

So that λis the decay constant.

When h = then:

h02=h0e−λt

12=e−λt

−λt12=−ln2

t12=ln2λ=0.693λ

When λis greater then is smaller:

First we cleaned the tube in case there will be any dirty which can close the way of water going out of the tube.

After that we started to add little quantities of water in order to calibrate the hole from which water would go out so that the whole quantity of water after that would be emptied within about a minute and this is related to the quality of the burette we used.

After we calibrated the hole we started to add the whole quantity needed to fill the burette

Total burette length L=58.2 u		 (u is the burette units)

| Time (sec) | Burette reading (u) |  |  | b (u) | h=L−b (u) |
| --- | --- | --- | --- | --- | --- |
|  | b1 | b2 | b3 |  |  |
| 0 | 0.0 | 0.0 | 0.0 | 0.00 | 58.20 |
| 10 | 6.1 | 6.0 | 5.7 | 5.93 | 52.27 |
| 20 | 11.4 | 11.4 | 11.5 | 11.43 | 46.77 |
| 30 | 16.6 | 16.8 | 16.3 | 16.57 | 41.63 |
| 40 | 22.0 | 22.0 | 21.9 | 21.97 | 36.23 |
| 50 | 26.6 | 26.9 | 26.5 | 26.67 | 31.53 |
| 60 | 31.5 | 31.2 | 31.2 | 31.30 | 26.90 |
| 70 | 36.6 | 36.0 | 36.4 | 36.33 | 21.87 |
| 80 | 40.5 | 40.7 | 40.7 | 40.63 | 17.57 |
| 90 | 44.3 | 44.5 | 44.2 | 44.33 | 13.87 |

h vs. t graph

(the best fitting curve was found by the computer, it has the equation y = 63.5e-0.0304t:

Table of measurements for the half-time from the plot

| t12(sec) | 23.08 | 22.31 | 23.08 | 23.08 | 22.69 | 25 |
| --- | --- | --- | --- | --- | --- | --- |

The average value of t12is:

t12= 23.20513 sec

The error in t12is:

ln(h) vs. t Graph

The best fitting line to the data was find by the computer (On least square fit method), its equation is:

ln(h)=−0.0304t+4.1517

The slope =−λ=−0.0304sec−1

The uncertainty in the slope, which also was found by the computer, is:

Δm=−Δλ=0.002sec−1Δλ=0.002sec−1

t12=Ln2λ=0.6930.0304=22.8sec

The uncertainty in t12is:

(h vs. t graph)

(ln(h) vs. t graph)

(h vs. t graph)

t12=23±2sec		(Ln(h) vs. t graph)

First we see that the t12with its range of error intersects with the range of error for t12 so that we see that our results is accepted within the rang of error we have.

As every experiment we expect some error to occur in it. One expected one is that the water as we noticed haven’t had the wanted continuity while the water exactly runs out especially at the end of the interval of time we achieved.

Another expected error is the difference (in time ) between stopping or starting the timer watch and closing the hole of the valve as a result of the delay in our brains signals. And that causes a quantity of water to be gone out or perhaps stopping the watch before stopping the flow of water.

Lastly, We can guess another error which is commonly expected while using any kind of instruments. It is that the burette we used with that short interval of time would affect our accuracy in finding equations or even having good fitting lines and that could be solved by getting a burette with a good quality which will make the  time interval longer , and so , the lines and calculations will be more and more accurate .