### Physics Department

### Physics 112

### Experiment No.5

### Capacitors And Conductors

### Student’s Name: Mudar Abed Rabbo

### Student’s No.: 1041433

### Partner’s Name: Khairallah

### Partner’s No.: 1040132

### Section: 6

### Date: 13/11/2005

### Instructor: Dr. Ziyad Izzat

Abstract:

The aim of the experiment: is to find out the time constant in RC, RL , and LC circuits.

The method used: is by using the CRO to measure the voltage in the RC and RL circuits and to measure the frequency in the LC circuit.

The main result:

τ≈τth (RC circuit)

τ≈τth (RL circuit)

fo≈fthωo≈ωth (LC circuit)

Theory:

RC circuits:

Charging a capacitor:

The voltage across the capacitor’s plates is defined by VC=QC, and while Q(t)=Cε1−e−tRC (when we are talking about the positive half period of the square wave), then VC=ε1−e−tRC.The value of RC is usually called the time constant (τ) of the RC circuit like the one shown in fig.1. τ is a measure of how fast the voltage across the capacitor rises. When t=τ, VC=0.63ε.

The current passing through the circuit is given by: I(t)=dQdt=εRe−tRC, and while the voltage across the resistor is VR=I(t)R=εe−tRC.

Discharging a capacitor:

Now, during the negative period of the square wave, the capacitor, the capacitor discharges according to the following formula:

Q(t)=Cεe−tRC

And so the voltage across the capacitor’s plates is:

VC=0.37ε

In this case the voltage decays to 0.37 of its maximum value within a time τ, which equals RC (the time constant).

The current passing through the circuit is: 
I(t)=dQdt=−εRe−tRC
and so, the voltage is given by:

VR=I(t)R=−εe−tRC

RL circuits:

The current passing through the RL circuit shown in fig.3 rises with time accordi

ng to the following equation:

I=εR1−e−RtL

The voltage across the resistor is VR=IR=ε1−e−RtL and across the inductor is VL=LdIdt=εe−RtL.

In this case the time constant equals LR. When t=τ, 
VR=0.63ε and VL=0.37ε.

LC circuits:

The following equation describes the voltage across the capacitor’s plates of the one in the circuit shown in fig.5, 
VC=VCocosωt+Φ
where VCO is the amplitude (constant) andω=1LC.

Fig.6 shows the voltage across the capacitor as a function of time.

Data:

R = 1 kΩ

C = 0.1 μF

L = 10 mH

RC circuit:

Vmax= 8 v

VC : τ 1 =0.8 × 10-4 sec

VR : τ2 = 1× 10-4 sec

RL circuit:

Vmax= 7.6 v

VR : τ 1 = 0.8 × 10-5 sec

VL : τ2 =1× 10-5 sec

LC circuit:

The value of frequency as we read from the signal generator is 5 × 103 Hz.

The value of frequency theoretically is 5033 Hz.

The value of frequency practically is 5 × 103 Hz.

Calculations:

RC circuit:

τ=R×C=1000×0.1×10−6=10×10−5sec. (Theoretically, in the two cases; charging and discharging).

τ1=8×10−5secτ2=10×10−5sec. (The practical values of time constant that we obtained by measuring the voltage across the capacitor and the resistance).

RL circuit:

τ=LR=10×10−31000=10×10−6sec. (Theoretically)

τ1=8×10−6secτ2=10×10−6sec. (The practical values of time constant that we obtained by measuring the voltage across the resistance and the inductor e).

LC circuit:

The value of frequency as we read from the signal generator is 5 × 103 Hz.

ωo=2πfo=2×227×5000=31428.6Hz

Theoretically:

ωo=2πfo=1LC=110×10−3×0.1×10−6=110−9=31622.7766Hzfo=31622.77662π≈5033Hz

Analysis of results:

As we saw from the results of the three circuits the values were somehow close to those of the theoretical ones, the small difference may be because of the visual inaccuracy while looking on the CRO.

Q1) τ→0

charging⇒VC=ε(1−e−tRC)=ε(1−e0)=0discharging⇒VC=εe−tRC=ε

charging⇒VC=ε(1−e−tRC)=εdischarging⇒VC=εe−tRC=0

Q2) We can only do so if we use large values of R,L, and C.