If an experiment works, something has gone wrong.

Birzeit University

Physics Department

Physics 112

Experiment No.7

Impedance & Reactance

Student’s Name: Ammar T. Ammar

Student’s No.: 1040385

Section: 7

Date: 28/4/2005

Instructor: Dr. Ziyad Izzat

2005
Copy left reversed

Abstract:

1)The aim of the experiment is to find the phase shift between each pair of the voltage difference of the capacitor the resistance & the inductor and to measure the frequency at which the phase shift between the current and the driving voltage is zero.

2)The method used is  by displaying the voltage difference of the capacitor the inductor , the resistance and the driving voltage on the oscilloscope's screen .

3)The main results are

f=11KHZϕCR=1.52 rad ϕLR=1.63 radϕLC=3.15 rad

Theory:

In the RLC circuit shown in figure1  the current is :

I(t)=ε(t)Zeq

Where

Zeq=ZR+ZC+ZLZR=R (the resistive impedance)ZC=−jωC (the capacitance impedance) ZL=jωL (the inductive impedance)

While:

1ωC is the capacitance reactance and ωL is the inductive reactance.

We Solve the equation of the current :

ε(t)=ε0Cos(ωt)

I(t)=ε0ejωLR+j(ωL−1ωC))(R−j(ωL−1ωC)R−j(ωL−1ωC)=R−j(ωL−1ωC)ε0ejωLR2+(ωL−1ωC)2R2+(ωL−1ωC)ε0ej(ωL+ϕ)R2+(ωL−1ωC)2=ε0R2+(ωL−1ωC)ej(ωL+ϕ)

=ε0R2+(ωL−1ωC)Cos(ωt+ϕ)=I0Cos(ωt+ϕ)

Where:

ϕ=tan−1(1ωC−ωLR)

I0=ε0R2+(ωL−1ωC)

The phase shift between the current and the voltage is given by φ=ωΔt see fig 2.

The voltage a cross the inductor is given by:

VL=LdI(t)dt=−ωLI0Sin(ωt+φ)

This equation shows that the voltage of the

inductor is just the inductive impedance

multiplied by the current with a phase shift=Π2  .

The voltage across the capacitor is given by :

VR=RI(t)=RI0Cos(ωt+φ)

And here VRis the current multiplied by the resistance..

According to the voltage across the capacitor :

VC=1CI(t)dt=1CI0Cos(ωt+φ)dt=I0ωCSin(ωt+φ)

Which is the current multiplied by the capacitive reactance with a phase shift of Π2 introduced.

To find the phase shift between two

harmonic functions using the oscilloscope

we set the oscilloscope to the x-y mode

and connect the functions to the X-channel

and Y-Channel , to have a shape like that

shown in fig 3 .

Let Vx=Vx0Cos(ωt) and Vy=Vy0Cos(ωt+φ)where φis the phase shift between the two functions .

ωt=0at point a shown in fig3 where Vx has its maximum value ,hence:

VY=VY0Cos(φ)φ=Cos−1(VYVY0)

Procedure:

1. We connected the circuit shown in figure 4.

2. We found the phase shift between the driving

Voltage and the the current for different frequencies.

3. We displayed each pair pf VR, VC & VL oscilloscope screen. And measured the phase shift between each pair of them.

Data & Calculations:

1)

| Frequency KHZ | a (unit) | b (unit) | a/b | Phase shift (rad) |
| --- | --- | --- | --- | --- |
| 0.5 | 1.4 | 1.4 | 1.000 | 1.57 |
| 1 | 3.3 | 3.4 | 0.971 | 1.33 |
| 3 | 3.4 | 3.8 | 0.895 | 1.11 |
| 5 | 4.8 | 6.4 | 0.750 | 0.85 |
| 7 | 2.2 | 4 | 0.550 | 0.58 |
| 9 | 1.2 | 4.6 | 0.261 | 0.26 |
| 10 | 0.5 | 4.7 | 0.106 | 0.11 |
| 11 | 0.1 | 4.8 | 0.021 | 0.02 |
| 15 | 1.7 | 4.4 | -0.386 | - 0.40 |
| 30 | 2.3 | 2.8 | -0.821 | -0.96 |
| 50 | 1.6 | 1.7 | -0.941 | -1.23 |
| 70 | 1.2 | 1.2 | -1.000 | -1.57 |
| 100 | 0.8 | 0.8 | -1.000 | -1.57 |

From the graph we found can see that the value of frequency that makes the phase equal to zero is 11 KHZ.

Theoretical value:

f=ω2Π=12ΠLC=10.7KHZ

2)

Experimental values:

**Phase shift between VC & VR (ϕ1):

ϕ=ωΔt=2ΠfΔt=11X103X22X10−6X2Π=1.52 rad

**Phase shift between VL & VR(ϕ2) :

ϕ=ωΔt=2ΠfΔt=11X103X24X10−6X2Π=1.63 rad

**Phase shift between VL & VC(ϕ3):

ϕ3=ϕ1+ϕ2=1.63+1.52=3.15 rad

Theoretical value of:

ϕ1=ϕ2=1.57 rad

ϕ3=3.14 rad

Results & conclusions:

f=11KHZϕCR=1.52 rad ϕLR=1.63 radϕLC=3.15 rad

From the experimental & theoretical values we have we conclude the following:

- The value of the frequency we have (11 KHZ) is almost equal to the theoretical one (10.7KHZ) and the discrepancy lies within the experimental error.

- The value of the phase between the voltage of the capacitor and the voltage of the resistance (1.52) is near to the real value , and the discrepancy can be attributed to the value of frequency at which we made the measurements which was (11 KHZ) a little bit more than (10.7KHZ) and that caused a little shift to the phase from the expected value.

- The value of the phase shift between the voltage of the inductor and that of the resistance is 1.63 which has a little discrepancy which we can attribute to the value of frequency at which we take the measurements which was 11KHZ (more than 10.7KHZ)

- The value of the phase shift between the voltage on the capacitor and that on the inductor(3.15 rad) has a little discrepancy from the theoretical value(3.14 rad) , such a discrepancy is expected as the experimental value is the algebraic sum of the two first phase shifts (which have a discrepancy from their theoretical values )

- We can measure the phase shifts using the external mode and compare them with the values of the internal mode, then with the theoretical values to have results almost identical with the theoretical ones.

AC-powered

As Sin(x)=cos(x+Π2)

The phase shifts between the current and the voltages across the circuit elements is related to φ which is a function of ω

look at the attached graph

which is the theoretical value of the frequency at which the phase shift between the current and the driving voltage is zero.