Damped Oscillation

Purpose

Abstract:

In this experiment we are going to deal with the ( CRO ) and the signal generator in making damping for an inductor changing it with the resistor , and see  the results on the screen of the oscilloscope .

Main results:

1- Over damping:

t1/2 =0.08ms

λ+ = 1.55*105  s-1

λ-  = -6.45*105  s-1

2- Critical damping:

t1/2 =0.04ms

λ+ =  λ- = 1*105

R=463Ω

3- Under damping:

R=63Ω

t1/2 = 1.1 *10-5  sec.            (Experimental value)

t1/2 = 0.966*10-5  sec.            (Theoretical value)

Theory

In this type of experiments the charge on the capacitor and hence is given by the formula:

Q (t) = A1 eλ+t + A2 eλ+t

Where A1 & A2 are constants

λ+  = ( -R/2L ) + { (R/2L)2 – ( 1/LC )}1/2

λ - = ( -R/2L ) - { (R/2L)2 – ( 1/LC )}1/2

For this type of solutions there is three cases emerge

1) Over damping:

(R/2L)2>(1/LC)

So as a result the both terms decay exponentially with time and voltage across the capacitor is said to be over damped.

As the following figure:

Figure 1

2) Critical damping:

(R/2L)2 = (1/LC)

Then the term under the square in the equation vanishes therefore the charge on the capacitor plates take the following form:

Q(t) = Ae(R/2L)t + Bte(-R/2L)t

Where B & A are constants

Figure 2

3) Under damping:

This case would happen if:

(R/2L)2<(1/LC)

Then the term of the square root becomes negative so the solution which is introduced:

Q(t) = Q0e-δt cos (ωt + Φ0 )

Where δ = R / 2L

ω = {(1/LC) – (R/2L)2}1/2

There is a quantity which is the time ½ t after which the amplitude (evolve) Q0eδt ½

Q0/2 = Q0eδt ½

T1/2 = {(2L) Ln (2)} / R

Vc = Q(t)/C

Apparatus:

1- Resistance decade box.

2- Inductor ( 10 mH )

3- Capacitor (0.01 μF )

4- Signal generator.

5- Oscilloscope

Procedure:

1- We connected the circuit shown in the figure.

2- We displayed the voltage across the capacitor on the CRO.

3- We changed the value of R to obtain the three damping cases.

Conclusion:

In this experiment, we did a three cases for damped oscillation … in the first case, we notice that the over damping happens for R greater than 2 KΩ. and in the second case which was the critical damping, we notice that it happens at approximately 2 KΩ.

Finally, in the under damping case... we notice that it happens for a value of R less than 2 KΩ. And it was in our experiment 454Ω. For the value of t1/2  measured in this case (0.9*10-5sec),  it was near the theoretical value (calculated)  which was( 0.966*10-5 sec.)