### Physics Department

### Physics 112

### Experiment No.9

### Filters

### Student’s Name: MOEZ ZABEN

### Student’s No.: 1061287

### Partner’s Name: MOHAMMAD MURAD

### Partner’s No.: 1061230

### Section: 8

### Date: 13/12/2007

### Instructor: Dr. Ziyad Izzat

Abstract:

The aim of the experiment: is to find the attenuation factor for both cases of low and high pass filters.

The method used: is by using the CRO to see the plot of the capacitance’s voltage vs. time and get the value of the voltage.

The main results:

A (high) vs. f:

⇒ω−3dB=11000rads

A (low) vs. f:

⇒ω−3dB=10000rads

Theory:

A filter is an electrical circuit that allows certain frequencies to flow while preventing others.

There are three types of filters: high pass filters, low pass filters, and band pass filters.

Low-pass RC filters:

If we took the circuit shown in fig.1a, using the generalized ohm’s law we can obtain the output voltage Voutas a function of the input voltage, Vinas follows:

equivalentimpedance⇒Zeq=R−jωCcurrent⇒I(t)=Vin(t)Zeqoutputvoltage⇒Vout(t)=VC(t)=Vin(t)R−jωC−jωC⇒Vout(t)=Vin(t)1+jωRCVco(ω)=Vino1+ω2C2R2

attenuationfactor(A):A=VCoVino=11+ω2C2R2ω−3dB=1RC⇒A=11+ωω−3dB2Due to the last equation we get three cases:

If we defined:

- If ω>>ω−3dB then A is extremely small and the output signal is highly attenuated.

- If ω<<ω−3dB  then A ≈ 1 and the amplitude of the out put signal is equal to that of the input signal, in other words the signal is passed without attenuation.

- If ω=ω−3dB thenA=12=0.707  and the amplitude of the output signal is 0.707 of the amplitude of the input amplitude. This value sets a practical boundary between passed signals and highly attenuated ones.

It is obvious from here that the circuit we are talking about, shown in fig.2a, acts as a low-pass filter.

High-pass RC filters:

This RC filter looks like the one shown in fig.1b, and we can get the attenuation factor in the same way of the low pass filter. And this gives:

A=11+ω−3dBω2

In the same way:

- If ω<<ω−3dB then A is extremely small and the output signal is highly attenuated.

- If ω>>ω−3dB then A ≈ 1 and the signal is passed without attenuation.

- If ω=ω−3dB then A=12=0.707 this again sets the boundary between passed signals and highly attenuated ones.

It is obvious from the discussion above that the circuit in consideration is acting as a high-pass filter.

Differentiators and Integrators:

If a low-pass filter is functioning in the highly attenuated region, whereω>>ω−3db, then, Vout(t)is extremely small and

VR(t)=Vin(t)−Vout(t)≈Vin(t)

On the other hand,

I(t)=dQ(t)dt=CdVC(t)dt=CdVout(t)dtVin(t)≈VR(t)=RI(t)=RCdVoutdtVout(t)=1RCVin(t)dt

The out put voltage is just integral of the input voltage. Under such conditions this circuit acts as integrator.

If an RC high-pass filter is functioning in the highly attenuated region whereω<<ω−3db, then, Vout(t)is extremely small and

VC(t)=Vin(t)−Vout(t)≈Vin(t)

On the other hand,

I(t)=dQ(t)dt=CdVC(t)dt=CdVin(t)dtVout(t)=RI(t)=RCdVin(t)dt

The output voltage is just the derivative of the input voltage. Under such conditions this circuit acts as a differentiator.

Data:

|  |  |  | Low |  |  |  | High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| f kHz | ωo kHz |  | Vout Volt |  | A |  | Vout Volt |  | A |
| 0.2 | 1.256636 |  | 6.00 |  | 1 |  | 0.68 |  | 0.11 |
| 0.4 | 2.513272 |  | 5.80 |  | 0.966667 |  | 1.50 |  | 0.25 |
| 0.6 | 3.769908 |  | 5.60 |  | 0.933333 |  | 2.00 |  | 0.33 |
| 0.8 | 5.026544 |  | 5.40 |  | 0.9 |  | 2.50 |  | 0.42 |
| 1 | 6.28318 |  | 5.20 |  | 0.866667 |  | 3.00 |  | 0.50 |
| 2 | 12.56636 |  | 3.80 |  | 0.633333 |  | 4.40 |  | 0.73 |
| 4 | 25.13272 |  | 2.40 |  | 0.4 |  | 5.20 |  | 0.87 |
| 6 | 37.69908 |  | 1.60 |  | 0.266667 |  | 5.40 |  | 0.90 |
| 8 | 50.26544 |  | 1.20 |  | 0.2 |  | 5.50 |  | 0.92 |
| 10 | 62.8318 |  | 1.00 |  | 0.166667 |  | 5.60 |  | 0.93 |
| 20 | 125.6636 |  | 0.50 |  | 0.083333 |  | 5.70 |  | 0.95 |
| 40 | 251.3272 |  | 0.26 |  | 0.043333 |  | 5.80 |  | 0.97 |
| 60 | 376.9908 |  | 0.16 |  | 0.026667 |  | 5.85 |  | 0.98 |
| 80 | 502.6544 |  | 0.11 |  | 0.018333 |  | 5.90 |  | 0.98 |
| 100 | 628.318 |  | 0.10 |  | 0.016667 |  | 5.95 |  | 0.99 |
| 200 | 1256.636 |  | 0.04 |  | 0.006667 |  | 6.00 |  | 1.00 |

Calculations:

Using the plots of the attenuation factor vs. frequency we can obtain the values of frequency that gives usω−3dB:

A (low) vs. :

⇒ω−3dB=10000rads

A (high) vs. :

⇒ω−3dB=11000rads

Theoretically the expected value for ω−3dB is:

ω−3dB=1RC=11×103×0.1×10−6=10000kHz

Now looking at the plots of the wave functions obtained by the different frequency wave types, we find that in the case of low-pass filter Vout is the integration of Vin. And in the case of the high-pass filter Vout is the differentiation of Vin.

Analysis of results:

As we can see from the results that we’ve got, the high-pass filter acts as a differentiator and the low-pass filter acts as an integrator. And the RC circuits that the filters consist of affect the amplitude of the voltage in a way that depends on the type of filter that is made from the circuit. Finally the values that we got were somehow equal to those of the theoretical calculations and that is due to several errors that may have occurred because of the resistivity of wires and tools.