____________________________________________________

Physics Laboratory 112

Electricity & Magnetism

Experiment  2

Source Internal Resistance Loading Problems And Circuit Impedance Matching

Student Name: waleed issawi

Student ID: 1071863

Partner's Name: me

Section: 8

Instructor: Dr. adnan jouda

Date: 4/1/2009

Abstract:

- the aim of the experiment is to find the value of  the load resistance RL that satisfies the condition of the maximum power transfer which is RL= R+Rin

- The method used: by reading different measurements of the current passing through the circuit in different values of the RL (variable resistance in the circuit).

Theory:

  - The electromotive force(emf) is the open circuit voltage difference between its terminals and the maximum value of the current it can deliver to a short circuit and it is the way the voltage source is characterized:

- Ideal voltage source (R~0) according to ohm’s law (I=V/R) be able to provide infinite current.

- In real voltage source ,it cant maintain its (emf) as a voltage difference across its terminals and it also can’t provide the circuit with unlimited current.

  - An internal resistance is added to every real voltage source so that it gives more realistic representation of a voltage source. if the internal resistance was small then the voltage source can maintain most of their (emf) as voltage difference and provide the circuit with higher current values .

  - The load is any component which consumes electrical power to produce useful work (RL).

Loading problem:

The current passing in circuits:

I= ε/( RL+ Rin)

So the voltage difference between the source and the terminals:

VRL= (ε RL)/ (RL+ Rin)

*IF RL<<Rin then VRL (approximately = ε) so the source delivers most of its (emf) as a voltage difference across its terminals.

*If RL can be compared to Rin then VRL is smaller than ε so an amount of the power is consumed inside the source to deliver unuseful heat energy in this case the source is said to be loaded.

Impedance matching:( maximum power transfer)

In any circuit we seek for consuming the maximum available power to produce useful work .

The power consumed in the RL:   P=I2 RL

P= (ε2 RL)/ (RL+ Rin) 2

The maximum available power can be obtained when: RL= Rin

Impedance matching is choosing the value of the load resistance that can transfer the maximum power which is when RL= Rin.

since the internal resistance is small we add an extra resistance so that their sum will equal the load resistance. This new resistance (R) can be considered as an additional internal resistance if we saw it from the RL and it will be considered from the source as an additional load resistance.

The additional resistance:

    - advantages: it helps to avoid loading problem and fulfilling the condition  of impedance matching for large load values.

    - Disadvantage :is that  this additional resistance consumes part of the power delivered to the circuit by the source.

Applying the conservation of energy we get:

1/I= RL 1/ ε + (Rin+R)/ ε

A plot of 1/I vs. RL gives a straight line with slope 1/ ε and y-intercept (Rin+R)/ ε.

For the maximum power transfer to the load resistance: RL=R+ Rin

Efficiency (η)

It’s a useful concept to use with power and the efficiency of a component with impedance(resistance) RL operated from a source with internal resistance Rin is the power dissipated in RL and divided in the circuit: η (RL)= RL / ( RL+ Rin)

.

Calculations:

From graph (1) which is a plot of 1/I vs. RL we can find the value of ε:0.075 m.A

slope = Δy/Δx taking two points on the graph .

ε = 1/slope =  .8 - .45/8-4=0.09 Ω.mA.

Rin = 1  kΩ

Conclusion:

Theoretically the value of RL should be equal to the sum of the resistance(R) and but there as  some error because of systemic errors that effected  the result ,and the number that I took is so small in order to make this error.