____________________________________________________

Physics Laboratory 112

Electricity & Magnetism

Experiment  3

Impedance And Reactance

Student Name: YASER ARAFAT ALHAWWARIN

Student ID: 1031039

Section: 6

Instructor: Dr. EDWARD SADER

Partner's Name: IBRAHIM

Date: 5\10\2004

Abstract:

- The aim of the experiment: is to show the difference and the similarities between VR, VC and VL graphs, and to measure the phase shift between each of them and the driving voltage.

- The method used: by using the time divisions on the CRO screen within the internal mode, and intercepts within the external mode, to analyze the graphs of our circuit which is powered by the signal generator with AC current.

Theory:

In the RLC circuit , which is powered by the AC supply the current equal to : I=Є(t)/Zeq and  Zeq= ZR+ZL+ZC where ,

ZR is the resistive impedance which equals R.

ZC is the capacitive impedance which equals –j/ωc.

ZL is the inductive impedance which equals jωL.

While the quantity 1/ωc is the capacitive reactance

and the ωL is the inductive reactance, an we note here

that the unit of the impedance is the ohm ( Ω ) and J= √ -1 , so the impedance is define as a complex number after dealing with it we get the following value for the current  , I=Io cos (ωt + Φ)

where  Io=Єo/ R2 + (ωL- 1/ωc)2   and     Φ= tan-1{1/ωc – ωL) R}.

The current heads the voltage by a time interval ∆t that is dependent on the frequency of the cosine function, so there is a phase shift between the given by:

Φ=ω∆t

- The voltage across the resistor =	VR= IR = R Io cos (ωt + Φ)

- VL can be obtained as:  VL = L dI (t) = Ld (Io cos (ωt + Φ))   , and hence

VL = -ωLIo sin (ωt+Φ)

- The voltage across the capacitor is:   Vc = 1/C∫ I(t)dt =  1/C∫Io cos (ωt + Φ) dt

Vc = Io sin (ωt+Φ) / ωc

We notice here that VR is just the current multiplied by the resistance (ohm's law) and VC and VL is just the current multiplied by the capacitive and inductive resistance with a phase shift of  л/2 .Besides , we note that the  phase shift between the current and the voltages across the different circuit elements related to Φ which is  function of ω (Φ=ω∆t)