College of Engineering

Civil Engineering Department

Surveying 1 –ENCE330

Experiment # 3

The

Objective:

The purpose of this experiment is to book the measurements of some field and mapping it.

Theory:

The field measurements should be booked in an organized way , and way of booking these measurement by dividing the paper in the middle by two line , with a separation of about 1.5 cm .these two line represent a plait  chains line and the spaces between them is used for booking distances along the chain line the spaces to either side of the   chain line are used to show measurement to  points of details in chain surveying , the topographical and made features are located and mapping by measuring with the tape the length of series of selected reference straight line , called chain line , and the locating points on the ground relative to these lines .

The plotting process has been facilitated lately by the vast developments in software and computer technology, computer programs, such as AutoCAD, are how

There are two ways:

-The method of ties; in this method, a point (which could be an edge of a building) is located by measuring two reasonable distances, called ties, between this point and two selected points on the chain line .

The method of    -