Physics laboratory

Phy.   112

Experiment 6

Damped Oscillation

Muhannad Hassan Abu Ayhour 1030691

Jeries Boloutta                          1030277

Sec.:1

Instructor:  Dr. Wae’l Qara;en

1/ 8/ 2006

Abstract:

This experiment aims to analyise and compare the behavior of an RLC circuit during the three different damping stages. It aims to find the range of R and t1/2 for each of the three stages by connecting an RLC circuit with a variable resistance to a CRO from which we can obtain our readings. The main results are:

i. Under damping:

  - t1/2 (exp) =  5*10-5 sec;      t1/2 (th) =  5.7*10-5 sec

  - Range for RL [0, 1800) Ω.

  - δ exp =13863       δ th=12100

- Critical damping:

- t1/2 (exp) =  1.5*10-5  sec.

- Value for RL =1800Ω.

- δ exp=46209.5

- Over damping:

- t1/2 (exp) =  3.5*10-5 sec.

- Range for RL [1800, ∞) Ω.

- δ exp=19804.2

Apparatus:

1- Resistance decade box.

2- Inductor (10 mH)

3- Capacitor (0.1μF)

4- Signal generator.

5- Oscilloscope

Theory:

For an RLC circuit like the one connected in the Fig the charge that accumulates on the capacitor can be given by the following relation: Q(t)=A1eλ+t+A2eλ−t---eq (1), where A1 and A2 are both constants and:

λ±=−R2L±(R2L)2−1LC --- eq (2).

When dealing with a circuit in which only R is variable we get three cases depending on R’s value:

*Underdamping:

For a very small value of R we get:R/2L2<1/LC. Therefore Q(t)=Q0e−Rt2Lcos((1LC)−R2L2+θ0) this is a sinusoidal function with exponentially decaying amplitude. We can define the half life (t1/2) of the amplitude (envelope) as the value when Q02=Q0e−Rt122L⇒t12=(2L)ln(2)R--- eq(3). Using .this method we can even measure the overall resistance of the components of our circuit by substituting t1/2 as we get it from the CRO when RL is set to 0.as this  fig shown .

*Critical damping:

This is considered a turning point between underdamping and overdamping: here (R2L)2=(1LC)which means that λ+=λ−=−R2L. And the charge decays exponentially according to the relation: Q(t)=Ae−Rt2L+Bte−Rt2Lin which A and B are both constants.

*Overdamping:

As a final stage, when the resistance is large enough: (R2L)2>1/LC, in this case equation (1) would apply as it is, and both terms in that equation would decay  exponentially decay.

Procedure:

- We built the RLC circuit shown in the Fig. (Using a variable resistance for R) and then we connected it to the CRO which was set to work in its internal mode.

- We set the variable resistance to be zero, and we measured t1/2, then using equation (3) we measured the total resistance of all the components (call this Rcomp).

- We set the variable resistance to a small value RL, then we found t1/2 experimentally using the CRO and theoretically using equation (3) (R in equation (3) was taken to equal Rcomp.+RL)

- Next, we increased the value of the variable resistance until the sinusoidal graph on the CRO turned into that of an exponential decay. This meant that we moved from the stage of underdamping to that of critical damping.

- We then measured t1/2 during critical damping.

- To reach the final stage, we increased the resistance to reach the stage of overdamping. Our final step was to measure t1/2 for this value of RL in the overdamping stage.

  - Data and Calculations :

  - C=0.1μF.

  - L=10mH.

- Underdamping:

t12(whenRL=200)=(2L)ln(2)ΣR=(2)(10×10−3)ln(2)(200+42)=5.7×10−5sec.(Theoretically)

t12=   50×10−6sec      (Experimentally)

δ exp= ln(2)t12= 13863           δ th = R2L=12100

- Critical damping:

- RL at which critical damping occurs ≈ 1800Ω

- t12=   1.5×10−5sec

- δ exp= 46209.5

- Overdamping:

- t12=3.5*10-5 sec

- δ exp= 19804.2

- Theory. Whereλ±=−R2L±(R2L)2−1LC. By substituting R = (RL+Rcomp.) = (1800+42) = 1842Ω,     we get: λ+ = -5599.1 sec-1, λ- = -957101sec-1

  - Analysis:

By analyzing the graphs, data and the numerical results we can see that:

- For the underdamping stage, the range of RL is [0, 1800) Ω.

- Critical damping occurs at RL = 1800 Ω.

- For the overdamping stage, the range of RL is approximately [1800, ∞) Ω.

- t1/2 found experimentally for the underdamping stage is close somehow to that found by theory within an order of about 10-5 sec.

- The values of RL, obtained experimentally and by theory, at which we move from the underdamping to the critical damping stage closely matched.

- From the decay constants we calculated above for the critical and overdamping stages, it is not very easy to conclude where the decaying occurs faster. But by comparing the graphs we got on the CRO for both stages ,we can see that in the critical damping stage we get a steeper graph which means that decaying occurs at a faster rate. Additionally, we can come to the same conclusion by looking at section IV Data; there we can see that t1/2 in the critical damping stage is less than that in the overdamping stage and t1/2 is inversely proportional to the decay rate.

  - Conclusion:

i. Under damping:

  - t1/2 (exp) =  5*10-5 sec; t1/2 (th) =  5.7*10-5 sec

  - Range for RL [0, 1800) Ω.

δ exp= ln(2)t12= 13863           δ th = R2L=12100

- Critical damping:

- t1/2 (exp) =  1.5*10-5 sec.

- or RL= 1800Ω.

Value f δ exp= 46209.5

- Over damping:

- t1/2 (exp) =  3.5*10-5 sec.

- Range for RL [1800, ∞) Ω.

- δ exp= 19804.2

We can conclude that for each of the three damping stages occurs at a different range of R: it begins with the underdamping stage for small values of R, then at a specific value of R we reach the critical damping stag, finally, we get to the overdamping stage for all larger values of R. The RLC circuit behaves in a very different way in each stage.

When calculating t1/2 for underdamping, the values obtained for t1/2 experimentally and theoretically differed by a small amount ,	To obtain the value of RL at which we move from one stage to another we had to change the value of the variable resistance until we got, what we think is the correct graph. So it was a matter of human judgment which means precision was somewhat compromised.

Finally, in this experiment, unlike previous ones, we have taken into consideration the resistance of other components of the circuit which means that we were able to reduce eliminate a source of error that was present in previous experiments.