BIRZEIT UNIVERSITY

PHYSICS DEPARTMENT

LAB 112

_______________________________________EXPERIMENT 7_______

*Impedance and Reactance*

*Student Name: mohammad modallal

*Student ID: 1120174

*Partner's Name: Ahmad  Dar Khalil

*Partner's ID :   1120443

*Section: 13

*Instructor: Dr.ziad

*Date: 23/3/2013

Abstract    :                                                                               ♣

♦  The aim of the experiment: is to determine the phase difference between the voltages of components which are inductor, resister and capacitor in an AC powered RLC circuit. Then to calculate the frequency at which the phase shift is zero. And to find the characteristics from VR, VC and VL graphs.

♦The method used: by adjusting the CRO on the internal mode and using the time divisions on it to analyze our circuit in finding the phases (Φ ). Then by using the external mode and changing the frequency of the input wave in order to take our data to draw the graph of (f vs. Φ). So we can find the frequency.

Apparatus:

1- Resistor 1 KΩ.

2- Capacitor 0.022 μF.

3- Inductor 10mH.

4- Signal generator.

5- Oscilloscope.

6- Circuit board (bread board).

Theory:

In our A.C. –powered RLC circuit that shown in fig(1) the current in the circuit is given by :

£ = IR + LdI/dt +Q/C

I(t) = £(t)/Zeq

£(t) =  £0 cos (ωt)

£ (t) = Id2Q/dt2 + RdQ/dt + Q/C

I(t) = £0/r cos (ωt) = I0 cos (wt)

The current is also a sinusoidal function of time.

Where Zeq = Zr + Zc + Zl

Since:

Zr = R , Zc = -J/ωC , Zl = JωL

Since 1/ωc is the capacitive reactance and ωL is the inductive reactance.

To find the impedance we need some knowledge about the complex number to treat the values of the current in the circuit

I(t) = £(t)/ ( (R2 + (ωL – 1/ωC)2)-1/2

And Φ = tan-1 (ωL + 1/ωC)/R

In the following fig (2) it sows a plot of but the voltage and the current in the circuit on a common time scale. It is obvious from equation (1) that the current heads or lags the voltage by a time interval that is dependent on the frequency of the cosine function. In other words, there exists a phase shift Φ = ω∆t between them.

The voltage across the inductor Vl can be obtained as follows

VI = LdI(t)/dt = Ld/dt ) I0 cos(ωt + Φ))

Vl = -ωLI0 sin (ωt + Φ )

Note that Vl is just the current multiplied by the inductive reactance with a phase shift of Π/2 .

And to find the voltage scross the resistors is

Vr = RI(t) = R0 cos (ωt + Φ )

From ohm’s law Vr is just the curentmultiplied by the resistance.

To find the voltage across the capacitor by:

Vc = 1/C ⌠I(t) dt = 1/c ⌠I0 cos (ωt + Φ)

Vc = I0/ωsin (ωt +Φ)

Since the Vc is the current multiplied by the capacitive reactance with a phase shift of Π/2 introduced.

The phase shifts between the current and voltages across the different circuit elements in fig(3) are also related to Φ which is a function of ω .

In order to find the value of he phase shift between two harmonic functions using the oscilloscope:

- Set the oscilloscope on the X-Y mode.

- Connect one function to X-channel and the other to the Y-channel.

- On the screen you will see an ellipse.

- let Vx = Vx0 cos (ωt )

And

Vy = Vy0 cos (ωt + Φ)

Where Φ is the phase shift between the two functions .

- at point a , defined in fig ( 4 ) , Vx has its maximum value , therefore , ωt  = 0 , and

Vy = Vy = Vy0 cos(Φ)

Therefore,

Φ = cos-1( Vy/ Vy0)

Both Vy and Vy0 are measured as shown in fig (4) .

Procedure:

1- Connect the circuit shown in fig (5) .

2- For five different frequencies find the phase shift between the driving voltage and the current.

3- Display Vr , Vc , Vl on the oscilloscope screen and measure their characteristics . Measure the phase shift between each of them and the driving voltage.

Conclusion:

We notice that the phase shift becomes zero when the frequency becomes 5.0 KHz. and we notice that the input voltage in phase with the output voltage when using a resistor. And there is a phase shift when using both inductor and capacitors…