____________________________________________________

Physics Laboratory 112

Electricity & Magnetism

Experiment 8

Resonance

Student Name: Amin abu hamdeh

Student ID: 1030957

Partner Name: Khaled Najada

Partner Name : Amer Alhamed

Instructor: Dr. Ziyad Izzat

Section: 6

Date: 9/5/2005

Abstract:

- Aim of the experiment: is to determine the shape of the frequency of the resonance curve, measure its bandwidth and to determine the quality factor.

- Method used: by measuring the characteristics of the LRC circuit, powered by a signal generator, at different frequencies.

Theory:

In an AC powered RLC circuit the current passing through the circuit is:

_

I0=V0 / R2+(ωL- 1\ ωC)2

From the equation we notice that I0 is maximum when:  ω L = 1/ ωC  and here, under this condition, we consider ω to be the natural angular frequency of the circuit

ω0=1/ (LC)

The resonance: is the phenomenon when the current in the circuit assumes its maximum value when the driving voltage frequency equals the natural frequency of RLC circuit.

And at resonance I0=v0/R (and the current is only limited by the resistance of the circuit)

The quality factor: it’s a measure of the sharpness of the resonance curve.

Q = ω L/R  and at resonance Q=1/R (L/C)1/2.

The bandwidth (Δω) is a practical value that measures the sharpness of the resonance curve and the frequency range between the maximum value of I and the value I0/(2)1/2

And the relation between the quality factor and the bandwidth:  Q = ω0/2(Δω)

Proof of:  Q=(1/R) (L/C)1/2

We know that Q = ω L/R

And we also know that ω=1/(LC)1/2

By substituting ω in the two equations we get our wanted result:

Q = (1/LC)1/2L/R  =  (L/c)1/2/R

Data:

C = 0.1 F        L = 10 mH

For    R1 = 1k:

| I (mA) | VR1 (V) | f (Hz) |  (Degrees) |
| --- | --- | --- | --- |
| 1.28 | 1.28 | 100 | Sin-1(4.3/4.3)= 90.0 |
| 6.20 | 6.20 | 500 | Sin-1(4.1/4.3)= 72.5 + |
| 11.20 | 11.20 | 1000 | Sin-1(3.45/4.2)=55.2 |
| 16.50 | 16.50 | 2000 | Sin-1(2.2/4.2)= 31.6 |
| 18.75 | 18.75 | 3000 | Sin-1(1.25/4.1)=17.8 |
| 19.25 | 19.25 | 4000 | Sin-1(0.5/4.0)= 7.2 |
| 19.50 | 19.50 | 5000 Resonace | Sin-1(0/4)= 0.0 |
| 19.25 | 19.25 | 6000 | Sin-1(0.4/4)= 5.7 - |
| 18.50 | 18.50 | 8000 | Sin-1(1.25/4.1)=17.8 |
| 17.75 | 17.75 | 10000 | Sin-1(1.6/4.1)= 23.0 - |
| 15.50 | 15.50 | 15000 | Sin-1(2.7/4.2)= 40.0 |
| 13.40 | 13.40 | 20000 | Sin-1(3.55/4.2)=57.7 |
| 10.60 | 10.60 | 30000 | Sin-1(3.8/4.2)= 64.8 |
| 7.60 | 7.60 | 40000 | Sin-1(4.1/4.3)= 72.5 |
| 6.60 | 6.60 | 50000 | Sin-1(4.2/4.3)= 77.6 |

For   R2 = 2k:

| I (mA) | VR1 (V) | f (Hz) |  (Degrees) |
| --- | --- | --- | --- |
| 1.35 | 2.70 | 100 | Sin-1(4.2/4.3)= 77.6 |
| 5.70 | 11.40 | 500 | Sin-1(3.6/4.3)= 56.8 + |
| 7.88 | 15.75 | 1000 | Sin-1(2.5/4.3)= 35.5 |
| 9.75 | 19.50 | 2000 | Sin-1(1.3/4.3)= 17.6 |
| 10.13 | 20.25 | 3000 | Sin-1(0.7/4.3)= 9.4 |
| 10.25 | 20.50 | 4000 | Sin-1(0.4/4.3)= 5.3 |
| 10.38 | 20.75 | 5000 Resonace | Sin-1(0/4.3)= 0.0 |
| 10.25 | 20.50 | 6000 | Sin-1(0.2/4.3)= 2.7 |
| 10.00 | 20.00 | 8000 | Sin-1(0.6/4.2)= 8.2 |
| 9.88 | 19.75 | 10000 | Sin-1(1.9/4.2)= 12.4 - |
| 9.50 | 19.00 | 15000 | Sin-1(1.6/4.2)= 22.4 |
| 9.00 | 18.00 | 20000 | Sin-1(2.1/4.2)= 30.0 |
| 8.00 | 16.00 | 30000 | Sin-1(3/4.2)= 45.6 |
| 7.00 | 14.00 | 40000 | Sin-1(3.75/4.2)=61.8 |
| 6.00 | 12.00 | 50000 | Sin-1(3.8/4.2)= 64.8 |

Calculations and results analysis: