### Physics Department

### Physics 112

### Experiment No.8

### Resonance

### Student’s Name: Mudar Abed Rabbo

### Student’s No.: 1041433

### Partner’s Name: Khairallah

### Partner’s No.: 1040132

### Section: 6

### Date: 7/12/2005

### Instructor: Dr. Ziyad Izzat

Abstract:

The aim of the experiment: is to find out the resonance frequency of the RLC circuit.

The method used: is by using the CRO to find out the voltage changes due to the change of frequency.

The main result:

When R = 1 kΩ:

fo=4.78x103kHz

Q=ωo2Δω=30×1032×100×103=2×π×4.78×1032×2×π×15.9×103=0.15

When R = 2 kΩ:

fo=4.78×103kHz

Q=ωo2Δω=300002×1700000=2×π×4.78×1032×2×π×27.1×103=0.088

Theory:

The amplitude of the current passing through the circuit shown in  is given by, Io=VoR2+ωL−1ωC2.

As we can see, we can get a maximum value of Io when

ωL=1ωC

⇒ωo=1LC under such a condition ω is the natural angular frequency of the circuit.

The resonance is that the current assumes its maximum value when the deriving voltage frequency equals the natural frequency of the RLC circuit.

Fig.2 shows a plot of the value of Ioas a function of ω. At resonance Io=VoR and the value of the current is only limited by the resistance of the circuit.

The quality factor:

A measure of the sharpness of the resonance curve is a quantity called the quality factor (Q), which is defined as Q=ωLR

At resonance Q=1RLC

Fig.3 shows a plot of resonance curve for different combinations of R, L, and C.

The band width (∆ω) is a practical value that measures the sharpness of the resonance curve. The band width (∆ω) is the frequency range between the maximum value of Ioand the value Io2, see fig.2.

The quality factor is related to the band width as follows:

Q=ωo2Δω.

Data:

| f kHz |  |  | VR1 | IR1 |  | VR2 |  | IR2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.2 |  |  | 0.36 | 0.36 |  | 0.7 |  | 0.35 |
| 0.4 |  |  | 0.65 | 0.65 |  | 1.1 |  | 0.55 |
| 0.6 |  |  | 0.9 | 0.9 |  | 1.5 |  | 0.75 |
| 0.8 |  |  | 1.1 | 1.1 |  | 1.7 |  | 0.85 |
| 1 |  |  | 1.35 | 1.35 |  | 1.9 |  | 0.95 |
| 2 |  |  | 2 | 2 |  | 2.2 |  | 1.1 |
| 3 |  |  | 2.2 | 2.2 |  | 2.3 |  | 1.15 |
| 4 |  |  | 2.2 | 2.2 |  | 2.4 |  | 1.2 |
| 5 |  |  | 2.3 | 2.3 |  | 2.4 |  | 1.2 |
| 6 |  |  | 2.2 | 2.2 |  | 2.3 |  | 1.15 |
| 8 |  |  | 2.2 | 2.2 |  | 2.3 |  | 1.15 |
| 10 |  |  | 2.1 | 2.1 | 2.3 |  | 1.15 |  |
| 20 |  |  | 1.5 | 1.5 | 2 |  | 1 |  |
| 40 |  |  | 0.95 | 0.95 | 1.6 |  | 0.8 |  |
| 60 |  |  | 0.65 | 0.65 | 1.25 |  | 0.625 |  |
| 80 |  |  | 0.5 | 0.5 | 1 |  | 0.5 |  |
| 100 |  |  | 0.4 | 0.4 | 0.78 |  | 0.39 |  |

Calculations:

The band width for the first resonance curveIR1vs.f:

Δω=2πΔf=(130−30)x1000=100x103

The band width for the second resonance curveIR2vs.f:

Δω=2πΔf=(200−30)x1000=170x103

First case R=1kΩ:

The resonance frequency:

fo=4.78x103kHz (Practically)

The angular resonance frequency:

ωo=1LC=31622.78 (Theoretically)

ωo=2πfo=2×3.14×4.78×103=300184 (Practically)

The quality factor:

Q=1RLC=11×10310×10−30.1×10−6=0.32 (Theoretically)

Q=ωo2Δω=30×1032×100×103=2×π×4.78×1032×2×π×15.9×103=0.15 (Practically)

Second case R=2kΩ:

The resonance frequency:

fo=4.78×103kHz (Practically)

The angular resonance frequency:

ωo=1LC=31622.78(Theoretically)

ωo=2πfo=2×3.14×4.78×103=300184(Practically)

The quality factor:

Q=1RLC=12×10310×10−30.1×10−6=0.158 (Theoretically)

Q=ωo2Δω=300002×1700000=2×π×4.78×1032×2×π×27.1×103=0.088 (Practically)

Analysis of results:

When the resistance was 1 kΩ the resonance frequency was almost the same as when the resistance was 2 kΩ. But the quality factor was larger when R was smaller.. And this somehow copes with the theoretical ideas.