____________________________________________________

Physics Laboratory 112

Electricity & Magnetism

Experiment  5

Resonance

Student Name: YASER ARAFAT ALHAWWARIN

Student ID: 1031039

Section:6

Instructor: Dr. EDWARED SADER

Partner's Name: IBRAHIM

Date: 30/11/2004

Abstract:

- Aim of the experiment: is to determine the shape of the frequency of the resonance curve, measure its bandwidth and to determine the quality factor.

- Method used: by measuring the characteristics of the LRC circuit, powered by a signal generator, at different frequencies.

Theory:

In an AC powered RLC circuit the current passing through the circuit is:

_

I0=V0 / R2+(ωL- 1\ ωC)2

From the equation we notice that I0 is maximum when:  ω L = 1/ ωC  and here, under this condition, we consider ω to be the natural angular frequency of the circuit

ω0=1/ (LC)

The resonance: is the phenomenon when the current in the circuit assumes its maximum value when the driving voltage frequency equals the natural frequency of RLC circuit.

And at resonance I0=v0/R (and the current is only limited by the resistance of the circuit)

The quality factor: it’s a measure of the sharpness of the resonance curve.

Q = ω L/R  and at resonance Q=1/R (L/C)1/2.

The bandwidth (Δω) is a practical value that measures the sharpness of the resonance curve and the frequency range between the maximum value of I and the value I0/(2)1/2

And the relation between the quality factor and the bandwidth:  Q = ω0/2(Δω)

Proof of:  Q=(1/R) (L/C)1/2

We know that Q = ω L/R

And we also know that ω=1/(LC)1/2

By substituting ω in the two equations we get our wanted result:

Q = (1/LC)1/2L/R  =  (L/c)1/2/R

Data:

C = 0.1 F        L = 10 mH

For    R1 = 1k:

| I (mA) | VR1 (V) | f (KHz) |  (Degrees) |
| --- | --- | --- | --- |
| 0.85 | 0.85 | 0.5 | Sin-1(1.6/1.8) |
| 1.6 | 1.6 | 1 | Sin-1(2.6/3.2) |
| 2.4 | 2.4 | 2 | Sin-1(3/4.8) |
| 2.8 | 2.8 | 3.5 | Sin-1(1.5/5.6) |
| 3 | 3 | 5.2 | Sin-1(0/6) |
| 2.2 | 2.2 | 17 | Sin-1(3/4.4) |
| 2.75 | 2.75 | 10 | Sin-1(2.2/5.5) |
| 2 | 2 | 20 | Sin-1(3/4) |

For   R2 = 2k:

| I (mA) | VR1 (V) | f (KHz) |  (Degrees) |
| --- | --- | --- | --- |
| 1.6 | 1.6 | 0.5 | Sin-(4.2/4.3)= 77.6 |
| 2.4 | 2.4 | 1 | Sin-1(3.6/4.3)= 56.8 |
| 1.2 | 1.2 | 3.5 | Sin-1(2.5/4.3)= 35.5 |
| 3 | 3 | 5.2 | Sin-1(1.3/4.3)= 17.6 |
| 3 | 3 | 10 | Sin-1(0.7/4.3)= 9.4 |
| 2.85 | 2.85 | 17 | Sin-1(0.4/4.3)= 5.3 |
| 2.9 | 2.9 | 20 | Sin-1(0/4.3)= 0.0 |

Calculations and results analysis:

Q=