# Contents

| Subject | Page |
| --- | --- |
| # Abstract | 2 |
| ## Theory & definitions | 2 |
| Apparatus | 4 |
| ## Procedure | 4 |
| Data | 4 |
| Computations | 4 |
| Conclusion | 7 |

# Abstract

Aim : is to study RC circuits used as filters , examining the characteristics of both types high and low pass filters . In addition , to study the relation between the input and output voltages of such filters .

Method used : Plotting a graph of the attenuation factor , A vs. the frequency & finding -3dB from the plot , after collecting the required data by the aid of the oscilloscope .

Results :

Practically :

Low pass w-3dB = 44.6 krad/s

High pass w-3dB = 54 krad/s

Theoretically :

w-3dB = 1/RC = 45.5 krad/s

Theory

Filters are classified into three types : high pass , low pass & band pass filters :

Low pass RC filters :

For the circuit shown in fig.* the relation between the input & output voltage is as follows :

Vout = A*Vin

Where , The attenuation factor , A is determined as follows :

A = Vout/Vin

= 1/( 1+ w2C2R2 )1/2

= 1/[(1 + (w/w-3dB)2 ]1/2

where , w-3dB = 1/RC

If this kind of filters are functioning in the highly attenuated region then the relation between the output and input voltage is determined as follows :

##### Vout(t) = (1/RC)Vin(t)dt

High pass RC filters :

For the circuit shown in fig.* the  attenuation factor , A is determined as follows :

A = 1/[(1 + (w-3dB /w)2 ]1/2

If this kind of filters are functioning in the highly attenuated region then the relation between the output and input voltage is determined as follows :

##### Vout(t) = RC(dVin(t)/dt)

Apparatus

- 1 KΩ resistor .

- Capacitor ( .022 F )

- Signal generator .

- Oscilloscope .

# Procedure

- Connect a circuit as shown in fig.1

- For some different frequencies , both the input and output voltages are measured by the aid of the oscilloscope .

- The previous step is repeated with the circuit shown in figure.3.

- For the circuit shown in fig.1 use the various types of wave functions of the input voltage and the function of the output voltage is noticed .

- The previous step is repeated with the circuit shown in figure.3

Data & Calculations

Table 1 : Data of Vin , Vout with  and A for low pass filters :

|  | VR | A |
| --- | --- | --- |
| 1.00 | 10.5 | 1 |
| 3.00 | 10 | .952 |
| 7.00 | 7.5 | .714 |
| 9.00 | 6.5 | .62 |
| 10.00 | 6 | .57 |
| 20.00 | 4.5 | .43 |
| 30.00 | 3 | .28 |
| 50.00 | 1.75 | .166 |
| 100.00 | 1 | .095 |

Table 2 : Data of Vin , Vout with  and A for high filters :

|  | VR | A |
| --- | --- | --- |
| 1.00 | 1.75 | 0.166 |
| 3.00 | 3.75 | 0.357 |
| 7.00 | 6.50 | 0.620 |
| 9.00 | 7.50 | 0.714 |
| 10.00 | 8.00 | 0.760 |
| 20.00 | 8.50 | 0.810 |
| 30.00 | 9.50 | 0.900 |
| 50.00 | 9.75 | 0.930 |
| 100.00 | 10.00 | 0.950 |

Calculation of w-3dB :

Practically :

Low pass w-3dB = 7.1*2* = 44.6 krad/s

High pass w-3dB = 8.6*2* =  54 krad/s

Theoretically :

w-3dB = 1/RC = 1/[(1000)*(0.022*10^-6)] = 45.5 krad/s

# Observations of the output wave forms with respect to various types of input wave forms :

# Conclusion

In summary the results of the test where as follows :

Practically :

Low pass w-3dB = 44.6 krad/s

High pass w-3dB = 54 krad/s

Theoretically :

w-3dB = 1/RC = 45.5 krad/s

Mainly , it is clear that the practical results are almost close to the theoretical values . In the other hand , the  output wave forms with respect to various types of input wave forms were following the theoretical formulas . The slight differences that are existed between the theoretical and practical variables could be justified by the difficulty to record the exact required value  from the screen of the oscilloscope , where the thickness of the line describing the behavior of the current or voltage is plenty thick , which in terms makes the mission seems hardly to be done .

Another reason could be the presence of some excess resistance in the elements that are not taken exactly into consideration & in the calculations and in terms , badly effects the results .