Abstract :

The aim of experiment is to check the two theorem, the thevenin and Norton techniques . using this two theorem in this electric circuit and find ( Ieq ) ,( Eeq ) , Req ,IL , by using this theorem and then a measuring this value and compared it with the value that calculated by using theorem . the result of this experiment was :

( Ieq )=  14.9   /  Req= 657    /  I3= 1.85       .

THEOREM :

* Thevenins theorem states that any network of resistors and supplies having two at put terminals can we replaced by series combination of a voltage source (  Eeq  ) and a resistors (Req) .

Thevenins techniques is especially important in ablating the current passing through and the voltage across any resistor ( ab)

To find the  Eeq and   :

- remove Rl and  and calculate the voltage difference at the network terminal .

- remove Rl kill all the square with network  through replacing voltage sources by short circuits and current source by open circuit and calculate equivalent resistance  Req .

- Construct thevenins equivalent circuit and calculate the current passing through and the voltage drop across Rl .

* Norton suggest similar techning that guess along the following to find Req :

- use exactly the same procedure used by thevenins to find Req .

- replace  Rl  by a short circuit ( wire ) and  calculate the current passing through wire , call it (Ieq ) .

- construct Norton's equivalent circuit using a current source and parallel resistance , calculate the current passing through and the voltage difference across Rl . those should be the same value at tained in org network..

- using thevenins and Norton equivalent circuit techning to find the value off current :

*  thevenins :

Remove   Rl and calculate Req = R1  R2 = R1*R2 / R1 +R2 ,

Calculate   as following using kirchoffs loop :

E1- E2  = I( R1+R2)

=     E1-I*R1

=     E1 – (E1-E2)*R1 / R1+R2

I R2         =    Eeq   /   Req + R2 .

* Norton's :

Ieq = I1 +I2                 ============          Ieq = E1/R1 + E2/R2 .

– IRL  = IRL  * RL       =========    IRL= Ieq * Req / Req + RL .

Apparatus :

Two voltage supplies                       circuit board

Digital MultiMeter                          three carbon resistors .

Procedure :

- connect the circuit shown in figure .

- remove R2 kill both source and measure the value of R2 .

- connect the sources back and measure (  Ieq  ).

- Replace R2 by a short circuit and measure Ieq .

- Construct thevinins equivalent circuit and measure IRL .

- Connect Norton equivalent circuit .

CALCULATION ;

Theoretical  value thevenins circuit :

Req =  R1*R2 / R1+R2  =   (  1*2.2) * 10 / ( 1+ 2.2) *10 = 688

Eeq  = E1 – ( E1-E2) *R1 / R1 +R2  =  12– (12 -6)*10 /(1+2.2)*10

= 10.13  V

IRL = I3 =  Eeq / Req + R3   =  10.13/(0.688 + 4.7 )*10 =  1.88  mA

Theoretical value Norton's circuit :

Req =  R1*R2 / R1+R2  =   (  1*2.2) * 10 / ( 1+ 2.2) *10 = 688

Ieq = E1/R1 +E2/R2 = 12/10  + 6/2.2*10 = 14.73   mA

I3   =   Ieq* Req  /  Req  + R3  = (14.73 * 0.688)./( 0.688 + 4.7)

= 1.88  mA

Resulted and conclusion :

After the experiment was finished we towed that the value of

Ieq   =     14.9

Eeq  =  10.2

Req  =  657

I3   =  1.85

And this values nearly equal the values that we were by using the two theorem . but there was some after current through wire as from the machines .