Factors for calculating control limits for control charts

For MEANS

| Sample size (n) | Constant (dn) | Factors for warning limits 1.96/dn√n | Factors for action limits 3.09/dn√n |
| --- | --- | --- | --- |
| 2 | 1.128 | 1.23 | 1.94 |
| 3 | 1.693 | 0.67 | 1.05 |
| 4 | 2.059 | 0.48 | 0.75 |
| 5 | 2.326 | 0.38 | 0.59 |
| 6 | 2.334 | 0.32 | 0.50 |
| 7 | 2.704 | 0.27 | 0.43 |
| 8 | 2.847 | 0.24 | 0.38 |
| 9 | 2.970 | 0.22 | 0.35 |
| 10 | 3.078 | 0.20 | 0.32 |

Factors for calculating control limits for control charts

For RANGES

| Sample size | Upper warning limit | Lower warning limit | Upper action limit | Lower action limit |
| --- | --- | --- | --- | --- |
| 2 | 2.81 | 0.04 | 4.12 | 0.00 |
| 3 | 2.17 | 0.18 | 2.98 | 0.04 |
| 4 | 1.93 | 0.29 | 2.57 | 0.10 |
| 5 | 1.81 | 0.37 | 2.34 | 0.16 |
| 6 | 1.72 | 0.42 | 2.21 | 0.21 |
| 7 | 1.66 | 0.46 | 2.11 | 0.26 |
| 8 | 1.62 | 0.50 | 2.04 | 0.29 |
| 9 | 1.58 | 0.52 | 1.99 | 0.32 |
| 10 | 1.56 | 0.54 | 1.94 | 0.35 |