Birzeit University

Civil Engineering Department – Surveying for Civil Engineering (CE337)

Final Exam

Part II (50 minutes)

Instructors: A. Arafat & K. Abaza				Student Name________

- A simple circular curve with 56o central angle and 250m radius. Determine assuming PC station = 348.11m:

- The minimum number of points needed to set out this curve.		   (6%)

- The distance and deflection angle needed to set out the 11th point. 		   (6%)

- The simple circular curve in problem (1) was shifted 2m inside to allow for the inclusion of left and right transition curves. Determine assuming 539.25m back tangent length:

- The station of junction points To, T1, and T2.					    (6%)

- The deflection angle and distance needed to set out sta. 440.0 m from To.	    (7%)

- The deflection angle and distance needed to set out sta. 540.0 m from T1.	    (5%)

- A vertical alignment with G1=-5%, G2=-3%, VPI sta. = 821.89m, VPI elev. = 224.83m, and vertical curve length = 240m. Determine the elevation of sta. 920.0m and sta. 1180.0m.									  (10%)

- For the mass diagram shown below, determine assuming 300m free hauling distance:

- Borrow/excess volume. 							    (2%)

- Location of borrow/excess volume if balance line located at 2000 ordinate.    (2%)

- Overhaul quantity (m4) if balance line located at 1000 ordinate.		    (6%)