Faculty of Engineering & Technology

Electrical & Computer Engineering Department

DIGITAL ELECTRONICS AND COMPUTER

ORGANIZATION LABORATORY - ENCS2110

Report #2

Experiment #5

SEQUENTIAL LOGIC CIRCUITS

Prepared by: 					ID:

Ro’a Nafi                                               1201959

Partners: Manar Shawahni   1201086

Leelyan Karajah   1201191

Instructor: Dr. Ahmad Alyan
Assistant: Mahmoud Hussain

Section: 13

Date:  16/4/2022

# Abstract

This experiment aims to understand the differences between combinational and sequential Logic circuits. In addition, to understand the operating principles of counters. study the operating principles and applications of various flip-flops, synchronous counters and asynchronous counters, how to construct counters with JK flip-flops.

Contents

Abstract	2

List of figures	4

List of tables	5

Pre lap	6

7

8

9

Theory	10

1- Sequential Circuits	10

2-Latches	10

2.1- The SR (Set-Reset) Latch	10

2.2-The D Latch	12

3- Flip-Flops	12

3.1-D Flip-Flop	12

3.2- JK Flip-Flop	13

3.3-T Flip-Flop	13

4- Registers	13

5- Counters	14

PROCEDURE	15

1-Latches and Flip flops	15

1.1-Constructing RS latch with Basic Logic Gates	15

1.2- Constructing RS latch with control input	15

1.3-Constructing D latch with RS latch	16

1.4- Constructing JK latch with RS latch	16

2-Constructing JK Flip-flop with master- slave RS latches	17

3- Registers	18

3.1- Constructing Shift Register with D Flip-Flops	18

3.2- 4-Bit Shift Register with serial and parallel load	19

4-Counters	20

4.1- 2-bit Synchronous Counter	20

4-2: 3-bit (divide-by-eight) Ripple Counter	21

4.3- BCD Counter	22

Task 2:	23

Task3:	23

Conclusion	24

DISCUSSION	24

References:	26

# List of figures

Table 2:SR latch with NAND gates                                 Figure 1:SR latch with NAND gates	11

Table 3:SR latch with NOR gates                          Figure 2:SR latch with NOR gates	12

Table 6: D latch                                                                                 Figure 3:D latch	13

Figure 4:negative edge flip flop	13

Figure 5:D latch	13

Table 7:JK Flip-Flop                         	                 Figure 6:JK Flip-Flop	14

Table 8:T flip flop	     Figure 7:T Flip-Flop	14

Figure 8:: 4-bit Register	14

Figure 9:3-bit ripple counters                                                                     Figure 10:3-bit synchronous counters	15

Figure 11:RS latch	16

Figure 12:RC latch with control	16

Figure 13:D latch	17

Figure 14:JK latch with RS latch	17

Figure 15:JK  flip flop	18

Figure 16:shift register with serial and parallel load	20

Figure 17: 3-bit Ripple Counter	22

Figure 18:3-bit Synchronous Counter.	24

Figure 19:- 0-to-5 counter	24

Figure 20::- 0-to-4 counter	25

# List of tables

Table 1:Sequential Circuits	11

Table 2:SR latch with NAND gates                                 Figure 1:SR latch with NAND gates	11

Table 3:SR latch with NOR gates                          Figure 2:SR latch with NOR gates	12

Table 4:RS latch with control input                                            Table 5:RS latch with control input	12

Table 6: D latch                                                                                 Figure 3:D latch	13

Table 7:JK Flip-Flop                         	                 Figure 6:JK Flip-Flop	14

Table 8:T flip flop	     Figure 7:T Flip-Flop	14

Table 9:RC latch	16

Table 10:RC latch with control	16

Table 11:D latch	17

Table 12: JK latch with RS latch	17

Table 13:JK  flip flop	18

Table 14:4-Bit Shift Register with serial and parallel load	20

Table 15:4-Bit parallel load:	21

Table 16:: 2-bit Synchronous Counter	21

Table 17:2-bit Synchronous Counter	21

Table 18: 3-bit Ripple Counter	22

Table 19:BCD counter	23

# Pre lap

# Theory

## 1- Sequential Circuits

A Sequential circuit is a combinational logic circuit that consists of inputs variable (X), logic gates (Computational circuit), and output variable (Z). it produces an output based on current input and previous input variables (include memory elements).

Table 1:Sequential Circuits

## 2-Latches

- Latches are used for storing binary information.

- The output change in the Latches occurs at the clock level.

### 2.1- The SR (Set-Reset) Latch

#### 2.1.1- SR latch with NAND gates:

This circuit is active low set/reset latch, when S (set) input is 0 Q goes to1

| S | R | Q | ~Q |
| --- | --- | --- | --- |
| 1 | 0 | 0 | 1 |
| 1 | 1 | 0 | 1 |
| 0 | 1 | 1 | 0 |
| 1 | 1 | 1 | 0 |
| 0 | 0 | 1 | 1 |

Table 2:SR latch with NAND gates                                 Figure 1:SR latch with NAND gates

#### 2.1.2-SR latch with NOR gates: (Pre lab):

| S | R | Q | ~Q |
| --- | --- | --- | --- |
| 1 | 0 | 1 | 0 |
| 0 | 0 | 1 | 0 |
| 0 | 1 | 0 | 1 |
| 0 | 0 | 0 | 1 |
| 1 | 1 | 0 | 0 |

R

S

Table 3:SR latch with NOR gates                          Figure 2:SR latch with NOR gates

#### 2.1.3- RS latch with control input:

| C | S | R | Next state of Q |
| --- | --- | --- | --- |
| 0 | X | X | No change |
| 1 | 0 | 0 | No change |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | indeterminate |

Table 4:RS latch with control input                                            Table 5:RS latch with control input

### 2.2-The D Latch

| C | D | Next state of Q |
| --- | --- | --- |
| 0 | X | No change |
| 1 | 0 | 0 |
| 1 | 1 | 1 |

Table 6: D latch                                                                                 Figure 3:D latch

when D = 1 and EN = 1 the gated latch D flip-flop is ENABLE and SET (Q = 1) when D = 0 and EN = 1 the latch is ENABLE and RESET (Q = 0) but when EN = 0 the latch is DISABLE no question of SET REST. That means at EN = 0, any change in input D does not affect the output (No Change Condition).

## 3- Flip-Flops

- flip-flops are used for storing binary information

- The output change in the flip-flop occurs only at the clock edge

### 3.1-D Flip-Flop

Figure 4:negative edge flip flop

| D | Q(t+1) |
| --- | --- |
| 0 | 0 |
| 1 | 1 |

characteristic equation:

Q(t+1) =D

Figure 5:D latch

### 3.2- JK Flip-Flop

| J | K | Next state of Q |
| --- | --- | --- |
| 0 | 0 | No change |
| 0 | 1 | 0 |
| 1 | 0 | 1 |
| 1 | 1 | ~Q |

Table 7:JK Flip-Flop                         	                 Figure 6:JK Flip-Flop

characteristic equation:

Q(t+1) =J.~Q(t)+ ~K. Q(t)

### 3.3-T Flip-Flop

| t | Next state of Q |
| --- | --- |
| 0 | Q |
| 0 | ~Q |

Table 8:T flip flop				 	Figure 7:T Flip-Flop

## 4- Registers

Is a collection of flip flops. A flip flop is used to store single bit digital data. For storing a large number of bits, the storage capacity is increased by grouping more than one flip flops. If we want to store an n-bit word, we have to use an n-bit register containing n number of flip flops.

Figure 8:: 4-bit Register

## 5- Counters

A special type of sequential circuit used to count the pulse is known as a counter, or a collection of flip flops where the clock signal is applied is known as counters.

The counter is one of the widest applications of the flip flop. Based on the clock pulse, the output of the counter contains a predefined state. The number of the pulse can be counted using the output of the counter.

Figure 9:3-bit ripple counters                                                                     Figure 10:3-bit synchronous counters

# PROCEDURE

## 1-Latches and Flip flops

### 1.1-Constructing RS latch with Basic Logic Gates

- Use IT-3008 module to construct the circuit shown in Figure1.

- Connect inputs A3, A4 to Pulser Switches SWA A (TTL), SWB B (TTL).

- Connect outputs F6 and F7 to Logic Indicators L1, L2.

Table 9:RC latch

Figure 11:RS latch

### 1.2- Constructing RS latch with control input

- Use IT-3008 module to construct the circuit shown in Figure2.

- Connect inputs A1, A5 to Pulser Switches SWA A, SWB B

- Connect outputs F6 and F7 to Logic Indicators L1, L2.

Table 10:RC latch with control

Figure 12:RC latch with control

The result is consistent with the theory.

### 1.3-Constructing D latch with RS latch

- Use IT-3008 module to construct the circuit shown in Figure3.

- Connect A1 to SW1; CK2 to SWA A and F6 to L1.

Table 11:D latch

Figure 13:D latch

### 1.4- Constructing JK latch with RS latch

- Use IT-3008 module to construct the circuit shown in Figure 4

- Connect CK2 to SWB B output; A1 to SW0; A5 to SW1; F6 to L1.

Table 12: JK latch with RS latch

Figure 14:JK latch with RS latch

## 2-Constructing JK Flip-flop with master- slave RS latches

- Use IT-3008 module to construct the circuit shown in Figure 5

- Connect CK2 to Pulser switch.

- Connect CK1 to SWA A output; J to SW1; K to SW0; F1, F2, F6, F7 to L3, L2, L1 and L0 respectively.

Figure 15:JK  flip flop

Table 13:JK  flip flop

This is a positive edge triggered flip flop

K =0 & j =0 give next state = present state

K =0 & j =1 give next state =0

K =1 & j =0 give next state =1

K =1 & j =1 give next state = ~present state

## 3- Registers

### 3.1- Constructing Shift Register with D Flip-Flops

- Use Block Shift Register 1 of module IT-3008

- Connect B (clear) to SW0; A (I/P) to SW1; CK to SWA A output; F1, F2, F3, F4 to L1, L2, L3, L4 respectively.

- Set SW0 to “0” to clear B and then set SW0 to “1”.

- Follow the input sequence for A(I/P) below, and observe output display at F1, F2, F3 and F4:

1) at A= “1”, send in a CK signal from SWA

2) at A= “0”, send in a CK signal from SWA

3) at A= “0”, send in a CK signal from SWA

4) at A= “1”, send in a CK signal from SWA

Data (Serial-in to Parallel-out):

- 1000

- 0100

- 0010

- 1001

Its shift 1bit to right on each clock pulse.

### 3.2- 4-Bit Shift Register with serial and parallel load

- Use Shift Register 2 module in IT-3008

- Connect Inputs A, B, C, D to SW0, SW1, SW2, SW3.

- Connect Outputs F1, F2, F3, F4 to L0, L1, L2, L3.

- Connect B1 (I/P) to DIP2.0 and A1 (MODE) to DIP2.1

Figure 16:shift register with serial and parallel load

- Connect CK (C1) to the clock generator TTL level output at 1Hz and change data at B1 with DIP2.0. Follow the input sequences for A1 in Table 5.7. Observe and record the outputs.

Table 14:4-Bit Shift Register with serial and parallel load

MODE is active low

- Connect LOAD (D1) to the clock generator TTL level output at 1Hz. Set A1 to “1”

Table 15:4-Bit parallel load:

The data has been loaded correctly

## 4-Counters

### 4.1- 2-bit Synchronous Counter

- Use IT-3007 module

- Connect CLK input to pulser switch.

- Connect counter outputs Q1 and Q0 to indication lamps.

- Apply clock pulses to CLK input

Table 16:: 2-bit Synchronous Counter

Table 17:2-bit Synchronous Counter

Apply counter outputs Q1 and Q0 to seven segment display

This counter counts from 1 to 3

### 4-2: 3-bit (divide-by-eight) Ripple Counter

- Use the IT-3007 module

- Connect CLK input to pulser switch.

- Connect counter outputs Q2, Q1 and Q0 to indication lamps.

- Apply clock pulses to CLK input

- Apply counter outputs Q2, Q1 and Q0 to seven segment display

Figure 17: 3-bit Ripple Counter

Table 18: 3-bit Ripple Counter

### 4.3- BCD Counter

- Use BCD counter (IC 7490) on IT-3008 module

- Connect C3, C4 to SW0 and SW1; D1, D2 to SW2 and SW3; F1~F4 to L1~L4, A2 to SWA A output; B2 to SWB B output.

- Connect F1 to B2, set C3, C4, D1 and D2 to ground and A2 to SWA A pulse. Measure and record the outputs F1, F2, F3, F4

Table 19:BCD counter

It should count from 0 to 9, but there was a glitch and we don't know why.

# Task 2:

Modify the circuit in Figure 5.16 to be 3-bit Synchronous Counter.

Figure 18:3-bit Synchronous Counter.

# Task3:

change the connection of counter in Figure.19 to count from:

- 0-to-5

Figure 19:- 0-to-5 counter

- 0-to-4

Figure 20::- 0-to-4 counter

# Conclusion

This experience clarified the concept of sequential circuits and memory unit (latches, flip-flop), The results of the experiment were as expected and nothing went wrong  and It took two and a half hour.

# DISCUSSION

1. Although latches are useful for storing binary information, they are rarely used in sequential circuit design, why?

- latches tend to make glitches

- Analyzing of Latch circuits is difficult because of its level sensitive property.

2.What is the disadvantage of the RS flip flop?

Its output is undefined when both inputs are 1 (S=R=1).

3.What is the difference between “synchronous” and “ripple” counters?

Ripple Counter, different flip flops are triggered with different clock, not simultaneously. While in Synchronous Counter, all flip flops are triggered with same clock simultaneously and Synchronous Counter is faster than asynchronous counter in operation.

# References:

- https://www.javatpoint.com/counters-in-digital-electronics

- Digital Lab manual.

- https://www.electrical4u.com/d-flip-flop-or-d-latch/