ENCS2110

DIGITAL ELECTRONICS AND COMPUTER ORGANIZATION LABORATORY

Experiment 10: Simple Computer Simulation

Name: Sara Totah

Student ID: 1181779

Instructor: Khader Mohammed

T.A: Enas Jawabreh

Date: 12/05/2021

# Task 1:

Modify the code to include the jump instruction.

1. Choose any opcode e.g., jump=4'b0001, you have to include the execution of jump which changes the PC to the specified address in the instruction.

| 10 | Load | 32 | 0011-0000-0010-0000b=16'h3020 |
| --- | --- | --- | --- |
| 11 | Add | 33 | 0111-0000-0010-0001b=16'h7021h |
| 12 | Store | 20 | 1011-0000-0001-0100b=16'hB014h |
| 13 | Jump | 11 | 0001-0000-0000-1011b=16'h100B |
| 32 | Data | 7 | Memory[32]=16'd7 |
| 33 | Data | 5 | Memory[32]=16'd5 |

The code added to include the jump instruction:

Discussion

Variables used:

Clock: simulates computer internal clock

- Registers: PC, IR, MBR, AR, AC

- Memory: 64*16

- State: To manage instruction cycle steps

Instruction types our program supports:

- Load: [0011]

- Store: [1011]

- Add: [0111]

- Jump: [0001]

Initialization:

- Memory:

initial begin

Memory[10] = 16'h3020; //load data at address 20Hex into AC

Memory[11] = 16'h7021; //add data at address 21Hex into AC

Memory[12] = 16'hB014; //store data into address 14Hex

Memory[13] = 16'h100B; //jump to address

- Data

Memory[32] = 16'd7; // store at address 20Hex or 32 decimal data = 7 decimal

Memory[33] = 16'd5; // store at address 21Hex or 33 decimal data = 5 decimal

- PC = 10 // the program should start from the address 10D = 0AH

- State = 0

- Instruction Cycle:

  - Instruction Fetch

    - The address in PC -> MAR

    - Data at the address saved in MAR Memory[MAR] -> IR

    - Increment PC

  - Operand Fetch

    - Take the address of the operand saved at IR -> MAR

    - Load data at the memory in MAR Memory[MAR] -> MBR

    - Store data in AC -> MBR

  - Instruction Execute //based on first 4-bits of IR

    - If instruction was ADD then AC = AC + MBR

    - If instruction was LOAD, then AC = MBR

    - If instruction was STORE then Memory[MAR] = MBR

    - If instruction was JUMP then PC = MAR

Code Explanation:

Instruction #1:  LOAD [20H]

| State 0 |  | PC 0AH |  |
| --- | --- | --- | --- |
| IR | MAR | MBR | AC |

MAR<=PC;

state = 1;

end

| State 1 |  | PC 0AH |  |
| --- | --- | --- | --- |
| IR | MAR 0000 0000 0000 1010 | MBR | AC |

1:begin

IR <= Memory[MAR];

PC<= PC + 1;

state =2;

end

| State 2 |  | PC 0BH |  |
| --- | --- | --- | --- |
| IR 0011 0000 0010 0000 | MAR | MBR | AC |

2:begin

MAR <= IR[11:0];

state = 3;

end

| State 3 |  | PC 0BH |  |
| --- | --- | --- | --- |
| IR 0011 0000 0010 0000 | MAR 0000 0010 0000 | MBR | AC |

3:begin

state = 4;

case(IR[15:12])

load:  MBR <= Memory[MAR];

add:   MBR <= Memory[MAR];

store: MBR <= AC;

endcase

end

| State 4 |  | PC 0BH |  |
| --- | --- | --- | --- |
| IR 0011 0000 0010 0000 | MAR 0000 0010 0000 | MBR 0000 0000 0000 0111 | AC |

else if (IR[15:12] == 4'h3)

begin

AC <= MBR;

state = 0;

end

| State 0 |  | PC 0BH |  |
| --- | --- | --- | --- |
| IR 0011 0000 0010 0000 | MAR 0000 0010 0000 | MBR 0000 0000 0000 0111 | AC 0000 0000 0000 0111 |

Instruction #2: ADD [21H]

| State 0 |  | PC 0BH |  |
| --- | --- | --- | --- |
| IR | MAR | MBR | AC 0000 0000 0000 0111 |

0:begin

MAR<=PC;

state = 1;

end

| State 1 |  | PC 0BH |  |
| --- | --- | --- | --- |
| IR | MAR 0000 0000 0000 1011 | MBR | AC 0000 0000 0000 0111 |

1:begin

IR <= Memory[MAR];

PC<= PC + 1;

state =2;

end

| State 2 |  | PC 0CH |  |
| --- | --- | --- | --- |
| IR 0111 0000 0010 0001 | MAR 0000 0000 0000 1011 | MBR | AC 0000 0000 0000 0111 |

2:begin

MAR <= IR[11:0];

state = 3;

end

| State 2 |  | PC 0CH |  |
| --- | --- | --- | --- |
| IR 0111 0000 0010 0001 | MAR 0000 0010 0001 | MBR | AC 0000 0000 0000 0111 |

3:begin

state = 4;

case(IR[15:12])

load:  MBR <= Memory[MAR];

add:   MBR <= Memory[MAR];

store: MBR <= AC;

endcase

end

| State 3 |  | PC 0CH |  |
| --- | --- | --- | --- |
| IR 0111 0000 0010 0001 | MAR 0000 0010 0001 | MBR 0000 0000 0000 0101 | AC 0000 0000 0000 0111 |

4:begin

if(IR[15:12] == 4'h7)

begin

AC <= AC + MBR;

state = 0;

end

| State 4 |  | PC 0CH |  |
| --- | --- | --- | --- |
| IR 0111 0000 0010 0001 | MAR 0000 0010 0001 | MBR 0000 0000 0000 0101 | AC 0000 0000 0000 1100 |

Instruction #3: STORE [14H]

| State 0 |  | PC 0CH |  |
| --- | --- | --- | --- |
| IR | MAR | MBR | AC 0000 0000 0000 1100 |

0:begin

MAR<=PC;

state = 1;

end

| State 1 |  | PC 0CH |  |
| --- | --- | --- | --- |
| IR | MAR 0000 0000 0000 1100 | MBR | AC 0000 0000 0000 1100 |

1:begin

IR <= Memory[MAR];

PC<= PC + 1;

state =2;

end

| State 2 |  | PC 0CH |  |
| --- | --- | --- | --- |
| IR 1011 0000 0001 0100 | MAR | MBR | AC 0000 0000 0000 1100 |

2:begin

MAR <= IR[11:0];

state = 3;

end

| State 3 |  | PC 0DH |  |
| --- | --- | --- | --- |
| IR 1011 0000 0001 0100 | MAR 0000 0001 0100 | MBR | AC 0000 0000 0000 1100 |

3:begin

state = 4;

case(IR[15:12])

load:  MBR <= Memory[MAR];

add:   MBR <= Memory[MAR];

store: MBR <= AC;

endcase

end

| State 4 |  | PC 0DH |  |
| --- | --- | --- | --- |
| IR 1011 0000 0001 0100 | MAR 0000 0001 0100 | MBR 0000 0000 0000 1100 | AC 0000 0000 0000 1100 |

else if (IR[15:12] == 4'hB)

begin

Memory[MAR] <= MBR;

state = 0;

end

| State 4 |  | PC 0DH |  |
| --- | --- | --- | --- |
| IR 1011 0000 0001 0100 | MAR 0000 0001 0100 | MBR 0000 0000 0000 1100 | AC 0000 0000 0000 1100 |

Memory [14H] = 0000 0000 0000 1100

Instruction #4: JUMP [1H]

else if (IR[15:12] == 4'h1)

begin

PC <= MAR;

state = 0;

end

Changes the address that should be executed next, in other words, the new address saved at the PC is the address of the new instruction to be excuted.