ENCS2110

DIGITAL ELECTRONICS AND COMPUTER ORGANIZATION LABORATORY

Name: Sara Totah

Student ID: 1181779

Instructor: Khader Mohammed

T.A: Enas Jawabreh

Date: 22/2/2021

NOR Gate Circuit:

- NOR gate used as NOT gate:

- When SW1 = 0, F1 = 1

- When SW1 = 1, F1 = 0

- Does the circuit act as a NOT gate?

Yes it does

- When SW0 = 0, F1 = 1

- When SW0 = 1, F1 = 0

- NOR gate used as Buffer:

- When SW0 = 0, F3 = 0

- When SW0 = 1, F3 = 1

- Does the circuit act as a buffer?

Yes

- NOR gate used as OR gate:

| SW1 (B) | SW0(A) | F3 |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 1 |

| SW1 (B) | SW0(A) | F3 |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 1 | 1 | 1 |

NAND Gate Circuit:

- NOT gate constructed with NAND gate:

- When SW1 = 0, F2 = 1

- When SW1 = 1, F2 = 0

- Does the circuit act as a NOT gate?

Yes it does

- AND gate constructed with NAND gate:

| SW1 (B) | SW0(A) | F3 |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 1 | 1 | 1 |

- OR gate constructed with NAND gates:

| SW1 (B) | SW0(A) | F3 |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 1 |

XOR Gate Circuit:

- XOR gate using NAND gates:

| INPUT |  | OUTPUT |  |  |  |
| --- | --- | --- | --- | --- | --- |
| D | A | F1 | F2 | F3 | F4 |
| 0 | 0 | 1 | 1 | 1 | 0 |
| 0 | 1 | 1 | 1 | 0 | 1 |
| 1 | 0 | 1 | 0 | 1 | 1 |
| 1 | 1 | 0 | 1 | 1 | 0 |

- XOR gate using Basic gates:

| INPUT |  | OUTPUT |  |  |
| --- | --- | --- | --- | --- |
| SW2(B) | SW1(A) | F1 | F2 | F3 |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 1 | 0 | 1 | 1 |
| 1 | 0 | 1 | 0 | 1 |
| 1 | 1 | 0 | 0 | 0 |

- AOI circuit:

When B X B1 is set to 0

| A1 | A | F3 | F4 |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 1 |
| 0 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 0 |

- Does F3 act as an AND gate between A and A1?

Yes.

When A XA1 is set to 0

| B1 | B | F3 | F4 |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 1 |
| 0 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 0 |

- When A×A1 is “0”, does F3 act as an AND gate between B and B1?

Yes.

- Does F3 equal to A×A1+B×B1?

Yes.

Logic Problems:

Post-Lab: