Faculty of Engineering & Technology

Department of Electrical and Computer Engineering

Second Semester, 2014/2015

ENC533– Advanced Digital Design

Course Project

Instructor: Dr. Abdellatif Abu-Issa

Overview

In this project you will build a simple part of a microprocessor. Firstly you will build two main blocks: the ALU and the register file, then you will connect them together and run a simple machine code program on them.

In order to ensure that your design is related to your group, various aspects of the design will be dictated by your student ID numbers. These include the machine code used by your design, the contents of the register file, and the purpose of the program that you will use to demonstrate the correctness of your design. This means that every group should have a unique design.

You have to work in groups of up to 3 students in this project and demonstrate your work to me and submit a report about your design and implementation of this assignment.

Part 1

The ALU

Write a VHDL description of an ALU with two 32-bit inputs, A and B, and a 32-bit output Result.

The result is derived from one or both of the inputs according to the value of a 6-bit opcode. The operations that the ALU can perform are listed below:

• a + b

• a - b

• |a| (i.e. the absolute value of a)

• -a

• max (a, b) (i.e. the maximum of a and b)

• min (a, b) (i.e. the minimum of a and b)

• avg(a,b) (i.e. the average of a and b – the integer part only and remainder is ignored)

• not a

• a or b

• a and b

• a xor b

The opcode that will be used to represent each of these operations is determined by the last digit of your students ID number. The table below shows which opcode you should use in your design for each instruction.

| Digit of ID no. | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| a + b | 8 | 6 | 6 | 4 | 6 | 3 | 4 | 4 | 1 | 5 |
| a – b | 9 | 8 | 9 | 11 | 2 | 15 | 10 | 14 | 6 | 8 |
| ¦a¦ | 2 | 10 | 1 | 8 | 5 | 13 | 3 | 8 | 13 | 13 |
| -a | 10 | 12 | 5 | 6 | 4 | 12 | 12 | 11 | 8 | 7 |
| max(a,b) | 12 | 14 | 7 | 13 | 7 | 7 | 7 | 10 | 7 | 3 |
| min (a,b) | 1 | 11 | 8 | 14 | 10 | 1 | 2 | 1 | 4 | 6 |
| avg(a,b) | 13 | 13 | 11 | 7 | 9 | 9 | 6 | 13 | 11 | 10 |
| not a | 5 | 15 | 14 | 9 | 13 | 10 | 13 | 6 | 15 | 2 |
| a or b | 4 | 2 | 13 | 12 | 8 | 14 | 14 | 9 | 3 | 15 |
| a and b | 11 | 3 | 12 | 10 | 1 | 11 | 11 | 5 | 5 | 4 |
| a xor b | 15 | 9 | 4 | 5 | 3 | 5 | 8 | 7 | 2 | 12 |

So, for example if your group have 3 students and their  IDs

1121432, 112567, and 112879 then the last digit is  = (2+7+9) mod 10 = 8

Then a+b is to be represented by opcode 1, a-b is to be represented by opcode 6, |a| is to be represented by opcode 13 and so on. (These are shown as denary (base 10) values; your design will of course have to use binary or hexadecimal values.)

The register file

Inside a modern processor there is a very small amount of memory that is used to hold the operands that it is presently working on. This is called the register file, and normally has the following appearance.

This is a very small fast RAM, typically holding 32 x 32-bit words, and therefore requiring a 5-bit address to select out one of the 32-bit words. It is unlike normal RAM in that it can process three addresses at the same time, two of which are always read operations, and one of which is always written to.

Output 1 produces the item within the register file that is address by Address 1. Similarly Output 2 produces the item within the register file that is address by Address 2. Input is used to supply a value that is written into the location addressed by Address 3.

The initial values stored in the register file are determined by the second-from-last digit of your student ID, and are shown in the table below:

| ID/ Location | 0 | 1 | 2 | 3 | 4 |  | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 7942 | 11662 | 12642 | 12996 |  | 4198 | 11930 | 4616 | 15034 | 5986 | 16302 |
| 2 | 13224 | 11562 | 10592 | 11490 |  | 5596 | 5348 | 11640 | 8854 | 12250 | 2994 |
| 3 | 15462 | 15330 | 6230 | 7070 |  | 14426 | 7308 | 11254 | 170 | 482 | 1658 |
| 4 | 8026 | 9594 | 8940 | 6026 |  | 7612 | 15684 | 6786 | 7226 | 14246 | 5474 |
| 5 | 3692 | 14746 | 8776 | 3322 |  | 6638 | 12346 | 6784 | 4480 | 5124 | 6784 |
| 6 | 9882 | 3288 | 9436 | 10344 |  | 10040 | 9716 | 12432 | 8928 | 1848 | 10836 |
| 7 | 8248 | 5932 | 3056 | 6734 |  | 3930 | 7820 | 13548 | 7302 | 5260 | 4648 |
| 8 | 3432 | 1978 | 4850 | 15834 |  | 4150 | 5190 | 13462 | 8922 | 16170 | 524 |
| 9 | 178 | 4912 | 3406 | 15314 |  | 6406 | 14702 | 13454 | 1044 | 4766 | 12200 |
| 10 | 2378 | 2380 | 12380 | 6000 |  | 5400 | 5630 | 11780 | 6706 | 4298 | 3286 |
| 11 | 8302 | 1926 | 548 | 12196 |  | 8572 | 2352 | 13170 | 258 | 610 | 14734 |
| 12 | 592 | 12726 | 13054 | 11290 |  | 16324 | 15424 | 2982 | 7354 | 1510 | 10998 |
| 13 | 7430 | 176 | 2800 | 13350 |  | 8840 | 2670 | 8096 | 3294 | 9794 | 4420 |
| 14 | 10572 | 8408 | 12988 | 2086 |  | 8258 | 4172 | 514 | 14740 | 7456 | 8754 |
| 15 | 7676 | 8394 | 956 | 6734 |  | 11228 | 4300 | 3600 | 6532 | 5580 | 3246 |
| 16 | 1238 | 13604 | 2194 | 7430 |  | 8462 | 4744 | 10870 | 10436 | 9300 | 9040 |
| 17 | 16008 | 10222 | 11914 | 14102 |  | 13284 | 1286 | 12528 | 11900 | 12314 | 8714 |
| 18 | 2426 | 7262 | 15864 | 13200 |  | 4676 | 8122 | 9860 | 14694 | 12806 | 12008 |
| 19 | 11930 | 10190 | 11832 | 3264 |  | 3980 | 4558 | 6166 | 8830 | 10478 | 1006 |
| 20 | 6724 | 8734 | 12346 | 2368 |  | 5634 | 8534 | 4520 | 8712 | 11556 | 6724 |
| 21 | 12790 | 12432 | 2192 | 15846 |  | 7632 | 13340 | 14436 | 4532 | 6778 | 12746 |
| 22 | 4842 | 8724 | 1840 | 11710 |  | 9846 | 6918 | 12136 | 9084 | 8430 | 5462 |
| 23 | 7108 | 5412 | 13996 | 14736 |  | 5442 | 11700 | 5134 | 13838 | 5700 | 11810 |
| 24 | 6296 | 11082 | 12054 | 5338 |  | 12488 | 10722 | 11958 | 10018 | 13422 | 7590 |
| 25 | 3322 | 2212 | 4434 | 5544 |  | 6656 | 3346 | 7688 | 1280 | 11224 | 4368 |
| 26 | 10848 | 6188 | 12152 | 1852 |  | 832 | 3300 | 5258 | 5814 | 1990 | 10358 |
| 27 | 14698 | 7056 | 9876 | 3898 |  | 4664 | 2386 | 12420 | 670 | 922 | 15252 |
| 28 | 16378 | 3744 | 8734 | 16252 |  | 6798 | 11212 | 3560 | 8832 | 6020 | 11954 |
| 29 | 15456 | 5766 | 8308 | 1048 |  | 14166 | 3504 | 1248 | 15186 | 15768 | 13704 |
| 30 | 1260 | 3412 | 3422 | 5642 |  | 3246 | 8712 | 8724 | 4512 | 5624 | 1478 |
| 31 | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 |

So, for example, if your students ID is 1121432, 112567, and 112879  then the second-from-last digit of your ID number is = (3+6+7) mod 10 = 6

so item 0 should be 0, item 1 should be 4616, item 2 should be 11640, and so on. (N.B. these values are in denary (i.e. base 10). You will need to convert them to binary or hexadecimal.)

Part 2

In this part you will connect the ALU with the RAM to form a simple microprocessor.

Clocking the register file

The register file that you created in the first part was a combinational circuit. This causes some serious problems if, for example, address 2 and address 3 are the same. The circuit would read output 2 from the location referenced by address 2, at the same time that the input is over-writing that location.

These problems can be solved by synchronising the register file to a clock. You will need to add an extra input named clock, and give the register file the following behaviour:

On the rising edge of the clock:

• Output 1 produces the item within the register file that is address by Address 1.

• Output 2 produces the item within the register file that is address by Address 2.

• Input is used to supply a value that is written into the location addressed by Address 3.

Under all other circumstances:

• the outputs are held constant at the values they assumed during the last rising edge of the clock.

You should check your design thoroughly, and in particular make sure that it behaves sensibly when the address for the input is the same as the address of one of the outputs.

Enabling the register file

The register file that you created is always sensitive to its inputs, even when the inputs have garbage values. This can cause problems because when the simulation initializes (which corresponds to the real hardware being switched on) all the values of the logic signals initializes to some random garbage value (denoted ‘U’ in VHDL, but in real life either a ‘1’ or a ‘0’ chosen at random).

Give the register file an enable input. When the enable input=1 the register file will operate normally. Otherwise the register file will ignore its inputs, and will not update its outputs.

Creating the core of the microprocessor

Now create a testbench that contains an instance of the register file and an instance of the ALU connected like this:

Machine instructions are supplied to this arrangement in the form of 32-bit numbers. The format of these instructions is as follows:

• The first 6 bits identify the opcode

• The next 5 bits identify first source register

• The next 5 bits identify second source register

• The next 5 bits identify destination register

• The final 11 bits are unused

So, for example, if you want to add the contents of register 1 and register 2 and put the result into register 3, then the machine instruction would be as follows:

• The first 6 bits supply the opcode for the add instruction

• The next 5 bits would address register 1, and the next 5 would address register 2

• The next 5 bits address register 3.

• The remaining bits are unused, and should be set to zero.

The enable signal to the register should go high when the opcode contains a valid value, and should be low otherwise.

Test out your design by supplying machine instructions to it, and check that the operation performed is correct. Make sure you understand the timing of instructions, and in particular the relationship between the clock cycle on which the instruction occurs, and the clock cycle on which the appropriate result is written to register.

Running a program

Now create a stream of machine instructions that will act as a small program. The program must act as follows:

If the third-form-last digit of your student ID is 1, 4 or 7:

You should write a machine code to find the maximum number stored in addresses from1 to 30. The values of registers 1-30 should not be altered by your program, but the value of register 0 may be over-written if you wish.

If the third-form-last digit of your student ID is 2, 5 or 8:

You should write a machine code to find the minimum number stored in addresses from1 to 30. The values of registers 1-30 should not be altered by your program, but the value of register 0 may be over-written if you wish.

If the third-form-last digit of your student ID is 0, 3, 6 or 9:

You should write a machine code to find the sum of the numbers stored in addresses from1 to 30. The values of registers 1-30 should not be altered by your program, but the value of register 0 may be over-written if you wish.

You should try to make your program as efficient as possible.

## Synchronising the data

All parts of the design that are synchronized to the clock should use only the rising edge of the clock. If you use both the rising edge and the falling edge, then it’s much easier to do the design in simulation, but the real life hardware would be very expensive and complicated to manufacture. You will therefore lose a substantial number of marks if you use both edges of the clock.

No ops

You may find it useful to create a no-op instruction, i.e. an instruction that instructs the microprocessor to “do nothing” for the next clock cycle, and allocate one of your un-used opcodes to this instruction.

## If you don’t manage to finish

Don’t panic. Obviously you will lose marks for having an incomplete design, but you can still achieve a reasonable mark provided that you do a good write up of the parts that you have finished.

## Writing up the assignment

# Format of write up

The assignment should be written up as a brief report, which should explain (in about 4-6 pages)

• The ideas behind your design and how it works (in particular what happens on what clock cycle and why);

• How you tested it and how you interpreted the results of your tests;

• Which parts work correctly.

Your VHDL code should be included as appendix 1

Print outs of your simulation results should be included as appendix 2 (not more than 5 pages; if you have a large number of results, then you should choose only the most meaningful results, and explain in the text what their significance is)

- Deadline and hand-in procedure

The deadline for submission (hard copy) is 02:00 pm on Tuesday 5th May (more than 5 weeks from now). You will lose 5% of your mark for each late day (Late submission is allowed until Thursday 14th May).

- Mark scheme

Style, structure and presentation of report 						10%

Description of design and testing process 						20%

Technical achievements in design, implementation and evaluation 			50%

Quality of code (good comments, clear layout, good coding style,

meaningful signal and entity names) 						10%

Judgment and creativity                                                                                              10%

#### Electrical and Computer Engineering Department

Project Feedback

Course Project (ENCS 533)

# Instructor: Dr. Abdellatif Abu-Issa

Student Name:…………………………………..         Student ID:………………….

# Marks

Report Presentation (10%)

Language (Spelling and Grammar), style of the report, caption of figures, page

numbering…etc.

Design Process and Outcome (80%)

- Description of the design and test process (20%)

- Technical Achievement in System Design and implementation (50%)

Quality of code  (10%)

Judgment and Creativity (`10%)

Demonstration of good judgment, imagination and creativity

in selecting and applying design methods. Good discussion and

analysing of the system and suggested improvements.

Total Mark (Out of 100)

Deducted Marks:  late days * 5% per day

FINAL ALLOCATED MARK (Out of 100)

Any evidence for any type of cheating:   □yes                 □no