Faculty of Engineering and Technology

Electrical and Computer Engineering Department

Digital Integrated Circuit

ENCS333

Quiz No. 3

- Prepared by:  Ayham Hashesh 		1161301

- Instructor: Dr. Khader Mohammad

- Section No.: 1

- Date: 28-4-2020

- List thee advantage of domino design (Dynamic Logic)?

- Full swing outputs (VOL = GND and VOH = VDD).

- Non-ratioed sizing of the devices does not affect the logic levels.

- Faster switching speed.

- What are the issues of domino design (Dynamic Logic)?

- Overall power dissipation usually higher than static CMOS.

- Pull down network starts to work as soon as the input signals exceeds VTN, so VM, VIH and VIL equal to VTN. (Low noise margin)

- Needs a precharge/evaluate clock.

- List three CMOS Logic advantage?

- Full rail-to-rail swing: High Noise Margin.

- No direct path steady state between power and ground: No static power dissipation.

- Always a path to VDD or Ground in steady state: Low output impedance.

- Define Charge Sharing? Do we have charge sharing problem in the attached figure? if yes how do we solve it

- Charge sharing: Charge stored originally in CL is redistributed (shared) over CL and other capacitors leading to reduce roubstness.

- Yes we have charge sharing problem.

- To solve: Precharge internal nodes using a clock driven transistor (at the cost of increased area and power).

- What is the function of the attached layout? Is this an optimal design? Explain

- Nand Gate.

- No, It’s not the optimal design as there exists many capacitance which can be reduced if we shred the diffusion