;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

;

;	CALC.ASM	MPB	Ver 1.0		28-8-05

;

;	Simple calculator

;	Single digit input, two digit results

;	Integer handling only

;

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

PROCESSOR 16F877

;	Clock = XT 4MHz, standard fuse settings

__CONFIG 0x3731

;	LABEL EQUATES	;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

INCLUDE "C:\BOOK2\APPS\P16F877A.INC"

Char	EQU	30	; Display character code

Num1	EQU	31	; First number input

Num2	EQU	32	; Second number input

Result	EQU	33	; Calculated result

Oper	EQU	34	; Operation code store

Temp	EQU	35	; Temporary register for subtract

Kcount	EQU	36	; Count of keys hit

Kcode	EQU	37	; ASCII code for key

Msd	EQU	38	; Most significant digit of result

Lsd	EQU	39	; Least significant digit of result

Kval	EQU	40	; Key numerical value

RS	EQU	1	; Register select output bit

E	EQU	2	; Display data strobe

; Program begins ;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

ORG	0		; Default start address

NOP			; required for ICD mode

BANKSEL	TRISC		; Select bank 1

MOVLW	B'11110000'	; Keypad direction code

MOVWF	TRISC		;

CLRF	TRISD		; Display port is output

BANKSEL PORTC		; Select bank 0

MOVLW	0FF		;

MOVWF	PORTC		; Set keypad outputs high

CLRF	PORTD		; Clear display outputs

GOTO	start		; Jump to main program

; MAIN LOOP ;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

start	CALL	Init		; Initialise the display

MOVLW	0x80		; position to home cursor

BCF	Select,RS	; Select command mode

CALL	Send		; and send code

CLRW	Char		; ASCII = 0

CLRW	Kval		; Key value = 0

CLRW	DFlag		; Digit flags = 0

scan	CALL	keyin		; Scan keypad

MOVF	Char,1		; test character code

BTFSS	STATUS,Z	; key pressed?

GOTO	keyon		; yes - wait for release

GOTO	scan		; no - scan again

keyon	MOVF	Char,W		; Copy..

MOVWF	Kcode		; ..ASCIIcode

MOVLW	D'50'		; delay for..

CALL	Xms		; ..50ms debounce

wait	CALL	keyin		; scan keypad again

MOVF	Char,1		; test character code

BTFSS	STATUS,Z	; key pressed?

GOTO	wait		; no - rescan

CALL	disout		; yes - show symbol

INCF	Kcount		; inc count..

MOVF	Kcount,W	; ..of keys pressed

ADDWF	PCL		; jump into table

NOP

GOTO	first		; process first key

GOTO	scan		; get operation key

GOTO	second		; process second symbol

GOTO	calc		; calculate result

GOTO	clear		; clear display

first	MOVF	Kval,W		; store..

MOVWF	Num1		; first num

GOTO	scan		; and get op key

second	MOVF	Kval,W		; store..

MOVWF	Num2		; second number

GOTO	scan		; and get equals key

; SUBROUTINES ;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

; Include LCD driver routine

INCLUDE	"C:\BOOK2\APPS\SUBS\LCD.INC"

; Scan keypad .............................................

keyin	MOVLW	00F		; deselect..

MOVWF	PORTC		; ..all rows

BCF	PORTC,0		; select row A

CALL	Onems		; wait output stable

BTFSC	PORTC,4		; button 7?

GOTO	b8		; no

MOVLW	'7'		; yes

MOVWF	Char		; load key code

MOVLW	07		; and

MOVWF	Kval		; key value

RETURN

b8	BTFSC	PORTC,5		; button 8?

GOTO	b9		; no

MOVLW	'8'		; yes

MOVWF	Char

MOVLW	08

MOVWF	Kval

RETURN

b9	BTFSC	PORTC,6		; button 9?

GOTO	bd		; no

MOVLW	'9'		; yes

MOVWF	Char

MOVLW	09

MOVWF	Kval

RETURN

bd	BTFSC	PORTC,7		; button /?

GOTO	rowb		; no

MOVLW	'/'		; yes

MOVWF	Char		; store key code

MOVWF	Oper		; store operator symbol

RETURN

rowb	BSF	PORTC,0		; select row B

BCF	PORTC,1

CALL	Onems

BTFSC	PORTC,4		; button 4?

GOTO	b5		; no

MOVLW	'4'		; yes

MOVWF	Char

MOVLW	04

MOVWF	Kval

RETURN

b5	BTFSC	PORTC,5		; button 5?

GOTO	b6		; no

MOVLW	'5'		; yes

MOVWF	Char

MOVLW	05

MOVWF	Kval

RETURN

b6	BTFSC	PORTC,6		; button 6?

GOTO	bm		; no

MOVLW	'6'		; yes

MOVWF	Char

MOVLW	06

MOVWF	Kval

RETURN

bm	BTFSC	PORTC,7		; button x?

GOTO	rowc		; no

MOVLW	'x'		; yes

MOVWF	Char

MOVWF	Oper

RETURN

rowc	BSF	PORTC,1		; select row C

BCF	PORTC,2

CALL	Onems

BTFSC	PORTC,4		; button 1?

GOTO	b2		; no

MOVLW	'1'		; yes

MOVWF	Char

MOVLW	01

MOVWF	Kval

RETURN

b2	BTFSC	PORTC,5		; button 2?

GOTO	b3		; no

MOVLW	'2'		; yes

MOVWF	Char

MOVLW	02

MOVWF	Kval

RETURN

b3	BTFSC	PORTC,6		; button 3?

GOTO	bs		; no

MOVLW	'3'		; yes

MOVWF	Char

MOVLW	03

MOVWF	Kval

RETURN

bs	BTFSC	PORTC,7		; button -?

GOTO	rowd		; no

MOVLW	'-'		; yes

MOVWF	Char

MOVWF	Oper

RETURN

rowd	BSF	PORTC,2		; select row D

BCF	PORTC,3

CALL	Onems

BTFSC	PORTC,4		; button C?

GOTO	b0		; no

MOVLW	'c'		; yes

MOVWF	Char

MOVWF	Oper

RETURN

b0	BTFSC	PORTC,5		; button 0?

GOTO	be		; no

MOVLW	'0'		; yes

MOVWF	Char

MOVLW	00

MOVWF	Kval

RETURN

be	BTFSC	PORTC,6		; button =?

GOTO	bp		; no

MOVLW	'='		; yes

MOVWF	Char

RETURN

bp	BTFSC	PORTC,7		; button +?

GOTO	done		; no

MOVLW	'+'		; yes

MOVWF	Char

MOVWF	Oper

RETURN

done	BSF	PORTC,3		; clear last row

CLRF	Char		; character code = 0

RETURN

; Write display ...........................................

disout	MOVF	Kcode,W		; Load the code

BSF	Select,RS	; Select data mode

CALL	Send		; and send code

RETURN

; Process operations ......................................

calc	MOVF	Oper,W		; check for add

MOVWF	Temp		; load input op code

MOVLW	'+'		; load plus code

SUBWF	Temp		; compare

BTFSC	STATUS,Z	; and check if same

GOTO	add		; yes, jump to op

MOVF	Oper,W		; check for subtract

MOVWF	Temp

MOVLW	'-'

SUBWF	Temp

BTFSC	STATUS,Z

GOTO	sub

MOVF	Oper,W		; check for multiply

MOVWF	Temp

MOVLW	'x'

SUBWF	Temp

BTFSC	STATUS,Z

GOTO	mul

MOVF	Oper,W		; check for divide

MOVWF	Temp

MOVLW	'/'

SUBWF	Temp

BTFSC	STATUS,Z

GOTO	div

GOTO	scan		; rescan if key invalid

; Calclate results from 2 input numbers ...................

add	MOVF	Num1,W		; get first number

ADDWF	Num2,W		; add second

MOVWF	Result		; and store result

GOTO	outres		; display result

sub	BSF	STATUS,C	; Negative detect flag

MOVF	Num2,W		; get first number

SUBWF	Num1,W		; subtract second

MOVWF	Result		; and store result

BTFSS	STATUS,C	; answer negative?

GOTO	minus		; yes, minus result

GOTO	outres		; display result

minus	MOVLW	'-'		; load minus sign

BSF	Select,RS	; Select data mode

CALL	Send		; and send symbol

COMF	Result		; invert all bits

INCF	Result		; add 1

GOTO	outres		; display result

mul	MOVF	Num1,W		; get first number

CLRF	Result		; total to Z

add1	ADDWF	Result		; add to total

DECFSZ	Num2		; num2 times and

GOTO	add1		; repeat if not done

GOTO	outres		; done, display result

div	CLRF	Result		; total to Z

MOVF	Num2,W		; get divisor

BCF	STATUS,C	; set C flag

sub1	INCF	Result		; count loop start

SUBWF	Num1		; subtract

BTFSS	STATUS,Z	; exact answer?

GOTO	neg		; no

GOTO	outres		; yes, display answer

neg	BTFSC	STATUS,C	; gone negative?

GOTO	sub1		; no - repeat

DECF	Result		; correct the result

MOVF	Num2,W		; get divisor

ADDWF	Num1		; calc remainder

MOVF	Result,W	; load result

ADDLW	030		; convert to ASCII

BSF	Select,RS	; Select data mode

CALL	Send		; and send result

MOVLW	'r'		; indicate remainder

CALL	Send

MOVF	Num1,W

ADDLW	030		; convert to ASCII

CALL	Send

GOTO	scan

; Convert binary to BCD ...................................

outres	MOVF	Result,W	; load result

MOVWF	Lsd		; into low digit store

CLRF	Msd		; high digit = 0

BSF	STATUS,C	; set C flag

MOVLW	D'10'		; load 10

again	SUBWF	Lsd		; sub 10 from result

INCF	Msd		; inc high digit

BTFSC	STATUS,C	; check if negative

GOTO	again		; no, keep going

ADDWF	Lsd		; yes, add 10 back

DECF	Msd		; inc high digit

; display 2 digit BCD result ..............................

MOVF	Msd,W		; load high digit result

BTFSC	STATUS,Z	; check if Z

GOTO	lowd		; yes, dont display Msd

ADDLW	030		; convert to ASCII

BSF	Select,RS	; Select data mode

CALL	Send		; and send Msd

lowd	MOVF	Lsd,W		; load low digit result

ADDLW	030		; convert to ASCII

BSF	Select,RS	; Select data mode

CALL	Send		; and send Msd

GOTO	scan		; scan for clear key

; Restart ................................................

clear	MOVLW	01		; code to clear display

BCF	Select,RS	; Select data mode

CALL	Send		; and send code

CLRF	Kcount		; reset count of keys

GOTO	scan		; and rescan keypad

END	;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;