;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

;

;	KEYPAD.ASM	MPB 	Ver 1.0		28-8-05

;

; 	Reads keypad and shows digit on display

; 	Design file KEYPAD.DSN

;

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

PROCESSOR 16F877

PCL	EQU	002		; Program Counter

PORTC	EQU	007		; 7-Segment display

PORTD	EQU	008		; 3x4 keypad

TRISC	EQU	087		; Data direction

TRISD	EQU	088		; registers

Key	EQU	020		; Count of keys

; Initialise ports.........................................

BANKSEL	TRISC		; Display

CLRW			; all outputs

MOVWF	TRISC		;

MOVLW	B'00000111'	; Keypad

MOVWF	TRISD		; bidirectional

BANKSEL	PORTC		; Display off

CLRF	PORTC		; initially

GOTO	main		; jump to main

; Check a row of keys .....................................

row	INCF	Key		; Count first key

BTFSS	PORTD,0		; Check key

GOTO	found		; and quit if on

INCF	Key		; and repeat

BTFSS	PORTD,1		; for second

GOTO	found		; key

INCF	Key		; and repeat

BTFSS	PORTD,2		; for third

GOTO	found		; key

GOTO	next		; go for next row

; Scan the keypad..........................................

scan	CLRF	Key		; Zero key count

BSF	3,0		; Set Carry Flag

BCF	PORTD,4		; Select first row

newrow	GOTO	row		; check row

next	BSF	PORTD,3		; Set fill bit

RLF	PORTD		; Select next row

BTFSC	3,0		; 0 into carry flag?

GOTO	newrow		; if not, next row

GOTO	scan		; if so, start again

found	RETURN			; quit with key count

; Display code table.......................................

table	MOVF	Key,W		; Get key count

ADDWF	PCL		; and calculate jump

NOP			; into table

RETLW	B'00001100'	; Code for '1'

RETLW	B'10110110'	; Code for '2'

RETLW	B'10011110'	; Code for '3'

RETLW	B'11001100'	; Code for '4'

RETLW	B'11011010'	; Code for '5'

RETLW	B'11111000'	; Code for '6'

RETLW	B'00001110'	; Code for '7'

RETLW	B'11111110'	; Code for '8'

RETLW	B'11001110'	; Code for '9'

RETLW	B'10010010'	; Code for '*'

RETLW	B'01111110'	; Code for '0'

RETLW	B'11101100'	; Code for '#'

; Output display code......................................

show	CALL	table		; Get display code

MOVWF	PORTC		; and show it

RETURN

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

; Read keypad & display....

main	MOVLW	0FF		; Set all outputs

MOVWF	PORTD		; to keypad high

CALL	scan		; Get key number

CALL	show		; and dsiplay it

GOTO	main		; and repeat

END

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;