# Introduction:

In this section, some information about the interrupts (hardware and software) will be introduced, in addition to brief information about the programmable interrupt controller (PIC), and how it is programmed.

## Programmable interrupt controller (8259 PIC):

### What is a PIC?

The Intel 8259A Programmable Interrupt Controller (see Fig.1.1) handles up to eight vectored priority interrupts for the CPU. It is cascadable for up to 64 vectored priority interrupts without additional circuitry. It is packaged in a 28-pin DIP, uses NMOS technology and requires a single a5V supply. Circuitry is static, requiring no clock input. The 8259A is designed to minimize the software and real time overhead in handling multi-level priority interrupts. It has several modes, permitting optimization for a variety of system requirements.[1]

Fig.1.1: Programmable Interrupt Controller (PIC) [3]

Fig.1.2: 8259 PIC Block Diagram

As shown in (Fig.1.2) the PIC contains three main internal registers:

- Interrupt Request Register (IRR) : the IRR is used to indicate all the interrupt levels which are requesting service

- In-Service Register (ISR) : the ISR is used to store all the interrupt levels which are currently being serviced

- Interrupt Mask Register (IMR) : it stores the bits which disable the interrupt lines to be masked. The IMR operates on the output of the IRR. Masking of a higher priority input will not affect the interrupt request lines of lower priority [2].

### Programming PIC:

PIC accepts two types of command words which are generated by the CPU:

- Initialization Command Word (ICW):  There are four Initialization Command Words for the 8259A that are selected with the help of logic level of A0 pin. When the 8259A is first powered up, it must be sent ICW1, ICW2 and ICW4. If the 8259A is programmed in cascade mode by ICW1, then we also must program ICW3 [4].

- Operation Command Word (OCW): These are the command words which command the 8259 to operate in different interrupt modes [5].

### Modes of Operation for 8259 PIC: [1]

- Fully nested mode

- Rotating priority mode

- Special mask mode

- Polled mode

- Automatic End of Interrupt (AEOI) Mode

- Edge and Level Triggered Modes

- Buffered Mode

### Steps for programming a PIC :

At the beginning ICW1 must be initialized, which will decide if ICW4 is needed or not that is used to initialize different modes such as buffered and fully nested different types and Normal End of Interrupt. Furthermore, ICW1 can determine if we use a single PIC or we Cascade Multiple PICs together , PIC ICW3 is needed when Cascade mode is used to determine if the IRQ is an input from a slave or not also it implies the number of slaves connected to the master PIC when programming Master Slave combination of PICs. After initializing the PIC, it is possible now to specify in which mode does the PIC work in using OCWs to mask some IRQ or to change the mode of operation for the PIC or determine to use interrupt technique or polling technique or to get Data from the PIC , all these can be obtained by the OCW’s for the PIC [1].

## Interrupts:

An interrupt is a signal to the processor emitted by hardware or software indicating an event that needs immediate attention. An interrupt alerts the processor to a high-priority condition requiring the interruption of the current code the processor is executing (the current thread). The processor responds by suspending its current activities, saving its state, and executing a small program called an interrupt handler (or interrupt service routine, ISR) to deal with the event. This interruption is temporary, and after the interrupt handler finishes, the processor resumes execution of the previous thread. There are software and hardware interrupts [5].

### Software Interrupts:

This type of interrupts is caused either by an exceptional condition in the processor itself, or a special instruction in the instruction set which causes an interrupt when it is executed. Software interrupt instructions function similarly to subroutine calls and are used for a variety of purposes, such as to request services from low level system software such as device drivers [5].

### Hardware Interrupts:

This type of interrupts is an electronic alerting signal sent to the processor from an external device, either a part of the computer itself such as a disk controller or an external peripheral [5].

### Interrupt service routine:

A program which is executed when receiving an interrupt, interrupt handlers have many functions which vary based on the reason of the interrupt who triggered the interrupt service routine. Interrupt handlers are triggered by software interrupts or hardware interrupts and used for servicing hardware devices and transitions [5].

The starting address of the interrupt service routine for each interrupt is obtained by using “interrupt vector table” 4 bytes for each one 2 bytes for IP and 2 bytes for CS , the interrupt vector table (as shown in Fig.1.3) in locations 0000H to 3FFH which is 1kbyte , each interrupt needs 4 bytes which leaves to use the ability to store for 256 interrupt [6].

For Example INT 2, is located in 2*4 = 8 and so the IP will be located in 00008H and 00009H whereas the CS will be located in 0000AH and 0000BH.

Fig.1.3: Interrupt Vector Table [6]

Interrupt vector address is 20 bit computed from IP and CS. The four bytes for each interrupts are ordered as follows:Figure 1.4 (CS and IP Representation in interrupt Vector Table) [6]

# Procedure and Discussion:

## 2.1 Task 1: User Defined Software Interrupts:

In this part, we have written an assembly program that changes the ISR (interrupt service routine) for INT 21h and replaces it the one we have written. The following code shows that.

CODE SEGMENT
          ASSUME CS:CODE,DS:CODE,ES:CODE,SS:CODE
  V_TAB EQU 21H*4
   SEG_D EQU 0000H
            ORG 1000H
           MOV AX,SEG_D
           MOV DS,AX
           MOV BX,V_TAB
           MOV AX,OFFSET INT_SER
           MOV WORD PTR[BX],AX
           INC BX
           INC BX
           MOV DX,0
           MOV WORD PTR[BX],DX
           MOV AX,1234H
           MOV BX,1234H
           INT 21H
           NOP
           NOP
           INT 3
INT_SER: ADD AX,BX
 IRET
CODE ENDS
     END

In the code above, we have changed the data in the interrupt vector table and we made the entry that represents INT 21H points to the service routine we have written, the address of the entry is 4*21H and the first 2 bytes represent the code segment and the other 2 bytes represent the IP . We changed the values of the code segment and IP in the Interrupt vector table and we replaced them with the code segment and offset that point to our service routine. After that when INT 21H is activated then the code we have written will be executed which means that the value of AX and BX will be added together and saved in AX and at the end of the execution of the program above the value in AX would be 2468H.

## 2.2 Task 2: 8259a Interrupt Control (polling technique):

In this part, we have written an assembly program that is used to control the LEDs on the kit such that one LED is ON at any time and when we press a button the next LED will be lit. In this program we used the polling technique, in which we keep checking a specific value to know if it has changed or not and taking actions on that basis. The following code shows that.

CODE SEGMENT
 ASSUME CS:CODE , DS:CODE,ES:CODE ,SS:CODE
PPIC_C EQU 1FH
PPIC EQU 1DH
PPIB EQU 1BH
PPIA EQU 19H
INTA EQU 10H
INTA2 EQU 12H
ORG 1000H 
 CALL INIT
MOV AL, 80H
OUT PPIC_C,AL
MOV AL,00000000B
OUT PPIC ,AL
MOV AH,11110001B
MOV AL,AH
OUT PPIB ,AL
L2 : MOV AL, 0CH ;ocw3 enable poll
OUT INTA ,AL
IN AL,INTA
TEST AL, 80H
JZ L2
SHL AH,1
TEST AH, 00010000B
JNZ L1
OR AH ,11110000B
JMP L3
L1: MOV AH , 11110001B
L3: MOV AL,AH
OUT PPIB ,AL
MOV AL,20H;END OF INTERRUPT COMMAND
OUT INTA ,AL
JMP L2


INIT PROC NEAR
     MOV AL,13H
     OUT 10H,AL
     MOV AL, 40H
 OUT 12H,AL
MOV AL, 01H
OUT 12H,AL
mov al ,0fbh
out 12H,al
   RET
INIT ENDP
CODE ENDS
END

At first, have programmed the PIC (programmable interrupt controller) using a procedure we have written (INIT), we have send the value of ICW1 (13H) which means that the PPI is in single mode and the ICW4 is required since we are using Intel 8086 processor. After that we have send the ICW2 (40H) which defines the interrupt vector. And we have sent ICW4 that is used to choose the processor which is Intel 8086 in our experiment. Finally, we have sent the interrupt mask (OCW1) which is used to choose which interrupts are set and which are not.

After programming the PIC, we have programmed the PPI in order to control the LEDs. In this task we use polling technique, so we have enabled the poll command using OCW3 and after that we read the poll command and compare it in order to check if there is an interrupt or not. If there is an interrupt we change the LEDs that are lit and we send the end of interrupt command. If there is no interrupt we keep checking till and interrupt is detected and we repeat the previous procedure.

## 2.3 Task 3: 8259a Interrupt Control (interruption technique):

In this task, we want to control the seven segment display such that it counts from 0 to 9 but it can step from a number to another only when we press a button which causes an interrupt. The following code shows that.

CODE SEGMENT
   ASSUME CS:CODE, DS:CODE,ES:CODE, SS:CODE
 PPIC_C EQU 1FH
 PPIC   EQU 1DH
PPIB    EQU 1BH
PPIA    EQU 19H
INTA EQU 10H
INTA2 EQU 12H
INT_V EQU 42H*4
STACK EQU  540H
ORG 1000H
 XOR BX,BX
MOV ES,BX
MOV DS,AX
MOV SS,BX
MOV SP,STACK
MOV AX,OFFSET INT_SER
MOV BX,INT_V
MOV WORD PTR ES:[BX],AX
XOR AX,AX
MOV WORD PTR ES:[BX+2],AX
CALL INIT
MOV AL,80H
OUT PPIC_C,AL
MOV SI,OFFSET DATA
MOV AL,BYTE PTR CS:[SI]
OUT PPIA,AL
STI
L2 : NOP
JMP L2
INT_SER:
MOV AL,BYTE PTR CS:[SI]
CMP AL,00H
JNZ L3
MOV SI,OFFSET DATA
JMP INT_SER
L3 : OUT PPIA,AL
INC SI
MOV AL,20H
OUT INTA, AL
STI
IRET
INIT PROC NEAR
MOV AL,13H
OUT INTA ,AL
MOV AL,40H
OUT INTA2,AL
MOV AL,01H
OUT INTA2,AL
MOV al ,0fbh
OUT  INTA2,al
RET
INIT ENDP
DATA : DB  0C0H,0F9H,0A4H,0B0H,99H,92H,82H,0F8H,80H,90H,00H 
CODE ENDS
END

At first, we change the values of CS and IP in the entry of INT 42H in interrupt vector table to make it points to our service routine which is responsible of loading the data of a number into the 7-segment display. After that we program the PIC in the same way we did in the previous task. We program it to work in single mode and we used ICW4 because we are using Intel 8086 processor and we sent the interrupt mask (OCW1). Also we programmed the PPI which is used to interface the 7-segment display. After that the program is stuck in an infinite loop, when we press an interrupt happens and the execution of the program is stopped in order to execute the interrupt service routine which in our case loads the data of the next number to be displayed after that we send the end of interrupt command and the program goes back to its execution.

## 2.4 Task 4: 8259a Interrupt Control (interruption technique - C Code):

This task is similar to the previous one. In this task, we want to control the 7-segment display such that it counts from A to F but it can step from a number to another only when a button is pressed, we have written a C code in order to do that and here it is.

#include "mde8086.h"
#define INT_V 0x42
int data[6] = {0x88,0x80,0XC6, 0XC0,0x86,0X8E  }; 
int index =0 ;
void wait(long del)
{
while( del-- );
}
void int_ser(void){
INTERRUPT_IN; 
index ++;
if( index > 5)
 index =0   ; 
outportb( PPI1_A, data[index] );
outportb( INTA, 0x20);
asm pop ds;
asm pop es;
asm pop dx;
asm pop cx;
asm pop bx;
asm pop ax;
asm pop di;
asm pop si;
asm iret;
}
void main(void)
{
unsigned long far *intvect_ptr;
intvect_ptr = ((unsigned long far *)0);
asm CLI;
outportb( INTA, 0x13 );
outportb( INTA2, 0x40 );  
outportb( INTA2, 0x01 );  
outportb( INTA2, 0xfb );
outportb( PPI1_CR, 0x80 );
outportb( PPI1_B, 0xf0 );
outportb( PPI1_C, 0x00 );
outportb( PPI1_A, 0xCC ) ;
*(intvect_ptr+INT_V) = ( unsigned long )int_ser;
asm STI;
while(1);
}

At first, we have written a function that represents the interrupt service routine which is responsible of loading the data of the next number o the 7-segment display. After that in the main function we have programmed the PIC so that it works in single mode and we used ICW4 because we are using Intel 8086 and we also have sent the interrupt mask (OCW1), we also programmed the PPI. After that the program is stuck in an infinite loop, when we press an interrupt happens and the execution of the program is stopped in order to execute the interrupt service routine which in our case loads the data of the next number to be displayed after that we send the end of interrupt command and the program goes back to its execution. In the code above we used ASM instruction in order to write assembly instructions in the C program.

# 3.0 Conclusion:

In this experiment, we have learned about the Intel 8086 interrupts, we learned about different types of interrupts, the way they work and how they are handled. In this experiment we concentrated on hardware interrupts and the needed circuits to interface them, so we used the programmable interrupt controller (8259a PIC). We learned how to program it and use in different modes. We used it in the polling technique in which we keep checking a special bit to see if it has changed or not, we also used it in the interruption technique.

# 4.0 References:

[1] http://pdos.csail.mit.edu/6.828/2010/readings/hardware/8259A.pdf

Access date: 20/04/2014

[2] http://shaece.files.wordpress.com/2012/01/8259.ppt

Access date: 20/04/2014

[3] http://upload.wikimedia.org/wikipedia/commons/thumb/1/13/Intel_8259.svg/310px-Intel_8259.svg.png

Access date: 20/04/2014

[4] http://xa.yimg.com/kq/groups/19362334/44974163/name/8259+PIC

Access date: 20/04/2014

[5] http://en.wikipedia.org/wiki/Interrupt

Access date: 20/04/2014

[6] Interrupts MDA-8086 Kit Programmable Interrupt Controller Application, Experiment 5.

Access date: 20/04/2014