Department of Computer System Engineering

COMPUTER DESIGN LAB

ENCS 411

Report 1

Experiment No. 2

Control of Dot Matrix and 7 –Segment

Student Name:

Ibrahim Abu Farha                                    1110024

Othman Alkhamra                                     1110017

Instructor: Dr. Ahmad Afaneh

Teaching Assistant: Eng. Yazan Abu Farha

Section: 3

Date: 24/03/2014

# Abstract:

In this experiment we were introduced to the 82C55 PPI and its modes of operation, the PPI can be programmed using Assembly Language in order to control it. Many devices can be interfaced and controlled using the PPI such as Seven Segment display and an 8x8 dot Matrix.

# Introduction

In this section the 82C55 PPI will be introduced, and other components that will be interfaced with the PPI such as 8X8 dot Matrix and Seven Segment display.

## Programmable Peripheral Interface (82C55 PPI):

The 82C55 PPI (see Fig.1.1) is an I/O port chip used for interfacing I/O devices with microprocessor, the 82C55 is a popular interfacing component, that can interface any TTL-compatible I/O device to a microprocessor [1].

Fig.1.1 (82C55 PPI –IC chip) [2]

The 82C55 PPI chip has 40 pins, Consist of three 8-bits width Port (A, B and C), 8 bit Data Bus and control signals WR, RD ,RESET ,CS ,A0 ,A1 , VCC and GND as shown in Fig.1.1 . The 82C55 PPI Requires insertion of wait states if used with a microprocessor using higher than an 8 MHz clock [2].

There are three different modes of operation where PPI ports can be programmed to work in:

- Mode 0: (Buffered input latched output).

- Mode 1: (Strobe input strobe output).

- Mode 2: (Bidirectional input output).

The mode of the port is determined using the command Register in the PPI that is used to control the direction and the mode of the ports as shown in Fig.1.2 [1].

Fig.1.2: (Command Register & modes of Operation for PPI) [1]

From Fig.1.2 we can see that group A (Port A and the upper part of port C) works in all modes (mode 0, 1 and 2), whereas group B (Port B and the lower part of port C) works only in modes 0 and 1.

## Dot Matrix 8X8

The Dot Matrix (see Fig.1.3) consists of 8X8 Red LEDS and 8X8 Green LEDS. These LEDS can be controlled using PORT A, Port B and Port C to display any character we want on it.

Fig.1.3: (Dot Matrix ) [3]

Ports A and B are used to drive the cathode of the LEDs, while Port-C would drive the anodes terminals. Port-A drives the green LEDs while port-B drives the red LEDs. If the two LEDs with the same coordinates are enabled then the outcome will be an orange dot. Ports A and B select the rows while Port-C select the columns as shown in Fig.1.3 [3].

## Seven Segment Display

A seven-segment display  (SSD), or seven-segment indicator (see Fig.1.4), is a form of electronic display device for displaying decimal numerals that is an alternative to the more complex dot matrix displays. Seven-segment displays are widely used in digital clocks, electronic meters, and other electronic devices for displaying numerical information [4].

Fig.1.4: (Seven Segment Display) [5]

The display consists of seven active low segments (A, B, C, D, E, F, G) and an active low decimal point (P). The segments and P are driven by the 82C55 Port-A signals as shown in Table 1.1. For example to display 2 we need 10100100B => A4H sent to port A.

Table 1.1: (Port A Signals) [3]

# 2.0 Procedure and Discussion

## 2.1 Task 1

In this part, we wrote an assembly program that is used to control the DOT MATRIX. The    following code will make a column in the matrix to be ON and the column will be moving to the right.

COLUMN:
MOV AL, 01H	; enable column zero (C0)
SHOW: 
OUT PPIC, AL; sending the value to Port-C
CALL TIMER		; making some delay
CLC				; clear the carry flag if the shift
SHL AL, 1		; moving the '1' to the next column
JNC SHOW		; still there are column to light
JMP COLUMN		; start again from column 0

In the code above we control the movement of the column through manipulating the value on Port C in the PPI which is connected to the anode of the matrix.

## 2.2 Task 2

This task is similar to the previous one. The difference is that here we want to make a row to be ON and make it scroll from top down to the bottom of the matrix. This can be accomplished by setting all the anodes to be high, the value on Port C is 0xFF and we control the movement of the row through port A or B. the following code has been used to do that.

MOV AL, 0FFH	
OUT PPIC, AL
ROW:
MOV AL, 7FH	
SHOW: 
OUT PPIB, AL 
CALL TIMER		
CLC				
shr AL, 1		
JC SHOW			
JMP ROW

## 2.3 Task 3

In this part, we wrote a code in order to display the character “N” on the DOT MATRIX. In order to do that, we display it row by row with a small delay between each of them. The following code shows that.

MOV AL, 0FFH	
OUT	PPIA, AL
DO:
    MOV AL, 42H    
    OUT PPIC, AL
    MOV AL,0BFH    
    OUT PPIB, AL
   CALL TIMER
    MOV AL, 46H     
    OUT PPIC, AL 
    MOV AL, 0DFH 
    OUT PPIB, AL
    CALL TIMER
    MOV AL, 4AH     
    OUT PPIC, AL 
    MOV AL, 0EFH 
   OUT PPIB, AL
   CALL TIMER
    MOV AL, 52H      
    OUT PPIC, AL 
    MOV AL, 0F7H  
    OUT PPIB, AL
    CALL TIMER            
    MOV AL, 62H     
    OUT PPIC, AL 
    MOV AL, 0FBH   
    OUT PPIB, AL
    CALL TIMER
    MOV AL, 42H     
    OUT PPIC, AL 
    MOV AL, 0FDH    
    OUT PPIB, AL

CALL TIMER
 JMP DO

## 2.3 Task 4

In this part we wrote an assembly program that counts from 0 to 9 and displays the number on the 7-segment display. The following code shows how we have done that.

CODE  SEGMENT
ASSUME  CS:CODE, DS:CODE, ES:CODE, SS:CODE
PPICW   EQU     1FH
PPIA    EQU     19H
ORG  1000H
MOV  AL, 80H    ; sending control word
OUT  PPICW, AL  
PREPARE:
    MOV BX, 0     ; preparing for traversal
PRINT:
    CALL TIMER
    MOV AL, DAT[BX]
    OUT PPIA, AL
     INC BX
    CMP BX, 10  ; displaying the 9 character
     JL PRINT
     JMP PREPARE
INT 3
TIMER:
    MOV CX, 0FFFFH	
TIME:
	NOP 
	NOP
	NOP
	NOP
	LOOP TIME
	RET
DAT DB 0C0H, 0F9H, 0A8H, 0B0H, 99H, 92H, 82H, 0F8H, 80H, 90H

CODE ENDS

END

# 3.0 Conclusion

In this experiment we have learned how to use the 82C55 PPI, how to program it and how it is used into interface many devices like the 7-segment display and 8*8 DOT MATRIX. We also have learned how to use the 7-segment display in order to show numbers or using it as counter display. Moreover, we learn how to use the DOT MATRIX to display variety of shapes and characters.  In this experiment we faced some problems; one of them was finding the proper period of delay between each row and another when we displayed a character on the DOT MATRIX.

# 4.0 References:

# [1] teacher.en.rmutt.ac.th/ktw/13-104-252/7%208255.ppt

Access date: 23-03-2014

[2] http://www.ece.unm.edu/~jimp/310/slides/8086_IO2.html

Access date: 23-03-2014

[3] Manual Sheet, Experiment 2 Control of Dot Matrix and 7-Segment MDA-8086 Kit – PPI

Application, Spring Semester 2013/2014.

[4] http://en.wikipedia.org/wiki/Seven-segment_display

Access date: 23-03-2014

[5] http://www.hobbyprojects.com/components/images/7segment.gif

Access date: 23-03-2014