Faculty of Information Technology

Computer Systems Engineering Department

COMPUTER DESIGN LAB #ENCS411

EXP #8

Programmable Interval Timer

MDA-8086 Kit – PPI Application

Name : Mohammad Modallal

ID : 1120174

Section# : 3

Dr. Khader Mohammad

Date:27/4/2015

- Contents

1	Abstract	3

2	Introduction to PIT	4

2.1 Defnition of PIT	4

2.2 Features  	4

2.3 Examples of usage	4

2.4 8253/4 Functional Description	5

2.5 Programming the 8254	6

3    Procedure	7

3.1	Configuring PIT on PC	7

3.2	assembly program	8

3.3	Configuring PIT on MDA-8086 Kit	9

3.4	increase the delay by a factor of 5 times	11

4	Conclusion	14

5	References	15

1.Abstract

The aim of this experiment is to understanding , configuring and testing the Programmable Interval Timer (PIT) devices[ 8253/4]  on the MDA_8086 kit and Personal Computers.

2.Introduction

2.1 Definition

The Intel 8253 /4 are Programmable Interval Timers (PITs), which perform timing and counting functions. They were primarily designed for the Intel 8080/8085-processors, but later used in x86-systems. They (or an equivalent circuit embedded in a larger chip) are found in all IBM PC compatibles[1].

2.2 Features

The programmable interval timer consists of three independent 16-bit programmable counters (timers). Each counter is capable of counting in binary or BCD[2].

2.3 Examples of usage[2]

The programmable interval timer is useful wherever the microprocessor must control real time events.for examples , motor speed and direction control, and for clock and an events counter.

The timer also appears in the personal computer decoded at ports 40H–43H to do the following:

1. Generate a basic timer interrupt that occurs at approximately 18.2 Hz.

2. Cause the DRAM memory system to be refreshed.

3. Provide a timing source to the internal

2.4 8253/4 Functional Description

Figure 2–1 shows the pin-out of the 8254, and a diagram of one counter. Each counter contains a CLK input, a gate input, and OUT  pins. The clock input is the timing source for each of the internal counters. This input is often connected to the PCLK signal from the microprocessor system bus controller, the gate pin controls the timer in some modes, and the OUT pin is where we obtain the output of the timer[2].

There are many pins  that connect to the microprocessor :data bus pins (D7–D0) and address inputs A1, A0. The address inputs select one of four internal registers within the 8254(Table 2–1) [3] .

The personal computer contains an 8253 timer or its equivalent, decoded at I/O ports 40H–43H. Timer zero is programmed to generate an 18.2 Hz signal that interrupts the microprocessor at interrupt vector 8 for a clock tick. The tick is often used to time programs and events in DOS. Timer 1 is programmed for 15 μs, which is used on the personal computer to request a DMA action used to refresh the dynamic RAM. Timer 2 is programmed to generate a tone on the personal computer speaker[2].

2.5 Programming the 8254

Each counter is individually programmed by writing a control word, followed by the initial count (Figure 2–2). The control word allows the programmer to select the counter, mode of operation, and type of operation (read/write). The control word also selects either a binary or BCD count. Each counter may be programmed with a count of 1 to FFFFH.or to 10000 in BCD[2].

The minimum count of 1 applies to all modes of operation except modes 2 and 3, which have a minimum count of 2.

3.Procedure

3.1 Configuring PIT on PC

Setp1: I  Using  Dos Command prompt and  I  start debuging.

Setp2: I type the following instructions:

O 43 B6           // Program the command register mode3 (show fig 3.1)

O 42 11           // send the count 11h to least_segnificant byte of counter 2

O 42 11           // send the count 11h to most_segnificant byte of counter 2

O 61 33           // send 33 to  port B (the port  that controls the speaker) to enable G2 speaker

O 61 32           // send 32 to  port B (the port  that controls the speaker) to disable G2 speaker

O 42 55

O 42 3F

O 43 B0           // Program the command register mode 0 (show fig 3.2)

O 42 11            // send the count 11h to least_segnificant byte of counter 2

O 42 11            // send the count 11h to most_segnificant byte of counter 2

O 42 FF

O 42 FF

From the  previous code , the output Frequency = input  frequency  count  = 1.19MH4369 ≈272Hz.  (note 1111H =4369)

Figure 3.1

Figure3.2

3.2 assembly program that enables the speaker with the tone frequency

Step1: I write the following code and save it to an Assembly file(.asm).

The following code enable the  speaker with a tone  frequency of 5 KH [ the count = 238 ].

To make the output frequency to equal  5KH , 7 KH, 12KH :

count(5KHz) = input  frequency output Frequency   = 1.19MH5K =238.

count(7KHz) = input  frequency output Frequency   = 1.19MH7K =170.

count(12KHz) = input  frequency output Frequency   = 1.19MH12K =99.

3.3   Configuring PIT on MDA-8086 Kit

Srep1: I connected the  P2 using a jumper cap as shown in (Fig3.3).

Figure 3.3

Setp2: I write the following code and save it to an Assembly file(.asm).I Compile and build this ASM file and execute it on MDA-8086 kit.

The following code contain  ISR to control four LEDs to light, and we used the  PIC by connecting the output of counter 1 to IR0 using P2.

The above code  use PIT(counter 1) , PIC(IR0) ,P2 and PPI (port B) to light the LEDs.I send the value  ffffH to the counter 1 .after the counter reach FFFFH+1 the interrupt is occur and the next LEDS is light and so on…

When I press the GATE1 the LEDs are stoping continue lighting since the GATE1 connected to the ground.The counter is stoped  work when G=0.

3.4 increase the delay by a factor of 5 times.

Srep1: I connected the  P2 using a jumper cap as shown in (Fig3.4).

Figure 3.4

Setp2: I write the following code and save it to an Assembly file(.asm).I Compile and build this ASM file and execute it on MDA-8086 kit.

The following code contain  ISR to control four LEDs to light(figure below), and i used the  PIC by connecting the output of counter 2 to IR1 using P2 and i connecting the output of counter one to the clk of counter2 .

Each time the counter1 count  up to 5 the counter 2  incremant by 1

so the total count=FFFFH*5.

4.Conclusion

In this experiment i  learned the Programmable Interval Timer [PIT]and how it work in  MDA-8086 Kit  . I study the  implementation of PIT which consist of three independent counter(timer).I learned how I can use the PIT counter`s to control real-time event .I use the PIT to light the LEDs and I change the speed of lighting by decreasing frequency.

5.Reference

[1] http://en.wikipedia.org/wiki/Intel_8253

[2] The Intel Microprocessors by Barry.B.Bary (Eighth Edition).

[3] Experement#8  lab manual.