# Faculty of Engineering & Technology

# Electrical & Computer Engineering Department

ENEE236

# Assignment 2

Prepared by : Tareq Shannak

Id Number : 1181404

Instructor : Dr. Nasser Ismael

Section : 1

Date : 21/8/2020

# Objectives

- Design an CE Amplifier using npn BJT transistor

- Testing the voltage gain with different inputs

# Calculations

# Result

I supposed C=10uF.

|  | Ib | Ic | Ie |
| --- | --- | --- | --- |
| Theoretical | 7.43uA | 1.3mA | 1.31mA |
| Practical | 7.805uA | 1.324mA | 1.331mA |

Vce = 12.25-1.931=10.319 (near 9.5v)

### Testing the voltage gain with amplitude=15mV, and different frequencies: 1 KHz, 5KHz, 10KHz Using PARAM.

To check the voltage gain, here are the max points.

|  | F=1KHZ | F=5KHZ | F=10KHZ |
| --- | --- | --- | --- |
| Vin | 14.999m | 14.999m | 14.822m |
| Vout | -374.224m | -360.317m | -356.269m |
| Av | -24.94993 | -24.02273485 | -24.0365 |

The results are too close of the theoretical value (-24).

### Testing the voltage gain with amplitude=1V, Using ac sweep (1Hz-1MHz).

In any frequency of the three frequencies, we will have the same output. The second plot is with DB Scale.

From first plot, |AV|=23.805 (approximately 24)

From the second one, 20*log(|Av|)=27.533 => log(|Av|)=1.37665 => |Av|=23.804 (=24).

NOTE: I had tried too much to make this condition (Zin>35K), but this will force me to change the values that I chose to accredited values and it will broke the value of AV.