# Birzeit University

Faculty of Engineering

Department of Electrical Engineering

Analog Communication system ENEE433

Example Set (1)

Example 1

Suppose that the rectangular RF pulse is given by

x(t)=Acos(2πfct),.....0≤t≤T

Is applied to a linear filter with impulse response given by

h(t)=x(T−t)

- Find the complex envelope x(t)and h(t).

- Find the complex response y(t).

- Find the frequency domain Y(f).and estimate its band width.

- Find y(t)and plot it in time domain.

Example 2

The message signalm(t)=−0.2+0.6sinw1tvolts where f1=3.57Mhz is to modulate a carrier using Amplitude modulation (AM) whereka=1.25volts−1 . If the carrier amplitude and frequencyAc=100volts fc=100MHz .

- Find the modulated signals(t)and plot it in time domain .

- Find the spectrum S(f)and plot it in frequency domain .

- Find the bandwidth and the average power of the modulated signal.

- What is the power efficiency of the transmitted signal.

Example 3

A modulated signal is defined by

s(t)=K1+K2m(t)cos(2πfct)−K2g(t)sin(2πfct)

Is applied to a coherent demodulator whose local oscillator is 2cos(2πfct+θ), where θ is a constant phase error between transmitter and receiver.

- Draw a block diagram of the demodulator.

- Determine the output of the demodulator as a function of the phase error θ.

- Choose the constants K1,K2 and g(t) to find the output of the demodulator for DSB-SC modulated signal.

- Suggest a receiver that overcomes the phase error and draw its block diagram.

Example 4

For the frequency-modulated signal with s(t)=10cos2π108t+10cos2π103t

- Find the message signal m(t).

- Find the power of the modulated signal and the maximum frequency deviation Δf.

- Find the modulation index  β.Is this a NBFM or WBFM, explain.