EE302 Problem Set 8

Complete the following problems from the textbook Principles of Electronic Communication Systems, 3rd ed.

Chapter 3

Questions: 22, 23, 27

Problems: 13

Additional Problems

- In this problem we will consider modulating a high frequency carrier with the band-limited square wave depicted below.  This modulating signal consists of just the 1st, 3rd, and 5th harmonics of a square wave.

  - What is the period (T) of this signal ? (note: two cycles are depicted)			______

  - Write the Fourier series expansion of this modulating signal vm(t).

  - Based on the previous expansion, sketch the frequency-domain plot of this waveform labeling the harmonic frequencies and their amplitudes.

Now consider amplitude modulating this signal with a fc = 50-kHz carrier as depicted below.  The amplitude of the carrier (Vc) is 10-V.

  - Use your knowledge of amplitude modulation, sketch the resulting AM signal in the frequency domain labeling all of the frequency components and amplitudes.