Faculty of Engineering and Technology

Electrical and Computer Engineering Department

Communication Lab #ENEE4103

EXP 6&7.Report

Pulse Code Modulation “PCM”

Prepared by:

Ehab Amriah				1120871

Partners:

Ahmad Zeid‎‏                                 1131767

Ahmad Dar Khalil			1120443

Assistant : Eng. Rana Alqaisi

Date : 06/8/2016

Sec: 01

- Abstract

In this report we discussed the Pulse code modulation.

In part1 we explained about linear quantization, nonlinear quantization and encoding.

And in part2, we explained about PCM transition, Quantization Noise quantization noise variation, difference pulse code modulation (DPSM)

Pulse-code modulation (PCM) is a method used to digitally represent sampled analog signals, a message signal is represented by a sequence of coded pulses, which is accomplished by representing the signal in discrete form in both time and amplitude. This type of modulation is most basic form of digital pulse modulation, addition to the sampling and quantization processes are wasteful, therefore, this way is common in digital communication. The basic operations performed in the transmitter of a PCM system are sampling, quantization, and encoding. The low-pass filter prior to sampling is included merely to prevent aliasing of the message signal. The quantizing and encoding operations are usually performed in the same circuit, which is called an analog-to-digital converter. The basic operations in the receiver are regeneration of impaired signals, decoding, and reconstruction of the train of quantized samples.

Table of Contents

1. Theory …………………………………………………………………………… 1

2. Procedure and Discussion  ……………………………………………….7

3. Conclusion ……………………………………………………………………….26

4.References ………………………………………………………………………..27

II.Theory

Part#1:

- Linear and nonlinear Quantization:

Quantization is the process of assigning a discrete value from a range of possible values to each sample obtained. The number of possible values will depend on the number of bits used to represent each sample. Quantization can be achieved by either rounding the signal up or down to the nears available value, or truncating the signal to the nearest value which is lower than the actual sample. The process results in a stepped waveform resembling the source signal. The difference between the sample and the value assigned to it is known as the quantization noise.

Figure2.1: PCM system

Figure2.2: Uniform& Non-uniform Quantization

Figure2.3: Linear Quantization

Figure2.4: Non-linear Quantization

- Resolution of Quantizer :

The amplitude of the signal. This number amplitude steps is indicated in terms bits. 2 bits correspond to 4 amplitude steps (2^2), 4 bits to 16 amplitude steps (2^4).
The recorded amplitude values are discrete and approximated , and because of the "jump" from one discrete amplitude step to another, a certain degree of noise (quantization noise) arises in the digital signal.  The more bits the amplitude resolution, the smaller the quantization noise: each additional bit improves the quality, as the signal to noise ratio gets lower, the more the amplitude steps are. For example, with an eight bit sample we get 256 amplitude steps, with signal to noise ratio 256:1. With a 16 bit sample we get 56636 amplitude steps, with a much lower signal to noise ratio.

For any number of steps, an amplitude corresponding to half the number of steps can be computed, as amplitude is measured from the zero reference value (which divides the pattern of oscillation of the particles in two). So, for 8 bits (256 amplitude steps), 128 amplitude values are recorded.

Figure2.5: the difference in the resolution if we use 2 bit or 4 bit

- Encoding:

Encoding is the process of representing the sampled values as a binary number in the range 0 to n. The value of n is chosen as a power of 2, depending on the accuracy required. Increasing n reduces the step size between adjacent quantization levels and hence reduces the quantization noise. The down side of this is that the amount of digital data required to represent the analogue signal increases.

Figure2.6

Part#2 :

Pulse code modulation technique has many operations to convert the signal from analog form to the digital form which are sampling, quantizing and encoding as the following figure.

Figure 2.7 [1]

At the receiver, there is many operations on the signal to recover it to the original message as the following figure .

Figure 2.8 [1]

 The operations on the transmitter:

- Sampling : every message signal is converted from analog to the samples by using deltas and train of pulses

- Quantization: there is two types of the quantization, uniform quantization & non-uniform quantization, these operations convert the samples to the quantized samples with different amplitudes.

- Encoding process: it is an important step to convert from analog to digital, which it has parameters such as the number of bits “R” & the number of levels “L”, to determine the bits correctly and the message at digital form.

- DPCM: if we know a sufficient part of a redundant signal, we may infer the rest, or at least make the most probable estimate. In particular, if we know the past behavior of a signal up to a certain point in time, it is possible to make some inference about its

future values , such a process is commonly called prediction .

- Suppose then a message signal m(t) is sampled at the rate Fs=1/Ts to produce a sequence of correlated samples seconds apart , this sequence is denoted by The fact that it is possible to predict future values of the signal provides motivation for the differential quantization.

- the input signal to the quantizer is defined by :

- Which is the difference between original message & Predicted message signal.

 DPCM System at the transmitter & receiver:

1) At the transmitter:

Figure 2.9

2) At the receiver:

Figure 2.10

III. Procedure & Discussion

3.1 Linear Quantization :

We connected the circuit as shown in the figure 3.1

Figure (3.1)

--Then we set PCM modulator and PCM demodulator to PCM linear quantization

--We active all bits (8 bit quantization ) ,by making all leds light red.

--We load esample Quant.labx from CassyLAb 2.

-- We set the potentiometer to -9.5V and we turned it slowly to the right (increasing the the DC voltage).

--UA1 describe the input voltage which increasing from -10 V  to 10 V  and the output voltage (after quantization ) at UB1.

Figure(3.2)

--The figure above show if we put PCM modulator and PCM  demodulator  to linear Quantization We get a linear relation between input and output voltage (UA1 and UB1).

3.2  Nonlinear Quantization :

--we put PCM modulator and PCM demodulator to Non-linear Quantization.

--The figure above shows if we put PCM modulator and PCM demodulator to linear   Quantization.

--We get a linear relation between input and output voltage (UA1 and UB1).

--The figure above shows if we put PCM modulator and PCM demodulator to linear Quantization.

--We get a linear relation between input and output voltage (UA1 and UB1).

Figure3.3

--The figure above shows if the figure above show what happened after we put PCM modulator and PCM demodulator to Non-linear Quantization

We get the figure describe relation between input and output voltage (UA1 and UB1)

3.3 Compressor /expander characteristic :

after we put PCM in PCM modulator  to  non linear and PCM demodulator  to linear Quantization.

the result shown in the figure below :

Figure3.4

3.4 Resolution of Quantizer :

Figure3.5

Linear  linear 5bit

3.5 Encoding:

Figure3.6

The kits  connected as shown  above:

--We set PCM modulator and PCM demodulator to : PCM .linear quantization

--We Turned  on all leds (8 bit)

--After we load code.labx and set the dc potentiometer to maximum lef(-10v)

When we move to the positive value the only bit that lights on it’s for polarity and it’s the last bit

Figure3.7

3.6 PCM Transmission

* We connected the Circuit as the following figure:

Figure 3.8

- Then, we connected UA1 to the PAM Modulator CH1 & UB1 to the PAM Demodulator CH1, after that, we select linear quantization.

- After that , we adjusted All bits which are activated , then we adjusted PAM : duty cycle & Sampling frequency , then we turn on two generators one of them sine wave with fm1=300Hz , A1=10 Vp-p & another one with triangle wave with fm2=200 Hz , A2=5 Vp-p .

By using PCMTrans.labx , our result was as the following figure :

Figure 3.9

- Then , we connected UA1 to the PAM Modulator CH2 & UB1 to the PAM Demodulator CH2 .

Figure 3.10

- We noticed that CH1 Passing the signal of the first function generator , CH2 Passing the other signal from the second function generator.

3.7 Quantization Noise

* We connected the Circuit as the following figure:

Figure 3.11

- Then , we connected UA1 to the PAM Modulator CH2 & UB1 to the PCM Demodulator CH1 , and the function generator with triangle wave with 30 Hz & 12 Vp-p , then we select linear quantization & Activating all Bits .

- By using QNoise.labx , our result was as the following figure :

Fig3.12

- At 5Bits .

Fig3.13

At 5bits and 300Hz

Fig3.14

- We noticed that , increasing of the frequency , the noise will be increase .

- Also . At lower amplitudes the quantization error becomes dependent on the input signal, resulting in distortion, In order to make the quantization error independent of the input signal, noise with an amplitude of 2 least significant bits is added to the signal. This slightly reduces signal to noise ratio .

3.8 Quantization Noise variations :

We changed the function generator to the Sine wave .

At 30 Hz :

Figure 3.14

At 5 Bits :

Figure 3.15

At 5 Bits & 300 Hz :

Figure 3.16

- We selected nonlinear quantization at the same settings :

Fig3.17

At 5 Bits :

Figure 3.18

At 5 Bits & 300 Hz :

Figure 3.19

- In nonlinear quantization the error form differ from linear quantization ; But the variations are the same with the increasing of frequency & Bits .

3.9 Difference Pulse Code Modulation “DPCM” :

- We connected the Circuit as the following figure:

Figure 3.20

- Then we connect both channels to PAM Modulator with function generator with triangle : 30 Hz & 12 Vp-p .

- Then , we selected linear quantization & Activating All Bits .

- Then we done all settings to convert to DPCM mode .

- Then we done all settings in PAM & PCM Modulators to achieve the desired output .

- By using DPCM.labx the desired output as the following figure :

At the 1st Predictor :

Fig3.21

At the 2nd Predictor :

Figure 3.22

- Then , we turned off 4-bits as the following :

- The output Before turning off 4-bits is :

Figure 3.23

- The output After turning off 4-bits is :

Figure 3.24

- We noticed that , the predictor can predicate the signal approximately at 1st predictor & 2nd predictor, until turning 4 bits off the view “axis” was different , but the predicted signal is approximately the same.

IV. Conclusion

I concluded from this Experiment that the term “modulation” in pulse-code modulation is a misnomer. In reality, pulse-code modulation is a source-encoding strategy, by means of which an analog signal emitted by a source is converted into digital form. In addition in DPCM, like DM, is subject to slope-overload distortion whenever the input signal changes too rapidly for the prediction filter to track it. Also PCM & DPCM suffers from quantization noise .

V.References

- [1]”Introduction to Analog & Digital Communications”3.8.2016 at 8:30PM

# [1]”http://www.hum.uu.nl/uilots/lab/courseware/phonetics/basics_of_acoustics_1/quantization.html   3.8.2016 at 8:37PM

- [1]”http://www.hum.uu.nl/uilots/lab/courseware/phonetics/basics_of_acoustics_1/quantization.html  3.8.2016 at 9:00PM

- [1]”http://nptel.ac.in/courses/IIT-MADRAS/Principles_of_Communication1/Pdfs/1_9.pdf   3.8.2016 at 9:45PM