Department of Electrical and Computer Engineering

Communication lab – ENEE411

Report No.2

FM Modulation and Demodulation

Instructor: Dr. Wasfi Kafri

T.A: Eng. Nour Shanak

Prepared by:

Maysam Mahmoud		1111577

Shoroq Zaid				1111529

Wafa Makho 			1120256

Shahd Anati 			1120523

## Abstract

The main objective of this experiment is to understand the concept of frequency modulation (FM) and demodulation. Also, this experiment aims to create a modulated FM wave using MC4046 and LM566 modulators and to notice the influence of changing the frequency of the signal at the modulator voltage output.

Furthermore, this experiment aims to understand the calculation of the frequency deviation, the bandwidth and the modulation coefficient. To clarify the objectives we divided this report into different parts starting with the introduction part in order to illustrate the main concepts. Then, the procedure part shows the steps that we followed with some explanations for results that we got. Finally, the conclusion part summed up all the main ideas that we discussed.

Table of Contents

1.	Introduction	1

1.1	Frequency Modulation (FM)	1

1.2	Frequency Demodulation 	2

1.2	Advantages and Drawbacks of Frequency Demodulation 	3

2.	Procedure and Discussion	4

2.1	Modulation and Demodulation using MC4046 and PLLMC404 6Modules	4

2.2	Modulation using LM566 Modules	7

2.3	Modulation and Demodulation using PLL LM565 Module	9

2.4	Voltage and Frequency conversion using LM565 Module	9

3.	Questions	12

4.	Conclusion	14

References	15

## Introduction

Modulation is the process of varying one or more properties of a periodic waveform, called the carrier signal, with a modulating signal that typically contains information to be transmitted [1]. In this Experiment, we will discuss the frequency modulation and demodulation

### Frequency Modulation (FM)

The concept of the Frequency Modulation (FM) was invented by Edwin H. Armstrong in 1933. Frequency modulation is a form of analog angle modulation in which the message signal m(t), varies the frequency of a carrier wave [1]. So that the instantaneous frequency fi(t)  is varied linearly with the message signal m(t) in a relation that is shown below where fc is the frequency of the unmodulated carrier and kf is the frequency sensitivity of the modulator (Hertz per volt) [2].

fi (t) = fc + kf m (t)

The simplest way to get this output signal is by using voltage controlled oscillator (VCO) that takes the massage signal as input and produce an output signal xFM(t) that has the frequency fi (t) [1]. Integrating previous equation with respect to time and multiplying the result by 2π, we get

θ (t)= 2π f t +2π k ∫ m (τ) dτ

Hence, the FM output signal, xFM(t), is given by the following equation:

Figure 1.1 below shows the massage signal, carrier signal and the modulated signal using FM modulation.

Figure 1.1: Frequency Modulated Signal [3].

### Frequency Demodulation

FM demodulation is the act of extracting the original information-bearing signal from a modulated carrier wave [4]. This process involves changing the frequency variations in a signal into amplitude variations at baseband. There are several techniques and circuits that can be used each with its own advantages and disadvantages [5].

Figure 1.2: FM modulation and demodulation [5].

There are a number of circuits that can be used to demodulate FM. Each type has its own advantages and disadvantages. Some of these demodulators are: Slope FM detector, Ratio detector, Foster-Seeley FM detector and Phase locked loop (PLL) FM demodulator [5].

The modulated signal of FM modulation is:

Where:  = modulating signal,  = carrier frequency,= carrier amplitude,
= sensitivity factor
If we take ,then the equation become :-

Where :    = frequency deviation

When we differentiate the last equation we observe the output after the envelope detector as the following:

### Advantages and Drawbacks of Frequency Modulation

The FM has many advantages over other modulation methods, some of these advantages are [5]:

- Does not require linear amplifiers in the transmitter.

- Constant power.

- Enables greater efficiency than many other modes.

- Resilient to noise.

On the other hand, there are some drawbacks of the FM, such as:

- Requires more complicated demodulator.

- Some other modes have higher data spectral efficiency.

- Sidebands extend to infinity either side.

## Procedure and Discussion

In this part, we discussed four different issues. The first one was modulation and demodulation using MC4046 and PLLMC4046 modules. The second issue was modulation using LM566 modules. The third issue was modulation and demodulation using PLL LM565 module. The last one was the voltage and frequency conversion using LM565 module. All the steps that we followed and the needed figures were provided clearly.

### 2.1 Modulation and Demodulation using MC4046 and PLLMC4046 Modules

In this part, we are going to perform FM modulation and demodulation. First of all, before connecting the signal generator we have to adjust the variable resistor VR1 to get an output square wave signal with a free running frequency of 20 KHz (The free running frequency is a frequency at which a normally driven oscillator operates in the absence of a message signal). The following figure shows the signal that we got.

Figure 2.1: Output at FM/OP (adjusting free running frequency of VCO).

The next step was to connect the signal generator to the audio signal input (Audio I/P) and insert a sin wave signal with an amplitude of 10vp-p and frequency of 1 KHz, as shown in figure 2.2.

Figure 2.2:  The message signal.

We noticed that there are some frequency variations. As required we took three measurements of frequency variations which are 13.73 KHz, 21.06 KHz and 23.35 KHz.

The following figure shows a sin wave input signal and the corresponding modulated signal.

Figure 2.3: The modulated signal for sin wave input signal.

The same steps were repeated but using triangular input signal. The following figure shows the input signal and the modulated signal.

Figure 2.4: The modulated signal for triangular wave input signal.

We took three measurements of frequency variations which are 20.8 KH, 12.8 KHz and 15.93 KHz.

Also, the same steps were repeated but using square wave input signal. The following figure shows the input signal and the modulated signal.

Figure 2.4: The modulated signal for triangular wave input signal.

We took three measurements of frequency variations which are 26.7 KHz, 12.73 KHz and 25.5 KHz.

The next task was to adjust the free running frequency of VCO output port (TP1) to 20 KHz by changing VR1, after that we connected (FM/OP) of VCOMC4046 to (FM I/P) of the PLLMC4046. Then, we changed the amplitude of input signal at (Audio I/P) to a suitable value in order to recover the transmitted signal.

### Modulation Using LM566 Modules

First of all, J1 and J3 were set to short circuit and J2 to open circuit. After that, the oscilloscope was connected to (FM O/P) port and VR1 was adjusted to obtain the frequency of (FM O/P) equal to 20 KHz.

Then, the signal generator was connected to (Audio I/P) and sinusoidal signal of amplitude 10v p-p and frequency 1 KHz was selected.

Figure 2.7: The modulated signal of LM566 module.

After that, we connected the oscilloscope to the output of the FM modulator (FKM O/P) port, then three variations in frequency were taken which are: 27.9 KHz, 24.8 KHz and 16.1 KHz.

We repeated the last two steps for triangular and square signals.

For triangular signal the following figure shows the modulated signal and the input signal.

Figure 2.8: The modulated signal for triangular input signal.

We took three variations in frequency which are: 24.21 KHz, 19.6 KHz and 15.7 KHz.

For square signal the following figure shows the modulated signal and the input signal.

Figure 2.9: The modulated signal for square input signal.

We took three variations in frequency which are: 21.9 KHz, 15.1 KHz and 14.03 KHz.

The next task was to calculate the maximum frequency deviation, the modulation coefficient and the desired bandwidth for the square modulating signal.

m(t) square wave with fm= 1KHz

- Δf =  fmax – f0 = 21.9 – 20 = 1.9 KHz

- B = Δf/fm = 1.9K/1K =1.9 KHz

- BW = 2 fm (1+B) = 5.8 KHz

### Modulation and Demodulation Using PLL LM565 Module

First of all, J3 was set to short circuit and J1 and J2 open circuit to choose C5=10nf in LM565 module. The oscilloscope was connected to (VCO O/P) port and the variable resistor VR1 was adjusted so that the free running frequency (fo) equal to 20 KHz. The output port of (FM O/P) of the VCO LM566 module was connected to the input port (FM I/P) of the PLL LM565. Then, the function generator was used to generate a sinusoidal signal with maximum amplitude and frequency equals 1 KHz, and the output of the signal generator is connected to Audio signal input (Audio I/P), the oscilloscope was connected to the output of the FM detector (Audio O/P) and the final output is shown in figure below .

Figure 2.10: Demodulated signal in yellow against the original signal in blue.

We can notice from the previous figure, the modulated signal is a bit noisy due to some issues in the wires. Also we can’t adjust the variable resistances precisely to get extremely perfect output.

### 2.4 Voltage and Frequency Conversion using LM565 Module

In this part we will control the frequency of the input signal and capture the output voltage. First step was to make the connections as required by letting J1 and J3 open circuits and make J2 short circuit. Then we connected the function generator to get square-wave input signal with initial frequency equal 2 kHz and then attached it to the pin (FM I/P) of LM565 module. The output voltage was taken from the pin (Audio O/P).

To find the relation between the frequency and the amplitude we need to take many points and then plot these points in a graph.  The values for the input signal frequency were: 0.5 kHz, 1 kHz, 1.5 kHz and so on. The results were as shown in the table 2.1 below and the graph is plotted as shown in figure 2.11 below.

| Frequency | 0.5 kHz | 1 kHz | 1.5 kHz | 2 kHz | 2.5 kHz | 1 kHz | 3.5 kHz | 4 kHz | 4.5 kHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Amplitude | 0.93 V | 0.63 V | 0.46 V | 0.36 V | 0.30 V | 0.25 V | 0.23 V | 0.2 V | 0.18 V |

Table 2.1: Amplitude of (Audio O/P) at Each Frequency.

Figure 2.11: Amplitude of (Audio O/P) at Each Frequency Graph.

As we can notice from the figure above the relation between the voltage and the frequency of the VCO is a linear relation with a negative slope. In this case the frequencies are in the linear region of the VCO.

After that, we were asked to change the connections by letting J1 and J2 open circuits and make J3 short circuit. Then, we connected the function generator to get square-wave input signal with initial frequency equal 20 kHz and then attached it to the pin (FM I/P) of LM565 module. The output voltage was taken from the pin (Audio O/P).

Same as the previous requirements we found the points and filled it in the table as shown below. To find the amplitude we took the peek to peek value that is shown on the oscilloscope and divided it by 2 to find the amplitude.

| Frequency | 8 kHz | 12 kHz | 14 kHz | 15 kHz | 18 kHz | 22 kHz | 23 kHz | 24 kHz | 25 kHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Amplitude | 0.106 V | 0.0750 V | 0.0620 V | 0. 062 V | 0. 062 V | 0.0520 V | 0.046 V | 0.046 V | 0.046 V |

Table 2.2: Amplitude of (Audio O/P) at Each Frequency.

We drew amplitude vs. frequency graph using the points in table 2.2, as shown in the following figure.

Figure 2.12: Amplitude of (Audio O/P) at Each Frequency Graph.

The relation that is shown in the figure above is almost constant as the values are so close to each other and that is because the VCO is in the saturation mode in this range of high frequencies.

## Questions

What are the significant frequencies and the power in each harmonies of FM modulated wave for: β=0.5(refer to Bessel function table) and β=2(refer to Bessel function table(. When the carrier wave is: Vc(t) =8 cos(2π50000t(,What is the 99% power bandwidth for the each β? Refer to the table of Bessel functions.

Vc(t) =8 cos(2π50000t)        		Jn(B) = (-1)^n J-n(B) => (J1(B))^2 = (J-1(B))^2

For β = 0.5

- For n=0     P0 = (AC)22 (Jo (0.5)) ^2

P0 =  32*(0.94) ^2

P0 =  28.2752.

- For n=∓1  P1 = (AC)22 ((J1 (0.5)) ^2 + (J-1(0.5)) ^2)

P1  =  32 * (0.24^2 + 0.24^2)

P1   =  3.6864

- For n=∓2  P2 = (AC)22 ((J2 (0.5)) ^2 + (J-2(0.5)) ^2)

P2 = 32 * (0.03^2 + 0.03^2)

P2= 0.0576

We will stop at n=1 because  J0 2(t)+ 2J1 2(t)=0.9988  since J0 2(t)+2 J1 2(t)+ … + 2Jn(t) >= 99%

Bandwidth= 2(1+ β) fm = 2*(1+0.5)*fm = 3*fm

For β = 2

- For n=0     P0 = (AC)22 (Jo (0.5)) ^2

P0= 32 * (0.22) ^2

P0= 1.5488.

- For n=∓1  P1 = (AC)22 ((J1 (0.5)) ^2 + (J-1(0.5)) ^2)

P1= 32 * (0.58^2 + 0.58^2)

P1= 21.5296

- For n=∓2  P2 = (AC)22 ((J2 (0.5)) ^2 + (J-2(0.5)) ^2)

P2= 32 * (0.35^2 + 0.35^2)

P2 = 7.84

- For n=∓3  P3 = (AC)22 ((J3 (2)) ^2 + (J-3(2)) ^2)

P3= 32 * (0.13^2 + 0.13^2)

P3= 1.0816

- For n=∓4  	P4 = (AC)22 ((J4 (2)) ^2 + (J-4(2)) ^2)

P4 = 32 * (0.03^2 + 0.03^2)

P4 = 0.0576

We will stop at n=3 because J0 2(t)+2 J1 2(t)+  … + Jn(t)>= 99%

Bandwidth= 2(1+ β) fm = 2*(1+2)*fm =  6*fm.

## Conclusion

The aim of this experiment was understood successfully. This experiment helped us to understand the basics of the FM modulation and demodulation using different circuits each time. In modulation process we have used MC4046 and LM566 modulators. In addition, we noticed the influence of changing the frequency of the signal at the modulator voltage output.

Furthermore, we discussed the issue of voltage and frequency conversion using LM565 module and noticed that the relation between the input voltage and the output frequency is linear; when we increase the input voltage, the output frequency decreases. All the results were accepted, but in the demodulation process the demodulated signal was a bit noisy due to some issues in the wires. Also we can’t adjust the variable resistances precisely to get extremely perfect output. In general, this experiment was very useful and understandable.

## References

[1] http://www.silabs.com/Marcom%20Documents/Resources/FMTutorial.pdf, accessed on 21/6/2015 at 3:07PM.

[2] http://wits.ice.nsysu.edu.tw/course/pdfdownload/99CS/Comm-04-Phase%20and%20Frequency%20Modulation.pdf, accessed on 21/6/2015 at 3:28PM.

[3] http://ironbark.xtelco.com.au/subjects/DC/lectures/7/, accessed on 21/6/2015 at 3:48 PM.

[4] https://en.wikipedia.org/wiki/Demodulation, accessed on 21/6/2015 at 9:00PM.

[5] http://www.radio-electronics.com/info/rf-technology-design/fm-reception/fm-demodulation-detection-overview.php, accessed on 21/6/2015 at 9:15PM.