Faculty of Engineering and Technology

Electrical and Computer Engineering Department

Communications Lab (ENEE4103)

____________________________________________________

Experiment no.4 Report

Frequency Modulation

____________________________________________________

Prepared by:

Hadeel Abdallah		ID: 1111230

Partner’s name:

Saja Kharosha		ID: 1110710

Instructor: Mr. Mohammad Jubran

TA: Eng. Salma Sbeihat

March 10, 2015

Abstract

This experiment was carried out to apply FM modulation technique. Several properties of FM modulation were applied. Throughout this report, the procedure will be discussed of how to apply FM modulation, and how to detect zero crossings, and how to calculate the modulation index. Moreover, a comparison between AM and FM modulations.

Table of Contents

Theory	1

Procedure and Discussion	2

Part One: Dynamic Response of FM	2

Part Two: The Characteristic of FM Modulator	3

Part Three: Determination of the Frequency Deviation	4

Part Four: Measurements in the Frequency Domain of FM	5

Part Five: Determining the Carrier Zero Crossings by Varying the Frequency Deviation ∆FFM.	5

Part Six: Determine the carrier zero crossings by varying the modulated frequency fm.	6

Part Seven: FM Spectrum for Square Wave Modulation.	7

Questions	8

Conclusion	9

References	10

# Theory

In AM Modulation, the amplitude of a sinusoidal carrier wave is varied according with the baseband signal. In angle modulation, in which the angle of the carrier wave is varied according to the baseband signal, in this method the amplitude of the carrier wave is maintained constant. Angle modulation has two common forms: phase modulation PM and frequency modulation FM.

In frequency modulation (FM), the instantaneous frequency fi(t) is varied linearly with the message signal m(t), as shown by fit=fc+kfmt,  where kf is a constant that represents the frequency sensitivity of the modulator, expressed in Hertz per volt. The instantaneous phase of FM is expressed as θit=2πfct+2πkf0tmτdτ. Therefore, the frequency modulated signal is described as st=Accos⁡[2πfct+2πkf0tmτdτ].

Properties of FM signals: first, the constancy of transmitted power, the amplitude of FM waves is maintained at a constant value equal to the carrier amplitude Ac for all time t, irrespective of the sensitivity factor kf, therefore, the average transmitted power is: Pav=12Ac2. Second, nonlinearity of the modulation process, FM signals violate superposition laws, which complicates the spectral analysis and noise analysis. Third, Irregularity of zero-crossing, zero crossings are defined as the instants of time at which a waveform changes its amplitude from a positive value to a negative value or vice versa, FM signals do not have perfect regularity in their spacing across the time scale. Fourth, visualization difficulty of message waveform.

The most important property is the trade-off of increased transmission bandwidth for improved noise performance, the most important advantage of angle modulation over amplitude modulation is the realization of improved performance, this is attributed to the fact that the transmission of a message signal by modulating the angle is less sensitive to the presence of additive noise, however, this is attained at the increase of transmission bandwidth.

Frequency deviation is the maximum departure of the instantaneous frequency of the FM signal from fc, ∆f=kfAm. The modulation index is the ratio of the frequency deviation ∆f to fm, β=Δffm. Depending on the value of the modulation index β distinguish between two cases of FM: Narrow-band FM, for which β is small compered to one radian and Wide-band FM, for which β is large compered to one radian.

The spectrum of an FM signal contains a carrier component and an infinite set of a side frequencies located symmetrically on either side of the carrier at frequency separations  of fm, 2fm, 3fm, … the amplitude of the carrier component varies according to the Bessel function of β.

# Procedure and Discussion

This procedure will discuss FM modulation: dynamic response, characteristic of FM modulator, frequency deviation, measurements in the frequency domain of FM, determine the carrier zero crossings by varying the frequency deviation ΔFFM and by varying the modulating frequency FM, and finally the spectrum of a square wave.

## Part One: Dynamic Response of FM

Figure 1: FM modulation result of a sinusoidal message signal in the time domain.

The FM block was connected as indicated in the first part of the experiment, and the parameters were set as follows: fc=20KHz, fm=1.0KHz and Am=10Vpp, however, in the manual the value of Am was 20Vpp but the limitations of the equipment were 10Vpp. After setting up part one, the result was obtained as shown in figure 1. Although it is known that the result of FM modulation is a signal that has irregular zero crossings in the time domain, this irregularity is not noticeable in the following graph, because the variance of the phase that depends on the message signal is very small.

## Part Two: The Characteristic of FM Modulator

Figure 3: Spectrum of FM for a -10 DC voltage message

Figure 2: Spectrum of FM for a sinusoidal message wave

The spectrum of FM modulated signal is shown in figure 2, it has spectrums at the three dominant frequencies fc, fc+fm , fc-fm that are equal to 20KHz, 21KHz and 19KHz respectively. Then, a DC signal was connected to the FM modulator; the message signal amplitude was varied from -10 to 10 Vp-p by increasing one step at a time. At DC voltage -10 the spectrum of the signal is shifted to the left and by increasing its value the spectrum shifts to the right, at 0 DC voltage the spectrum is the same for the carrier signal (no shifts), the measured values are included in table 1, and the relationship between voltage and frequency is discussed later on.

Figure 4: Spectrum of FM signal for 0 DC voltage message.

Figure 5: Spectrum of FM signal for 10 DC voltage message.

| DC voltage | FM frequency (KHz) | DC voltage | FM frequency (KHz) |
| --- | --- | --- | --- |
| -10 | 19.330 | 1 | 20.099 |
| -9 | 19.427 | 2 | 20.160 |
| -8 | 19.489 | 3 | 20.221 |
| -7 | 19.543 | 4 | 20.282 |
| -6 | 19.611 | 5 | 20.349 |
| -5 | 19.684 | 6 | 20.422 |
| -4 | 19.757 | 7 | 20.502 |
| -3 | 19.836 | 8 | 20.551 |
| -2 | 19.879 | 9 | 20.618 |
| -1 | 19.971 | 10 | 20.703 |
| 0 | 20.010 |  |  |

Table 1: DC voltage vs. the change of FM frequency.

From the above table, figure 6 was drawn, it is obvious that there is a linear relationship between the DC voltage of the message signal and the frequency of the FM modulated signal.

Figure 6: DC voltage of the message signal vs. frequency relationship of FM wave.

Finally, the coefficient of the FM modulator was calculated as follows:

KFM=ΔfVCO∆U1=20.703-19.33010+10=1.37320=.0686K=60.86

## Part Three: Determination of the Frequency Deviation

Frequency deviation is the maximum departure of the instantaneous frequency of the FM signal from fc, it was calculated as follows:

∆FFM=KFM . ∆v2=60.86 . 102=343.25

## Part Four: Measurements in the Frequency Domain of FM

The parameters at this part were set as follows: fc=20KHz, Am=10Vpp and fm=300Hz and 200Hz respectively. The following two spectrums were obtained. It is noticed from the two graphs that decreasing the message signal frequency will result in more frequencies spectral lines in the FM spectrum and the amplitude of the frequencies in the spectrum varies according to the Bessel function.

Figure 8: The FM spectrum when fm=200Hz.

Figure 7: The FM spectrum when fm=300Hz.

## Part Five: Determining the Carrier Zero Crossings by Varying the Frequency Deviation ∆FFM.

The parameters at this part were set as follows: fc=20KHz, Am=0 Vpp and fm=100Hz. The following spectrums were obtained. It is noticed that increasing the message signal amplitude will result in more frequencies spectral lines in the FM spectrum and the amplitude of the frequencies in the spectrum varies according to the Bessel function. However, increasing the amplitude of the message signal until the carrier disappears happens at Am=7 Vpp, this value will be used to calculate the modulation index. The modulation index is the ratio of the frequency deviation ∆f to fm.

ηFM=Δffm=KFMAMfM=60.86×72×100=2.40

Figure 9: Spectral of FM at Am=0 vpp.

Figure 10: Spectral of FM at Am=4 vpp

## Part Six: Determine the carrier zero crossings by varying the modulated frequency fm.

The parameters at this part were set as follows: fc=20KHz, Am=10 Vpp and fm=3.0KHz. Zero crossings are the instants of time at which a waveform changes its amplitude from a positive value to a negative value or vice versa. The frequency of the message signal was decreased gradually to detect the first three zero crossings where the carrier spectral line disappears. The first zero crossing was at fm=275Hz, in figure 12. Moreover, the dominant frequencies were 18.887, 19.702, 19.427, 19.530, 20.082, 20.251, 20.526 and 20.801. The second zero crossing was at fm=120Hz and the third zero crossing was at fm=55.12Hz. The spectrums of the zero crossings are in the next page. The modulation index of each zero crossing was calculated in table 2 according to the following equation:

ηFM=Δffm=KFMAMfM=60.86×102×fM

| Zero crossing frequency (fM) in Hz | Modulation index η |
| --- | --- |
| 275 | 1.106 |
| 120 | 2.536 |
| 55.21 | 5.511 |

Table 2: Modulation index for zero crossing frequencies.

Figure 14: Spectral of FM at fm=55.21 (third zero crossing).

Figure 13: Spectral of FM at fm=120 (second zero crossing).

Figure 12: Spectral of FM at fm=275 (first zero crossing).

Figure 11: Spectral of FM at fm=3KHz.

## Part Seven: FM Spectrum for Square Wave Modulation.

A square wave signal was applied to the FM modulator with the following parameters: Am=10Vpp, duty cycle 50% and FM=200Hz. When the signal is a square wave higher frequency bandwidth is needed. The spectrum of the square wave is shown in figure 15.

Figure 15: The spectrum of a square wave signal after FM modulation.

# Questions

Q.1. Which carrier parameters can be used for angle modulation?

Phase in phase modulation PM and frequency in frequency modulation FM.

Q.2. Which angle modulation method is least affected by noise? Why?

Noise is better suppressed in FM systems as compared to PM Systems, because the information of the message signal is transmitted by the variations of the instantaneous frequency of a sinusoidal carrier wave, and its amplitude is maintained constant. Therefore, any variations of the carrier amplitude at the receiver input must result from noise or inference.

Q.3. What does pre-emphasis mean?

Pre-emphasis is to artificially emphasize the high frequency components of the message signal prior to modulation in the transmitter, and therefore before the noise is introduced in the receiver, in effect the low-frequency and high-frequency portions of the power spectral density of the message are equalized in such a way that the message fully occupies the frequency band allotted to it.

Q.4. When is de-emphasis used in FM demodulation? How does it function?

De-emphasis is performed at the receiver side after pre-emphasis has been used at the sender side. It de-emphasizes the high-frequency components, so as to restore the original power distribution of the message. In such a process, the high-frequency components of the noise at the discriminator output are also reduced, thereby effectively increasing the signal-to-noise ratio of the system.

Q.5. In FM with η = 1.0 how high are the amplitudes of the 1st sideline compared to AM for m = 100%? Given that the amplitude of the unmodulated carrier is AC = 1 V.

Q.6. Assuming in FM that the spectrum for a harmonic signal A1 = 5 V and fM1 = 1000 Hz is known. Can conclusions be drawn regarding the spectrum, which is produced for a different harmonic modulation signal, with, for example A1 = 3 V and fM1 = 100 Hz? Compare this with AM!

No, conclusions cannot be drawn because FM modulation is a nonlinear process, and therefore its spectral is more complex and follows Bessel function. However, AM is a linear modulation process and its spectral analysis is more simple.

# Conclusion

Frequency modulation is a form of angle modulation, which is a form of continuous wave modulation. Angle modulation uses a sinusoidal carrier whose angle is varied in accordance with a message signal.

In FM, the instantaneous frequency of a sinusoidal carrier is varied in proportion to the message signal. The instantaneous frequency is defined as the derivative of the phase with respect of time, except for the scaling factor (12π).

Unlike amplitude modulation, FM is a nonlinear modulation process; therefore, the spectral analysis of FM is more difficult than AM. The irregularities of zero crossings were detected.

In FM, the carrier amplitude, and therefore the transmitted average power are maintained constant, and this is an advantage of FM over AM in reducing the effects of noise or interference at reception. As the modulation index increases, the transmission bandwidth increases. Thus, FM provides a practical method for the tradeoff channel bandwidth for improved noise performance, which is not feasible in AM.

# References

[1] Communication systems by Simon Haykin and Michael Moher, fifth edition, pages 78-85.

[2]http://www.egr.msu.edu/classes/ece458/radha/ss07Keyur/Lab-Handouts/Session6-Experiment1.pdf  access date: March 10, 2015.

[3] http://wow.eecs.berkeley.edu/ergen/docs/Pages%20from%20indexofdm.pdf access date: March 10, 2015.