Electric And Computer Systems Engineering

Department

Communication Lab

“ENEE411”

# Report Experiment #4

# Frequency Modulation

Prepared For : Dr.Mohammad Jubran.

Prepared By : Inas Musleh 1111683.

Partners :    Ali Sider 1111829 , Shatha Ajaj 1100478.

Section : Saturday 2:00 - 4:50.

14th March, 2015

# Abstract

The aims of  this experiment are to become familiar with the frequency modulation , understand its dynamic response  and the characteristic of FM modulator ,also to deal with the spectrum of the signals in frequency domain.

# Theoretical Background

Frequency Modulation (FM) is a form of modulation in which changes in the carrier wave frequency correspond directly to changes in the baseband signal. FM is considered an analog form of modulation because the baseband signal is typically an analog waveform without discrete, digital values.

Frequency modulation uses the information signal, m(t)  to vary the carrier frequency within some small range about its original value. Here are the three signals in mathematical form:

## Information: m(t).

## Carrier: Ac(t) = Ac sin ( 2  fc t + 

## FM: VFM (t) = Ac sin (2 fc + (f/fm) m (t) t + 

We have replaced the carrier frequency term, with a time-varying frequency. We have also introduced a new term: f, the peak frequency deviation. We  should be able to see that the carrier frequency term:  fc + (f/Am) m (t) now varies between the extremes of fc - f and fc + f. The interpretation of f becomes clear: it is the farthest away from the original frequency that the FM signal can be. Sometimes it is referred to as the "swing" in the frequency.

We can also define a modulation index for FM, analogous to AM:

 = f/fm , where fm is the maximum modulating frequency used.

The simplest interpretation of the modulation index, is as a measure of the peak frequency deviation, f. In other words,  represents a way to express the peak deviation frequency as a multiple of the maximum modulating frequency, fm, i.e. f =  fm.

The  bandwidth of a FM signal may be predicted using: BW = 2 ( + 1 ) fm

where  is the modulation index and fm is the maximum modulating frequency used.

# Procedures And Calculations

## Part One : Dynamic response of FM

We  applied a sine wave from the function generator with amplitude (Am) equal to 20 Vpp , the frequency (fm)  equal to 1.0 kHz, and the carrier frequency (fc) equal to 20 kHz.

Using CASSY Lab2 (FM_TD) we load a diagram of an FM Signal to set the suitable values for the modulation, and then we used it  to display the output signal  by pressing F9 ,as shown in the Figure -1- below.

Figure -1- : The output Frequency Modulation signal in time domain .

How fast the cycle is completed is a function of the modulating frequency.

## Part Two: The Characteristic of the FM Modulator

We used the same setup in part one, and load the CASSY  Lab2 (FM_FFT),then we start measurement by pressing F9,the following Figure was the output .

Figure -2- : The output Frequency Modulation signal in frequency domain .

Then we used a DC-signal with -10 V of the function generator as input to FM modulator. The output signal  is shown in the Figure -3-.

Figure -3- : The output FM signal of  DC-input.

From the Figure -3- ,we can determine the frequency (fc) of the carrier from the spectrum, which is equal to 19 kHz.

Then we enhanced the DC- voltage in steps of 1 V and repeat each time the measurement of the carrier frequency. We note all frequency values in the table, as shown below.

| V | F(Hz) |
| --- | --- |
| -10 | 18.89 |
| -9 | 18.951 |
| -8 | 19.02 |
| -7 | 19.08 |
| -6 | 19.14 |
| -5 | 19.214 |
| -4 | 19.275 |
| -3 | 19.342 |
| -2 | 19.41 |
| -1 | 19.476 |
| 0 | 19.537 |
| 1 | 19.592 |
| 2 | 19.659 |
| 3 | 19.72 |
| 4 | 19.8 |
| 5 | 19.861 |
| 6 | 19.922 |
| 7 | 19.995 |
| 8 | 20.05 |
| 9 | 20.111 |
| 10 | 20.184 |

We draw the characteristic carrier frequency versus the control voltage of the VCO ,which is an oscillator designed so the output frequency can be changed by applying a voltage to its control port or tuning port. The Best fit line is as shown below:

Figure -5- : The characteristic of the FM modulator.

## y = 0.0647x + 19.535

The coefficient of the FM modulator can be determined from the Slope of this line ,

Where  KFM = fVCO /U1 ,which indicate to the minimum change in an input voltage that produces a change in the output frequency.

So the value of the coefficient of the FM modulator equal to 0.0647 *103 =64.7 .

## Part Three :Determination of the frequency deviation

The maximum frequency deviation ,which represents the maximum frequency difference between the instantaneous frequency and the carrier frequency, depends on the coefficient of the FM modulator and the amplitude of the modulating signal ,and sometimes it is referred to as the "swing" in the frequency.

, so its value will be :

FFM  = KFM * 20V/2

 = 64  * 20V/2

= 640.

## Part Four: Measurements in the frequency domain of FM

In this part we also use the same setup ,but we removed the bridging plug from the function generator to the FM modulator ,and  load the CASSY lab2(FM_FFT) .Then  We set the carrier frequency to 20.0 kHz with the potentiometer  and use a sinusoidal signal with AM = 20, and fm = 300 Hz of the function generator as the modulating signal sM(t).

The graph of the FM spectrum was as shown below

Figure -6- The spectrum of FM signal (fm=300Hz).

From the Figure -6-,we can see that the distance between two pulses  is 300 Hz. The pulses are ….,fc+2fm , fc+fm , fc , fc-fm , fc-2fm,…. ,the value of  fc is equal to 20 kHz ,which is the pulse  in the center .

We repeat the measurement of the FM spectrum for modulating signal Am=20 Vpp ,and fm=200Hz then fm=1 kHz to see the different between spectrum of FM signals.

Figure -7-:The spectrum of FM signal (fm=200Hz).

Figure -8-:The spectrum of FM signal (fm=1kHz).

We can obtain from the previous figures ,which display FM spectrum with different values of fm, that there are different  in the distance between pulses and in its number ,due to the change in the value of fm. For example ,when the fm=300Hz  ,the distance between two pulses  is 300Hz ,so we see more number of pulses with small distance between them ,but when fm=1kHz the distance is bigger and the number of pulses  is lower.

We also can see the attenuation in amplitude ,because the amplitude  depend on the value of fm  ,they also affected by Bessel function which is

Where  Jp(x) is the Magnitude of the frequency component ,P = Side frequency number (not to be confused with sidebands), X= Modulation index.

## Part Five: Determining the carrier zero crossings by varying the frequency deviation ΔFFM.

In this part we used  a sinusoidal signal with AM = 0 Vpp and fm = 100 Hz of the function generator ,then we  feed the modulating signal into the input of the FM modulator ,and load the CASSY Lab2 (FM_FFT). The output signal is shown in the following figure.

Figure -9- : The output modulated signal (AM=0V).

Then we slowly enhance the amplitude of the modulating signal AM of the function generator. We use Am=0.5V  then Am=1.0 V.

Figure -10- : The output modulated signal (AM=0.5V).

Figure -9- : The output  modulated signal (AM=1.0V).

We can observed from the previous  figures ,the decay of the carrier line. We continue reduced  the value of Am until the carrier line  disappeared ,as shown in the following figure.

Figure -11- :The output modulated signal when the carrier line disappear .

The value of the amplitude Am of the modulated signal when the carrier line disappeared was 7.4 ,which is peak to peak value ,to get  V peak we divide Vpeak to peak  by 2.

Now we can find the value of  the modulation index η , that determines exactly how much the carrier deviates from its center frequency,which is equal to

η FM = kFM *Am /fm

=  0.4 * Am

= 0.4 * (7.4/2)=1.48               ,(Vp=Vpp/2 = 7.4/2 )

## Part Six: Determine the carrier zero crossings by varying the modulating frequency FM.

We used a sinusoidal signal with Am = 20 Vpp , and fm = 3.0 kHz of the function generator as the modulating signal sM(t).Then we feed  the modulating signal into the input of the FM modulator ,and load the CASSY Lab 2 ( FM_FFT) .The output signal was as shown in Figure -12-.

Figure -12-: The output modulated signal (fm=3kHz).

Then we slowly reduce the frequency of the modulating signal fm of the function generator.

Figure -13-: The output modulated signal (fm=2.5kHz).

Figure -14-: The output modulated signal (fm= 1kHz).

We can observed from the previous  figures ,the decay of the carrier line. We continue reduced the value of Am until the carrier line  disappeared ,as shown in the following figure.

Figure -15-:The output modulation signal when the carrier line disappear .

From the previous figure ,The value of fm when the carrier line disappeared was 270Hz which is <500Hz .Then we calculated  the modulation index η :

η FM = kFM *Am /f   , Am=Vpp/2

=800/fm(Hz)

= 800/270

=2.9

## Part Seven: FM spectrum for square wave modulation

We Use a square wave signal with AM = 20 Vpp (amplitude 10 V, duty cycle 50%) and fm = 200 Hz of the function generator as the modulating signal sM(t).

Then we feed  the modulating signal into the input of the FM modulator. And load the CASSY Lab 2 ( FM_FFT) .The graphs of the FM spectrum with fm=200Hz ,500Hz respectively ,are  shown below.

Figure -16- :The FM spectrum (fm=200 Hz).

Figure -16- :The FM spectrum (fm=500 Hz).

From the previous figures ,we can obtain that when we increase the frequency fm ,the distance between pulses increase too. And also the fm affected on the Bandwidth  ,as we increase fm ,the bandwidth increase since

BW = 2 (η + 1 ) fm ,where η is the modulation index .

# Questions

Q.1. Which carrier parameters can be used for angle modulation?

Q.2. Which angle modulation method is least affected by noise? Why?

Q.3. What does preemphasis mean?

Increasing the amplitude of high frequency baseband signals in the FM modulator (transmitter) must be compensated for in the FM demodulator (receiver) otherwise the signal would sound quite tinny (too much treble).

The magnitude of the pre-emphasis response is defined by:

# Conclusion

Frequency Modulation (FM) is an important modulation scheme both because of its widespread commercial use, and because of its simplicity. In FM signals, because of the considerable side-bands produced, the efficiency is generally high. Recall (for example) that conventional AM is limited to about 33 % efficiency to prevent distortion in the receiver when the modulation index was greater than 1.

The bandwidth of an FM signal has a more complicated dependency than in the AM case (recall, the bandwidth of AM signals depend only on the maximum modulation frequency). In FM, both the modulation index and the modulating frequency affect the bandwidth. As the information is made stronger, the bandwidth also grows. In FM signals, the efficiency and bandwidth both depend on both the maximum modulating frequency and the modulation index. Compared to AM, the FM signal has a higher efficiency, a larger bandwidth and better immunity to noise.

The side-band structure is fairly complicated, but it is safe to say that the efficiency is generally improved by making the modulation index larger (as it should be). But if we make the modulation index larger, so make the bandwidth larger , which has its disadvantages. As is typical in engineering, a compromise between efficiency and performance is struck. The modulation index is normally limited to a value between 1 and 5, depending on the application.

there is virtually no interference picked up in the FM receiver. FM is sometimes called "static free, " referring to its superior immunity to random noise.