Communication Lab

Experiment #4: Frequency Modulation

Prepared By: Tamara Wa’ri 1110652 Partners: May Abdel Rahim 1110030 , Saleem Salameh

Dr. Mohammed Jubran 9/3/2015

# Abstract

This experiment describes the FM modulation using the direct method (VCO) in both the time and frequency domain.  It explains the properties and characteristics of FM modulation observed in measurements and through varying the parameters of the modulated signal. In this lab report, an introduction and procedure are explained with the obtained results. Moreover, discussing the results, comparison with AM modulation and conclusion are enlightened.

Table of Contents

Abstract	1

1.	Introduction	2

2.	Procedure and results	3

2.1	Dynamic response of FM	3

2.2	The Characteristic of the FM Modulator	3

2.3	Determination of the frequency deviation	5

2.4	Measurements in the frequency domain of FM	5

2.5	Determining the carrier zero crossings by varying the frequency deviation ΔFFM.	7

2.6	Determine the carrier zero crossings by varying the modulating frequency fM.	8

2.7 	     FM spectrum for square wave modulation	9

4.	Questions	10

5.	Discussion and conclusion	11

6.	References	11

# Introduction

Modulation: The process of modulation means to systematically use the information signal to vary some parameter of the carrier signal. The carrier signal is usually just a means to systematically vary one of the three parameters of the signal: amplitude, frequency or phase. Therefore, the type of modulation may be categorized as either

Figure 1: FM Generation with a VCO

1. AM: amplitude modulation

2. FM: frequency modulation

3. PM: phase modulation

Frequency modulation: Frequency modulation is a form of analog angle modulation in which the message or information signal m(t), varies the frequency of a carrier wave. The simplest approach to generating FM signals is to apply the message signal directly to a voltage-controlled oscillator (VCO) shown in figure 1. When a message signal exists, the instantaneous frequency of the output signal varies above and below the center frequency and is expressed by	 fi=fc + Kvcom(t) where KVCO is the voltage-to-frequency gain of the VCO expressed in units of Hz/V and KVCO*m(t), is the instantaneous frequency deviation. The instantaneous phase of the output signal expressed as θit=2πfct+2πkf0tmτdτ. Hence, the FM output signal, xFM(t), is given by the following equation: xFM(t) =Accos⁡[2πfct+2πkf0tmτdτ].

The amplitude of an FM signal is constant regardless of the message signal, giving it a constant envelope property with an output power equal to Ac2/2. The frequency-modulated output, xFM(t), has a nonlinear dependence to the message signal, m(t).

The quantity ∆f= KvcoAm represents the peak frequency deviation of the FM signal from the center frequency and is directly proportional to the amplitude of the message signal and the gain of the VCO. This quantity, ∆f , is called the maximum instantaneous frequency deviation. The ratio of the frequency deviation, ∆f , to the message signal frequency, fm, is called the modulation index, β. A key point of modulation index, β, is that it determines the bandwidth of the signal by determining the number of effective sidebands of an FM signal. In general, as the modulation index increases, the number of sidebands increases and the bandwidth goes up. However, the increase in modulation index due to decreasing message frequency (recall β = ∆ f/fm ) may not necessarily increase the FM bandwidth. The bandwidth is equal to the number of discrete spectral tones multiplied by the frequency spacing set by the message signal frequency fm. Frequency modulation can be classified as narrowband if the change in the carrier frequency is about the same as the signal frequency, or as wideband if the change in the carrier frequency is much higher (modulation index >1) than the signal frequency

The frequency spectrum of an actual FM signal has components extending infinitely, although their amplitude decreases and higher-order components are often neglected in practical design problems. For a single tone message signal, the number of significant sidebands in the output spectrum is a function of the modulation index. This can be seen by first writing the FM output signal in terms of nth order Bessel functions. A major advantage of FM compared with AM, is the possibility of improved Signal-to-noise ratio (SNR).

# Procedure and results

## Dynamic response of FM

The experiment was set up with the carrier frequency to center position: fC = 20 kHz. A sinusoidal signal with Am = 20 Vpp and fm = 1.0 kHz of the function generator was used as the modulating signal sM(t). The modulating signal was fed into the input of the FM modulator. The result of the FM modulated signal is shown in the figure below. It is noticed that the amplitude of the FM modulated signal is constant regardless of the message signal, giving it a constant envelope property as shown in figure 2.

Figure 2.1: FM modulated signal

## The Characteristic of the FM Modulator

The same experiment set up was used in this part. Result is shown below in figure 3 which represents the FM modulated signal with a sinusoidal input signal in the frequency domain. Using the potentiometer, the frequency of the carrier line was set to fc = 20.0 kHz as shown in figure 4. It is noticed that the spectrum of an FM signal contains a carrier component and an infinite set of a side frequencies located symmetrically on either side of the carrier at frequency separations  of fm, 2fm, 3fm,

Figure 4:  Spectrum of FM signal with fc=20 kHz

Figure 3: Spectrum of FM signal with sinusoidal input signal

A DC-signDA DC signal with -10 V of the function generator was used as input signal in order to study the characteristics of the VCO. The DC-signal was fed into the input of the FM modulator. The frequency fc of the carrier was determined from the spectrum in figure 5 where fc=20 kHz. It is noticed that by increasing the DC voltage form the FM signal shifts to the right.

Figure 5: Spectrum of FM signal with DC input voltage at DC=0

| DC voltage | fc=fvco |
| --- | --- |
| -10 | 19.385 |
| -9 | 19.458 |
| -8 | 19.519 |
| -7 | 19.58 |
| -6 | 19.647 |
| -5 | 19.714 |
| -4 | 19.788 |
| -3 | 19.855 |
| -2 | 19.916 |
| -1 | 19.977 |
| 0 | 20.05 |
| 1 | 20.13 |
| 2 | 20.184 |
| 3 | 20.245 |
| 4 | 20.306 |
| 5 | 20.38 |
| 6 | 20.441 |
| 7 | 20.508 |
| 8 | 20.581 |
| 9 | 20.642 |
| 10 | 20.715 |

The DC voltage was enhanced in steps of 1 V, each time the measurement of the carrier frequency fc = fVCO was repeated.  Measurements are shown below in Table 1. The characteristic of the carrier frequency versus the control voltage of the VCO for DC voltages from -10 to 10 was drawn as shown in figure 6. The coefficient of the FM modulator was determined from the linear relationship which represents the slope and can be calculated as follows:

KFM=ΔfVCO∆U1=20.508-19.38510+10=0.0662k

Figure 6: Linear relationship between fc (FM) and DC

Table 1: fc of varying DC voltages

## Determination of the frequency deviation

The maximum frequency deviation ∆f depends on the coefficient of the FM modulator Kvco and the amplitude of the modulating signal Am and it can be calculated as follows:

∆fFM=KFM . ∆v2=66.2 . 202=662

## Measurements in the frequency domain of FM

The same experiment setup of part one was used. Figure 7 below shows the spectrum of the FM modulated signal with DC input signal. As shown fc=19.922 kHz

Figure 7: spectrum of FM signal with DC input signal

The carrier frequency was set to 20.0 kHz using the potentiometer. The function generator was reconnected to the FM modulator with the bridging plug again. A sinusoidal signal was used of the function generator as the modulating signal sM(t with Am = 20 Vpp and fm = 300 Hz. The modulating signal was fed into the input of the FM modulator. The graph of FM spectrum is shown in figure 8. Steps were repeated with fm =200Hz. Since the spectrum of  FM signal contains a carrier component and an infinite set of a side frequencies on either side of the carrier at frequency separations  of fm, 2fm, 3fm, it was noticed that by increasing fm, distance between pulses increases thus less frequencies spectral lines appear in the FM spectrum. Regarding the amplitude values are the result of the Bessel function.

Figure 8: Spectrum of FM signal with fm=300 Hz

Figure 9: Spectrum of FM signal with fm= 200 Hz

## Determining the carrier zero crossings by varying the frequency deviation ΔFFM.

A sinusoidal signal of the function generator was used as the modulating signal sM(t) with AM = 0 Vpp and fm = 100 Hz. The modulating signal was fed into the input of the FM modulator. The amplitude of the modulating signal Am of the function generator was slowly enhanced to observe the decay of the carrier line as shown below in the figures and to observe the first zero crossing when the carrier line disappear; Ac=0. Values of Am with the corresponding Ac are recorded in table 2. The FM spectrum for the first carrier zero crossing is shown in figure 11. And the graph With AM the amplitude of the modulating signal in V and a fix modulating frequency fm = 100 Hz was sketched in figure 12. Am=7.3 Vpp when Ac=0. Using the Am value the modulation index is calculated as follows:

ηFM=Δffm=KFMAMfM=66.2×7.32×100=2.42

Figure 11: FM spectrum with first zero crossing

Figure 10: FM spectrum with Am=0 Vpp, fm=100 Hz

| AM | Amplitude of fc |
| --- | --- |
| 0 | 1.506 |
| 1 | 1.487 |
| 2 | 1.339 |
| 3 | 1.14 |
| 4 | 0.908 |
| 5 | 0.633 |
| 6 | 0.339 |
| 7 | 0.056 |
| 7.2 | 0.03 |
| 7.3 | 0.004 |
| 7.5 | 0.043 |
| 8 | 0.175 |
| 9 | 0.383 |

.

Table 2: varying Am with the corresponding Ac

Figure 12: Relationship between Ac and Am; first zero crossing at Am=7.3 Vpp

## Determine the carrier zero crossings by varying the modulating frequency fM.

A sinusoidal signal of the function generator was used as the modulating signal sM(t) with Am = 20 Vpp and fM = 3.0 kHz. The modulating signal was into the input of the FM modulator as shown in figure 13. The frequency of the modulating signal fm of the function generator was slowly reduced to observe the decay of the carrier line as shown in the figures below. It was noticed that the first zero crossing occurred at fm =277.7 Hz, second zero crossing occurred at fm=119.7 Hz and third zero crossing occurred at fm=76.66 Hz. Using the previous values the modulation index is calculated as follows: ηFM=Δffm=KFMAMfM=66.2×102×fM

- At the first zero crossing: ηFM=66.2×102×277.7=1.19

- At the second zero crossing: ηFM=66.2×102×119.7=2.76

- At the third zero crossing: ηFM=66.2×102×76.66=4.32

Figure 16: third zero crossing at fm=76.66 Hz

Figure 14: Second zero crossing at fm=119.7 Hz

Figure 13: FM spectrum with Am=20 Vpp and fm=2 kHz

Figure 145: First zero crossing at fm=227.7Hz

with AM = 20 Vpp (amplitude 10 V

# 2.7 	FM spectrum for square wave modulation

A square wave signal with of the function generator was used as the modulating signal sM(t) with AM = 20 Vpp, duty cycle 50% and fM = 200 Hz. The modulating signal was fed into the input of the FM modulator.  The duty cycle describes the amount of time the signal is in a high (on) state as a percentage of the total time of it takes to complete one cycle as shown in the figures below when varying the duty cycle. For square wave modulation a higher bandwidth is required. When varying fm as explained before the higher fm the less pulses appear on the spectrum with smaller distance fm between each pulse.

# Questions

Q.1 Which carrier parameters can be used for angle modulation?

Frequency and phase; FM modulation and PM modulation.

Q.2 Which angle modulation method is least affected by noise? Why?

FM is least affected by noise than PM Since FM signals have constant amplitude, FM receivers normally have limiters that remove AM noise, further improving SNR. FM with wider deviation can achieve even greater improvements.

Q.3. What does pre emphasis mean?

An Additional techniques to improve noise reduction, Pre-emphasis increases the magnitude of the higher signal frequencies, thereby improving the signal-to-noise ratio.

Q.4. When is de-emphasis used in FM demodulation? How does it function?

At the output of the discriminator in the FM receiver, a de-emphasis network restores the original signal power distribution.  system process designed to decrease, (within a band of frequencies), the magnitude of some (usually higher) frequencies with respect to the magnitude of other (usually lower) frequencies in order to improve the overall signal-to-noise ratio by minimizing the adverse effects of such phenomena as attenuation differences or saturation of recording media in subsequent parts of the system.

Q.5. In FM with η = 1.0 how high are the amplitudes of the 1st sideline compared to AM for

m = 100%? Given that the amplitude of the unmodulated carrier is AC = 1 V.

According to Bessel function for n=1 amplitudes of the 1st sideline=0.44

Q.6. Assuming in FM that the spectrum for a harmonic signal A1 = 5 V and fM1 = 1000 Hz is known. Can conclusions be drawn regarding the spectrum, which is produced for a different harmonic modulation signal, with, for example A1 = 3 V and fM1 = 100 Hz? Compare this with AM!

No because FM modulation in nonlinear unlike AM modulation. Moreover amplitude values of FM modulated signal are obtained from Bessel function.

# Discussion and conclusion

FM signals can be generated using either direct or indirect frequency modulation. Direct FM modulation can be achieved by directly feeding the message into the input of a VCO. For indirect FM modulation, the message signal is integrated to generate a phase-modulated signal. Frequency modulation is used in radio, telemetry, radar, and seismic prospecting. In radio systems, frequency modulation with sufficient bandwidth provides an advantage in cancelling naturally-occurring noise.

When comparing FM with AM modulation. Both transmit the information in the form of electromagnetic waves. AM works by modulating (varying) the amplitude of the signal or carrier transmitted according to the information being sent, while the frequency remains constant. This differs from FM in which information (sound) is encoded by varying the frequency of the wave and the amplitude is kept constant. AM has poorer sound quality compared with FM, but is cheaper and can be transmitted over long distances. It has a lower bandwidth so it can have more stations available in any frequency range.  FM is less prone to interference than AM. However, FM signals are impacted by physical barriers. FM has better sound quality due to higher bandwidth and reduction of noise.  Finally Zero crossing in modulated signal is not equidistant only in FM modulation.

# References

[1]: http://www.silabs.com/Marcom%20Documents/Resources/FMTutorial.pdf // accessed at

[2]: http://en.wikipedia.org/wiki/Frequency_modulation