Faculty of Engineering and Technology

Electrical and Computer Engineering Department

Communication Lab #ENEE4103

EXP #3&4.Report

Angle Modulation

Prepared by:

Ehab Amriah				  1120871

Ahmad Dar Khalil			  1120443

Ahmad Zeid‎‏                                1131767

Assistant : Eng. Rana Alqaisi

Date : 25/7/2016

Sec: 01

Table of Contents

Theory	1

Procedure and Discussion	4

2.1.	Angle Modulation – Part 1

Part One: Dynamic Response of FM	4

Part Two: The Characteristic of FM Modulator	5

Part Three: Determination of the Frequency Deviation	7

Part Four: Measurements in the Frequency Domain of FM	7

Part Five: Determining the Carrier Zero Crossings by Varying the Frequency Deviation ∆FFM.	7

Part Six: Determine the carrier zero crossings by varying the modulated frequency fm.	8

Part Seven: FM Spectrum for Square Wave Modulation.	9

2.2. Angle Modulation – Part 2

Part One: FM demodulation for loop filter T2	11

Part Two: FM demodulation for loop filter T1	13

Part Three: FM pre-emphasis	14

Part Four: Dynamic response of phase modulation (PM):	16

Part Five: Characteristic of the PM modulator .	16

Part Six:. The PM spectrum	17

Part Seven: PM demodulation for loop filter T2……………………………………....18

Part Eight:. PM demodulation for loop filter T1 	18

Questions	10

Conclusion	21

References	22

Abstract

In this experiment we will generate and plot an FM modulated signal for different types of message signals [Sinusoidal, Triangular and Square]. we will Describe The Frequency Modulation Technique by Simulate it`s circuit and notice the effects of the different changes in the Frequencies of the input signal at the modulator Voltage output .Understanding and becoming familiar with the FM . Dealing also with the spectrum of the signals [Sinusoidal, Triangular and Square].

Finally we will experimentally see the effect of demodulation circuit on different loop filters on different time constants.

- Theory

  - Frequency Modulation FM :

Frequency Modulation (FM) is a major form of modulation in which it conveys information over a carrier wave by varying its instantaneous frequency. This is mainly different from the  Amplitude modulation, in which the amplitude of the carrier is varied while its frequency remains constant . but here For generating angle modulation, the amplitude of the modulated carrier is held constant and either the phase or the time derivative of the phase is varied linearly with the message signal[2]

- First , consider the sinusoidal carrier signal to be:

[ Ac cos(2πfct+θ(t))]

where fc  is the carrier's base frequency and Ac is the carrier's amplitude .

- Instantaneous frequency is given as  = fit= 12π dθdt=fc+12π dødt

- for frequency modulation  :  12π dθ(t)dt=kf m(t)

*where Kf is the proportionality constant(hz/volt) and m(t) is the message signal. And θ (t) is given by :

- Combining the last two previous equations we get : Fi(t)=fc+kf m(t)

- So now The modulator combines the carrier with message signal m(t)= Am cos(2πfmt)]

signal to get the transmitted signal:

S(t)=Ac Sin[2π+2πKf-∞tmτ τd]

St=Ac cos(2πfct+kfAmfm sin⁡(2πfmt))

This is called the single tone frequency modulation which m(t) is a sinusoidal signal. Look at Fig 1.1

Figure1.1: output of fm

- WE will also introduce two new terms

- The [ Frequency deviation ] :

Frequency deviation  ∆f=  max12π dθdt

Frequency deviation for FM signals  ∆f=kfmax12π dθdt

*kf is a frequency deviation constant

- the modulation index β as :

* fm is the highest frequency component present in the modulating signal m(t),

- Now  β will lead us to two new Definitions : NBFM , WBFM ,

If β << 1 so the modulation is called narrowband FM [ NBFM ].

and its bandwidth is approximately =2

If β >> 1 so the modulation is called Wideband FM [ WBFM ].

bandwidth is approximately =2 . [1]

Note 1 : While wideband FM uses more bandwidth, it can improve signal-to-noise ratio significantly.

Note 2: FM is considered an analog form of modulation because the baseband signal is typically an analog waveform without discrete, digital values.

Note 3: The instantaneous frequency; fi(t) will have the same shape of that of the message since the message is hidden in the frequency

  - Carson’s Rule:

It`s  used to find the bandwidth and power of the FM by the Formula

BT=2(∆f+fm)

Or

BT=2(1+ β)Fm

And it indicates (98 %) of the power of a frequency-modulated signal lies within a bandwidth BT .[3]

  - Generation of FM signals

In a direct FM, the instantaneous frequency of the carrier is varied in accordance with m(t) by means  of a voltage – controlled oscillator (VCO).which  is a linear frequency modulator. It is an oscillator with a voltage reactance that controls the oscillator output frequency

The instantaneous frequency of the VCO is given by:

And The VCO Characteristics is :

The VCO output Represents the FM signal which is :

  - Demodulation of FM signals

An FM signal has the following form :
gm(t)=Ac Sin[2π+2πKf-∞tmτ τd]

So, its magnitude is constant with value of A. The information is contained in the frequency (or angle) of the FM signal. To extract the message signal contained in an FM signal, we can transfer the information from the angle to the magnitude by simply differentiating the FM signal. Since the derivate of a sinusoid results in multiplying the magnitude of the sinusoid by the derivate of its angle, the derivative of the above FM signal becomes :

dgfm(t)dt= -Ac  2π+Kfmtsin(2πt+Kf-∞tmτ τd)

So, the message signal of the above derivative is contained in the frequency of the sinusoid and also in its magnitude. It is AM + FM signal. Passing the derivative of the FM signal through an envelope detector will give the desired message signal at the output. Therefore, the following block diagram is an FM demodulator.

- Procedure and Discussion

  - Angle Modulation – Part 1

This part will discuss FM modulation: dynamic response, characteristic of FM modulator, frequency deviation, measurements in the frequency domain of FM, determine the carrier zero crossings by varying the frequency deviation ΔFFM and by varying the modulating frequency FM, and finally the spectrum of a square wave.

## Part One: Dynamic Response of FM

Figure2.1: FM modulation result of a sinusoidal message signal in the time domain.

The FM block was connected as indicated in the first part of the experiment, and the parameters were set as follows: fc=20KHz, fm=1.0KHz and Am=10Vpp, however, in the manual the value of Am was 20Vpp but the limitations of the equipment were 10Vpp. After setting up part one, the result was obtained as shown in figure 2.1. Although it is known that the result of FM modulation is a signal that has irregular zero crossings in the time domain, this irregularity is not noticeable in the following graph, because the variance of the phase that depends on the message signal is very small.

## Part Two: The Characteristic of FM Modulator

The spectrum of FM modulated signal is shown in figure 2.2, it has spectrums at the three dominant frequencies fc, fc+fm , fc-fm that are equal to 20KHz, 21KHz and 19KHz respectively. Then, a DC signal was connected to the FM modulator; the message signal amplitude was varied from -10 to 10 Vp-p by increasing one step at a time. At DC voltage -10 the spectrum of the signal is shifted to the left and by increasing its value the spectrum shifts to the right, at 0 DC voltage the spectrum is the same for the carrier signal (no shifts), the measured values are included in table 1, and the relationship between voltage and frequency is discussed later on.

Figure 2.4: Spectrum of FM signal for 0 DC voltage message.

Figure 2.3: Spectrum of FM for a -10 DC voltage message

Figure 2.1: Spectrum of FM for a sinusoidal message wave

Figure 2.5: Spectrum of FM signal for 10 DC voltage message.

| DC voltage | FM frequency (KHz) | DC voltage | FM frequency (KHz) |
| --- | --- | --- | --- |
| -10 | 19.330 | 1 | 20.099 |
| -9 | 19.427 | 2 | 20.160 |
| -8 | 19.489 | 3 | 20.221 |
| -7 | 19.543 | 4 | 20.282 |
| -6 | 19.611 | 5 | 20.349 |
| -5 | 19.684 | 6 | 20.422 |
| -4 | 19.757 | 7 | 20.502 |
| -3 | 19.836 | 8 | 20.551 |
| -2 | 19.879 | 9 | 20.618 |
| -1 | 19.971 | 10 | 20.703 |
| 0 | 20.010 |  |  |

Table 1: DC voltage vs. the change of FM frequency.

From the above table, figure 2.6 was drawn, it is obvious that there is a linear relationship between the DC voltage of the message signal and the frequency of the FM modulated signal.

Figure 2.6: DC voltage of the message signal vs. frequency relationship of FM wave.

Finally, the coefficient of the FM modulator was calculated as follows:

KFM=ΔfVCO∆U1=20.703-19.33010+10=1.37320=.0686K=60.86

## Part Three: Determination of the Frequency Deviation

Frequency deviation is the maximum departure of the instantaneous frequency of the FM signal from fc, it was calculated as follows:

∆FFM=KFM . ∆v2=60.86 . 102=343.25

## Part Four: Measurements in the Frequency Domain of FM

The parameters at this part were set as follows: fc=20KHz, Am=10Vpp and fm=300Hz and 200Hz respectively. The following two spectrums were obtained. It is noticed from the two graphs that decreasing the message signal frequency will result in more frequencies spectral lines in the FM spectrum and the amplitude of the frequencies in the spectrum varies according to the Bessel function.

According to the Bessel function.

Figure 2.7: The FM spectrum when fm=300Hz.

Figure2.8: The FM spectrum when fm=200Hz.

## Part Five: Determining the Carrier Zero Crossings by Varying the Frequency Deviation ∆FFM.

The parameters at this part were set as follows: fc=20KHz, Am=0 Vpp and fm=100Hz. The following spectrums were obtained. It is noticed that increasing the message signal amplitude will result in more frequencies spectral lines in the FM spectrum and the amplitude of the frequencies in the spectrum varies according to the Bessel function. However, increasing the amplitude of the message signal until the carrier disappears happens at Am=7 Vpp, this value will be used to calculate the modulation index. The modulation index is the ratio of the frequency deviation ∆f to fm.

ηFM=Δffm=KFMAMfM=60.86×72×100=2.40

Figure 2.9: Spectral of FM at Am=0 vpp.

Figure 2.10: Spectral of FM at Am=4 vpp

## Part Six: Determine the carrier zero crossings by varying the modulated frequency fm.

The parameters at this part were set as follows: fc=20KHz, Am=10 Vpp and fm=3.0KHz. Zero crossings are the instants of time at which a waveform changes its amplitude from a positive value to a negative value or vice versa. The frequency of the message signal was decreased gradually to detect the first three zero crossings where the carrier spectral line disappears. The first zero crossing was at fm=275Hz, in figure 12. Moreover, the dominant frequencies were 18.887, 19.702, 19.427, 19.530, 20.082, 20.251, 20.526 and 20.801. The second zero crossing was at fm=120Hz and the third zero crossing was at fm=55.12Hz. The spectrums of the zero crossings are in the next page. The modulation index of each zero crossing was calculated in table 2 according to the following equation:

ηFM=Δffm=KFMAMfM=60.86×102×fM

| Zero crossing frequency (fM) in Hz | Modulation index η |
| --- | --- |
| 275 | 1.106 |
| 120 | 2.536 |
| 55.21 | 5.511 |

Table 2: Modulation index for zero crossing frequencies.

Figure 2.13: Spectral of FM at fm=120 (second zero crossing).

Figure 2.11: Spectral of FM at fm=3KHz.

Figure 2.12: Spectral of FM at fm=275 (first zero crossing).

Figure 2.14: Spectral of FM at fm=55.21 (third zero crossing).

## Part Seven: FM Spectrum for Square Wave Modulation.

A square wave signal was applied to the FM modulator with the following parameters: Am=10Vpp, duty cycle 50% and FM=200Hz. When the signal is a square wave higher frequency bandwidth is needed. The spectrum of the square wave is shown in figure 15.

Figure 2.15: The spectrum of a square wave signal after FM modulation.

  - Angle Modulation – Part 2

## Part one: FM demodulation for loop filter T2:

### 2.1.1 Time response of the FM modulating system:

Figure 3: FM demodulation with fm = 1 KHz

Figure 1: FM demodulation with fm = 500 Hz

Figure 2: Setting fc to 20 KHz

The circuit for this part was connected setting fm to 500 Hz and Am to 20 Vpp for a sinusoidal modulating signal, where the modulating signal was fed into the input of the FM modulator and the loop filter of the FM demodulator was set to T2. The CASSY Lab 2 example FM_TDem.labx was loaded. Two signals were displayed on the CASSY software; the input of the FM modulator on channel A and the output signal of the FM demodulator on channel B as shown in figure 1. The frequency of the modulating signal was increased to 1 KHz after setting the carrier frequency to 20 KHz to enhance the signal as shown in figures 2 and 3.

### 2.1.2 Transfer characteristic of the FM modulating system:

In this part, a sinusoidal signal with Am = 2 Vpp anf fm = 500 Hz was used. The CLASSY Lab 2 example FM_FFT_Dem.labx was loaded and resulted with the output shown in figure 4. The amplitude of the demodulated signal AD with the spectrum analyzer was equal to 1.09 volts when fm was equal to 500 Hz. Table 1 shows the resulted AD for different values of the frequency of the modulating signal.

Figure 4: Transfer characteristic of the FM modulating signal

| AD | Fm |
| --- | --- |
| 1.09 | 500 |
| 1.09 | 1000 |
| 1.1 | 1500 |
| 1.05 | 2000 |
| 0.97 | 2500 |
| 0.82 | 3000 |
| 0.64 | 3500 |
| 0.53 | 4000 |
| 0.44 | 4500 |
| 0.34 | 5000 |

Table 3: AD for different values of Fm

Figure 5 shows the relation between the amplitude of the demodulated signal and the frequency of the modulating signal, which represents the response of a low-pass filter.

Figure 5: AD vs. Fm

## Part two: FM demodulation for loop filter T1:

### 2.2.1 Time response of the FM modulating system:

The same steps done in part 2.1.1 were repeated but with setting the loop filter of the FM demodulator to T1. The two displayed signals are shown in figure 6.

It’s shown form the figure that the signal is chopped, that’s because the voltage of the signal is higher than the voltage that is supplied by the VCO.

Figure 6: FM demodulation for loop filter T1

### 2.2.2 Transfer characteristic of the FM modulating system:

The output resulted in this part is shown in figure 7. The amplitude of the demodulated signal AD with the spectrum analyzer was equal to 4.62 volts when fm was equal to 500 Hz. Table 2 shows the resulted AD for different values of the frequency of the modulating signal.

| Fm | AD |
| --- | --- |
| 500 | 4.62 |
| 1000 | 2.42 |
| 1500 | 1.56 |
| 2000 | 1.11 |
| 2500 | 0.83 |
| 3000 | 0.64 |
| 3500 | 0.51 |
| 4000 | 0.43 |
| 4500 | 0.36 |
| 5000 | 0.27 |

Figure 7: Transfer characteristic of the FM modulating system for loop filter T1

Table 4: AD for different values of Fm

Figure 8 shows the relation between the amplitude of the demodulated signal and the frequency of the modulating signal, which represents the response of a high-pass filter.

Figure 8: AD vs. Fm

## Part three: FM pre-emphasis:

The components were assembled to build the circuit needed for this part with setting the loop filter of the FM modulator to T2. A sinusoidal modulating signal with Am =5 Vpp and fm = 500 Hz was used.

### 2.3.1 Time characteristic of the input signals:

The input of the pre-emphasis stage was displayed on channel A of the Sensor-CASSY2, and the CASSY Lab 2 example FM_preDemm.labx was loaded. The measurements were repeated sequentially for another two values of the frequency of the modulating signal: 1000 Hz and 2000 Hz. The sketch of the time response of the input signal of the pre-emphasis stage is shown in figure 9; where the signal in purple is for fm=500 Hz, the signal in green is for fm = 1000 Hz and the signal in blue is for fm = 2000 Hz.

Figure 9: Time characteristic of the input signals

### 2.3.2 Time characteristic of the output signals:

The steps in the previous parts were repeated but with displaying the signal at the output of the FM demodulator low-pass filter (.) on channel A as shown in figure 10; where the signal in purple is for fm=2000 Hz, the signal in green is for fm = 1000 Hz and the signal in blue is for fm = 500 Hz.

Figure 20: Time characteristic of the output signals

## Part four: Dynamic response of phase modulation (PM):

The circuit for this part was connected setting the carrier frequency to center position: fc =20 Hz, setting the function generator to a sinusoidal modulating signal with Am = 2 Vpp and fm = 1000 Hz and feeding the modulating signal into the input of the PM modulator. Two messages were displayed using the CASSY software; the modulating signal on channel A and the PM signal at the output of the PM modulator on channel B. The CASSY Lab 2 example PM_TDscope.labx was loaded and the output resulted is shown in figure 11.

Figure 11: PM modulation

## Part five: Characteristic of the PM modulator:

In this part, the function generator was set to DC, and different values of DC voltages from -1 volt to 1 volt were fed into the input of the PM modulator. The CASSY Lab 2 example PM_TD.labx was loaded. The phase shift between the carrier oscillations of the FM and PM output was determined for each value of the DC voltage, using the set marker/ vertical line and measure difference of CASSY Lab 2 as shown in figure 12. The phase shift is calculated by the following equation: Phase shift= (Δt * 2π)/T; where Δt is the time difference between the two signals, T is the period of the signals which equals 50 and 2π=360ₒ. The phase shift for each value of the DC voltage is shown in table 3 and the relation between the DC voltage and the phase value of the

shift is shown in figure 14.

Figure 13: Calculating the period T

Figure 12: FM and PM output

Figure 14: The phase shift vs. DC voltage

## Part six: The PM spectrum:

The CASSY Lab 2 example PM_FFT.labx was loaded and the spectrum at the PM modulator output was displayed on channel B of the sensor CASSY2 setting the function generator to a sinusoidal modulating signal with Am=2 Vpp and fm=300 Hz. The graph of the PM spectrum is shown in figure 15. The same steps were done for a modulating signal with fm = 200 Hz with the output shown in figure 16.

Figure 16: PM spectrum with fm=200 Hz

Figure 15: PM spectrum with fm=300 Hz

## Part seven: PM demodulation for loop filter T2:

The circuit for this part was connected with setting the toggle switch of the PM demodulator to T2 and loading the CASSY Lab 2 example PM_TDem.labx. Three different values were given to fm and their output was obtained as follows: figure 17 for fm=1000 Hz, figure 18 for fm=500 Hz and figure 19 for fm = 1500 Hz.

Figure 19: PM demodulation with fm=1500 Hz

Figure 18: PM demodulation with fm =500 Hz

Figure 17: PM demodulation with fm = 1 KHz

## Part eight: PM demodulation for loop filter T1:

The same steps in the previous part were done but with setting the toggle switch of the PM demodulator to T1. Three different values for fm were used; fm = 1000 Hz as shown in figure20, fm=500 Hz as shown in figure 21 and fm = 1500 Hz as shown in figure 22.

Figure 3: PM demodulation with fm = 1500 Hz

Figure 41: PM demodulation with fm = 500 Hz

Figure 20: PM demodulation with fm=1000 Hz

Questions

Q.1. Which carrier parameters can be used for angle modulation?

Phase in phase modulation PM and frequency in frequency modulation FM.

Q.2. Which angle modulation method is least affected by noise? Why?

Noise is better suppressed in FM systems as compared to PM Systems, because the information of the message signal is transmitted by the variations of the instantaneous frequency of a sinusoidal carrier wave, and its amplitude is maintained constant. Therefore, any variations of the carrier amplitude at the receiver input must result from noise or inference.

Q.3. What does pre-emphasis mean?

Pre-emphasis is to artificially emphasize the high frequency components of the message signal prior to modulation in the transmitter, and therefore before the noise is introduced in the receiver, in effect the low-frequency and high-frequency portions of the power spectral density of the message are equalized in such a way that the message fully occupies the frequency band allotted to it.

Q.4. When is de-emphasis used in FM demodulation? How does it function?

De-emphasis is performed at the receiver side after pre-emphasis has been used at the sender side. It de-emphasizes the high-frequency components, so as to restore the original power distribution of the message. In such a process, the high-frequency components of the noise at the discriminator output are also reduced, thereby effectively increasing the signal-to-noise ratio of the system.

Q.6. Assuming in FM that the spectrum for a harmonic signal A1 = 5 V and fM1 = 1000 Hz is known. Can conclusions be drawn regarding the spectrum, which is produced for a different harmonic modulation signal, with, for example A1 = 3 V and fM1 = 100 Hz? Compare this with AM!

No, conclusions cannot be drawn because FM modulation is a nonlinear process, and therefore its spectral is more complex and follows Bessel function. However, AM is a linear modulation process and its spectral analysis is more simple.

III . Conclusion

Frequency modulation is a form of angle modulation, which is a form of continuous wave modulation. Angle modulation uses a sinusoidal carrier whose angle is varied in accordance with a message signal.

In FM, the instantaneous frequency of a sinusoidal carrier is varied in proportion to the message signal. The instantaneous frequency is defined as the derivative of the phase with respect of time, except for the scaling factor (12π).

Unlike amplitude modulation, FM is a nonlinear modulation process; therefore, the spectral analysis of FM is more difficult than AM. The irregularities of zero crossings were detected.

In FM, the carrier amplitude, and therefore the transmitted average power are maintained constant, and this is an advantage of FM over AM in reducing the effects of noise or interference at reception. As the modulation index increases, the transmission bandwidth increases. Thus, FM provides a practical method for the tradeoff channel bandwidth for improved noise performance, which is not feasible in AM.

# IV.References

[1] Communication systems by Simon Haykin and Michael Moher, fifth edition, pages 78-85.

[2]http://www.egr.msu.edu/classes/ece458/radha/ss07Keyur/Lab-Handouts/Session6-Experiment1.pdf  access date: 24/7/2016.

[3] http://wow.eecs.berkeley.edu/ergen/docs/Pages%20from%20indexofdm.pdf access date: 24/7/2016.