Electrical and Computer Systems Engineering Department

Communications Lab

ENEE 4103

Prelab for EXP. No. 1

AM and DSBsc Modulation and Demodulation

Student Name: Aseel Awad           Student ID: 1130776

Instructor: Wael Hashlamoun        Section.No. 3

Assistant:Eng. Ahmad Faza           Date: 24/06/2016

1. Write the mathematical expression for the demodulated signal.

The demodulated signal is:

Y(t) =Ac|1+μcosωmt|

2. Use MATLAB command and M files to draw the demodulated signal for the following three cases:

a. Ac=16v, modulation index=0.22, modulating signal frequency=800Hz

The output is Y(t) =16 |1+0.22cosωmt|, the maximum value is 16|1+0.22| = 19.52 and the minimum value is 16|1-0.22| = 12.48. Since the modulating index µ is less than 1, the message signal can be extracted without distortion.

b. Ac=16v, modulation index=1, modulating signal frequency=800Hz

The output is Y(t) =16 |1+cosωmt|, the maximum value is 16|1+1| = 32 and the minimum value is 16|1-1| =0. Since the modulating index µ is 1, the message signal can be extracted without distortion.

c. Ac=16v, modulation index=1.85, modulating signal frequency=800Hz

The output is Y(t) =16 |1+1.85cosωmt|, the maximum value is 16|1+1.85| = 45.6 and the minimum value is 16(1.85-1)=-13.6 .We can see that there are some negative value which will be reflected to positive values using the absolute value function, this will cause a distortion or over-modulating problem and the  message signal can't  be extracted. So, we can conclude that the modulating index must be less than or equal to 1 to be able to extract the message signal correctly.