Electrical and Computer Systems Engineering Department

Communications Lab

ENEE 4103

Report for EXP. No. 1

AM and DSBsc Modulation and Demodulation

Student Name: Aseel Awad           Student ID: 1130776

Instructor: Wael Hashlamoun        Section.No. 3

Assistant:Eng. Ahmad Faza           Date: 12/12/2016

# Abstract

In this experiment, we have reviewed the theoretical definition for  two types of amplitude modulation; AM(normal Amplitude Modulation) and DSBsc (Double Side Band Surprised Carrie), the demodulation techniques that are used in both and how to how to find the spectrum of each modulation.

Table of Contents

Abstract	2

1. Theory	4

1.1 Modulation and Types of Modulation	4

1.2 Normal Amplitude Modulation	4

1.3 DSBsc Modulation	6

2. Procedure and Discussion	8

2.1 AM Modulation	8

2.2 DSBSC Modulation	10

2.3 Spectrum of AM Modulation	11

2.4 Spectrum of DSBSC Modulation	12

2.5 DSBsc Synchronous Demodulation	13

2.6 DSBsc synchronous demodulation with the aid of a PLL-Controlled VCO	14

3. Questions	15

4. Conclusion	17

5. References	18

# 1. Theory

## 1.1 Modulation and Types of Modulation

Modulation is defined as the process by which some characteristic of a carrier c(t) is varied in accordance with a message signal m(t). There are two types of modulation: [1] [2]

1. Amplitude Modulation is defined as the process in which the amplitude of the carrier c(t) is varied linearly with m(t). This experiment will consider two types of amplitude modulation. These are normal amplitude modulation, double sideband suppressed carrier modulation.

2. Phase Modulation is defined as the process in which the amplitude of the modulated carrier is held constant and either the phase or the time derivative of the phase is varied linearly with the message signal m(t).

A common form of the carrier, in the case of continuous wave modulation, is a sinusoidal signal of the form

ct=Ac cos⁡(2πfct+∅)

The baseband (message) signal m(t) is referred to as the modulating signal and the result of

the modulation process is referred to as the modulated signal s(t) . The following block

diagram illustrates the modulation process.

Figure 1.1: Modulation Process Block

## 1.2 Normal Amplitude Modulation

A normal AM signal is defined as:

Ac(1+kam(t))cos⁡(2πfct)

Where ka : Amplitude sensitivity (units 1/volt)

KaAm : the modulation index u= Vmax-VminVmax+Vmin

fm: the frequency of m(t) and  fc: the frequency of c(t).

Ac:  the amplitude of the carrier

The envelope of the modulating wave S(t) has the same shape as the message signal provided the following two requirements are satisfied:

1. The carrier frequency fc must be much greater than the highest frequency components fm of the message signal m(t)  fc >> fm

2. The modulation index must be less than unity. if the modulation index is greater than unity, the carrier wave becomes over modulated.

Spectrum of the AM signal

Note that:

1. The spectrum of m(t) has been shifted to fc resulting in a bandpass signal.

2. Two sidebands (upper sideband and lower sideband ) and a carrier form the spectrum of s(t).

3. The transmission bandwidth of s(t) = 2w.

Power Efficiency in AM

The power efficiency of a normal AM signal is defined as:

n= power in the sidebandspower in the sidebands+power in the carrier

Where it is in AM = μ2μ2+1 which is in the best case is 1/3 when µ=1.So that 2/3 of the power goes to the carrier.

Demodulation of the AM signal (The Envelope Detector)

The ideal envelope detector is one, which responds to the envelope of the signal, but insensitive to the phase variation.

The circuit that represents the ideal envelope detector is as shown below:

Figure 1.2: Ideal Envelope Detector Circuit

Although there are some advantages of the Amplitude modulation; such as being able to get the original signal from the modulated one due to the symmetry that we have in s(t),  still this type of modulation has some disadvantages since it's power efficiency is low besides having twice the B.W of  m (t) “large bandwidth".

## 1.3 DSBsc Modulation

A DSB-SC signals is an AM signals that has the form:

Spectrum of the DSBsc signal

The spectrum of s(t) is:

Notice that:

1.  No impulses are present in the spectrum at ∓fc, hence, no carriers are transmitted.

2. The transmission bandwidth of s(t)= 2w

3. Power efficiency = power in the sidebandspower in the sidebands+power in the carrier = 100% since there is no power in the carrier; all power in the sides.

Demodulation of the DSBsc signal

A DSB-SC signal is demodulated using what is known as coherent demodulation. This means that the modulated signal s(t) is multiplied by a locally generated signal at the receiver which has the same frequency and phase as the carrier c(t) at transmitting side.

Figure 1.3: Demodulation Process Block in DSBSC

The high frequency component can be eliminated using the LPF. The output is:

Therefore, m(t) has been recovered from s(t) without distortion  distortion less system.

# 2. Procedure and Discussion

## 2.1 AM Modulation

The audio signal generator was set to sine wave of frequency =2kHz and amplitude (Vp-p= 2 volts).

After that we connected the following circuit:

Figure 2.1:AM Normal Modulation Circuit Connections

Then the toggle switch is set to carrier on(to generate AM signal), and the Cassy Lab 2 example Diagram 3.1-1 is loaded. After pressing F9 we get the following Output which represent the AM modulated signal:

Figure 2.2: Modulated and Demodulated Signal for AM Normal Modulation

From the previous result, we can see that the red output is the modulated signal of the AM and it's envelope is the message signal (sine wave of frequency =2kHz). We can also see that there is no distortion in the output and we got the original signal since the modulation index is less than 1.

When we set Am to 3v, we get the following output which represent the over modulation signal:

Figure 2.3: Over Modulation Case in AM

A better approach is to determine m from the modulation trapezoid. For this Sensor - CASSY 2 is operated in XY modus and the message signal SM(t) is used for horizontal deflection. The result obtained when Diagram 3.1-2 is loaded and after pressing F9 is

Figure 2.4: The modulation trapezoid(XY modus) for AM

This can be used to determine the value of m which equal to  D-dD+d  and this help to detect the distortion in the signal.

Where

D: Peak-to-peak value of the maximum of the AM signal

d: Peak-to-peak value of the minimum of the AM signal

So, We kept running the circuit until we get an AM signal with clear D and d values

Figure 2.5: The modulation trapezoid(XY modus) for AM

From the trapezoid we measured the value of D = 4.09 and d = 0. So the modulation index = 4.09-04.09+0= 1.

This value tells us that the system is distortion less and the message signal can be recovered correctly.

## 2.2 DSBSC Modulation

The audio signal generator was set to sine wave of frequency =2kHz and amplitude (Vp-p= 2 volts).

Then the toggle switch is set to carrier off, and the Cassy Lab 2 example Diagram 3.1-3 is loaded. After pressing F9 we get the following Output Which represent the DSBsc Signal:

Figure 2.6: Modulated and Demodulated Signal for DSBSC

The modulated signal is a multiplication result of two sinosoidal functions.

When we feed the modulating signal with low pass filter, we got the same result, this is because the cut of frequency for the filter is 3.4 kHz and the input frequency is 2 kHz. So all the input frequencies will pass throw the low pass filter without elimination.

## 2.3 Spectrum of AM Modulation

The audio signal generator was set to sine wave of frequency =2kHz and amplitude (Vp-p= 2 volts).

Then the toggle switch is set to carrier on, and the Cassy Lab 2 example Diagram 3.2-1 is loaded. After pressing F9 we get the following Output :

Figure 2.7: Spectrum of AM Normal Modulation at Fm 2kHz

We can see from the previous result that the AM spectrum is 20 kHz(the carrier frequency), 22 kHz(Fc+Fm) and 18 kHz(Fc-Fm). When changing Am to 1v and fm to 3kHz we got the following FFT result:

Figure 2.7: Spectrum of AM Normal Modulation at Fm 2kHz

The upper sideband frequency is 23 kHz and the lower sideband is 17 kHz. So we can say that the USL is at f=fc+fm and the LSL is at f=f-fm. The bandwidth is 2fm.

The modulation index can be calculated from the spectrum.

The modulation index= kaAc. From the spectrum we know that AmKaAc/2 = 0.5, then the modulation index=0.5 .

## 2.4 Spectrum of DSBSC Modulation

The audio signal generator was set to sine wave of frequency =2kHz and amplitude (Vp-p= 2 volts). Then the toggle switch is set to carrier off, and the Cassy Lab 2 example Diagram 3.2-3 is loaded. After pressing F9 we get the following Output:

Figure 2.8: Spectrum of DSBSC  with sinosoidal input 2kHz

The previous result shows the spectrum of the modulated signal when the message signal is a sinusoidal signal. Where the spectrum of the modulated signal is an upper sideband pulse at fc+fm=22kHz and a lower sideband pulse at fc-fm =18kHz. We can notice that there is no pulse at the carrier frequency(20 kHz) since it is suppressed.

Then the audio signal generator was set to square wave of frequency =2kHz and amplitude (Vp-p= 2 volts). After pressing F9 we get the following output:

Figure 2.8: Spectrum of DSBSC  with impulse input 2kHz

This figure represent the spectrum of the modulated signal when the message signal is a square wave signal. The frequency response for the square wave is sinc .So the spectrum of the modulated signal is pulses at all the peaks of the sinc function. But  when the LP filter is connected the deltas will disappear.

## 2.5 DSBsc Synchronous Demodulation

The audio signal generator was set to sine wave of frequency =2kHz and amplitude (Vp-p= 2 volts).

After that we connected the following circuit:

Figure 2.10: DSBsc Synchronous Demodulation Circuit Connections

Then the toggle switch is set to carrier on, and the Cassy Lab 2 example Diagram 3.3-1 is loaded. After pressing F9 we get the following output:

Figure 2.11: Message and Demodulation Signals Output

We notice that the recovered signal from the demodulation and the input signal almost the same. However, there is an attenuation happened. If the carrier frequency on the transmitter and the carrier frequency on the receiver are in phase , there will be no attenuation.

## 2.6 DSBsc synchronous demodulation with the aid of a PLL-Controlled VCO

we connected the following circuit:

Figure 2.12: DSBsc Synchronous Demodulation PLL Circuit Connections

Then the Cassy Lab 2 example Diagram 3.3-1 is loaded. After pressing F9 we get the following output:

Figure 2.13: Input Carrier and PLL carrier recovery

This figure shows the auxiliary carrier input and the recovered carrier which recovered using the PLL carrier recovery. We can notice that the input and the output frequency are the same= 20 kHz (period of 5 ms).

# 3. Questions

Q.1. What is meant by modulation? Mixing?

Modulation is the process of varying one or more properties of a periodic  waveform, called the carrier signal, with a modulating signal that typically contains information to be transmitted.

Mixing is an active or passive device that converts a signal from one frequency to another and it can either modulate or demodulate.

Q.2. Name the reasons for performing modulation!

1.  To reduce the noise & distortions and equipment

2.  To fully utilize a signal channel by means of multiplexing , which means sending more than one signal at a time .

3.  To reduce the height of the antenna , as it depends on the wavelength of the signal .

4. To narrow band the signal .

Q.3. In DSB the carrier's peak values are affected by the instantaneous value of the message signal, but the spectrum shows that the carrier amplitude remains constant! How do you explain the apparent contradiction?

Carrier is not sent, so the power efficiency will be 100%

Q.4. Define amplitude deviation and the modulation index.

Amplitude deviation: The change in Amplitude of the carrier

Modulation index: The ratio of the frequency deviation to the modulating frequency

Q.5. Which methods of AM demodulation are you familiar with and how do they differ?

Am normal modulation , SSBsc and DSBsc , they are differ by power and bandwidth . SSB(Single sideband modulation) ,the modulation wave consists only of the upper sideband or the lower sideband ,the essential function of SSB modulation is therefore to translate the spectrum of modulating signal to a new location in the frequency domain. Single-sideband modulation avoids this bandwidth doubling, and the power wasted on a carrier, at the cost of increased device complexity and more difficult tuning at the receiver. AM normal modulation consists of both the lower and upper side and the carrier, it has maximum power efficiency 1/3 so 2/3 of the power is wasted in the carrier, the bandwidth of the modulated signal is twice the message band width. In DSBsc the carrier is suppressed to have 100% power efficiency, but the bandwidth remains twice the message bandwidth. Then, SSBsc has the min bandwidth and power consumption , next is DSBsc and then Am modulation .

Q.6. How high is the maximum efficiency in DSB? How can the efficiency be increased?

A normal Am transmission (DSB) has a maximum efficiency of 33.33% , since 2/3 of the power is in the carrier, but in DSBsc the power is reduced as there’s no carrier , and the efficiency will be 100%. So removing the carrier will increase the efficiency.

# 4. Conclusion

At this experiment we learned two types of amplitude modulation; AM normal modulation and DSBsc modulation. In AM normal modulation, we saw the modulated and demodulated signals in time domain, the demodulation technique that is used in AM normal modulation is envelope detector which is a simple technique. In order to avoid the distortion or over modulation we have to keep the modulation index less than one. The maximum power efficiency in this modulating  technique is 33.3%. Also, we study its spectrum and the range of bandwidth that its produce which was 2wm.

In DSBsc modulation, we also performed the time domain for the modulated signal. However, this modulation uses a coherent demodulation that needs a  synchronous demodulation with the aid of a PLL controlled VCO. This demodulation technique is more complex than the envelope detector. We study its spectrum of DSBsc which has a 100% power efficiency since the carrier is suppressed and the range of bandwidth that its produce which was 2wm same as AM normal modulation.

At the end, we achieved the aim of the experiment. Although we faced some problem when we started the experiment because we didn't connect the jumpers well so we didn't get an output. We also face some difficulties to understand the output and match it with theoretical knowledge but at the end, we understood everything well.

# 5. References

[1]  Principles of Communication Systems; lecture note,  by Dr. Wa’el Hashlamoun.

[2] Introduction to Analog & Digital Communications, Second Edition Simon Haykin, Michael Moher.