Faculty of Engineering and Technology

Department of Electrical and Computer Engineering

ENEE4103 , COMMUNICATIONS LABORATORY, First Semester, 2016-2017

Experiment #2: Single Sideband AM (SSB) and Ring Modulator

Report__

Student Name : Sharehan Badwan

Student Id : 1130167

Instructor: Dr. Wael Hashlamoun

Teaching Assistant: Eng. Ahmad Faza’

Section: 3

Date : 19/10/2016

Content:

Contents

Abstract:	2

1.Theory	3

1.1Single Sideband Modulation	3

Generation of SSB:1.2	3

1.3 Spectrum of Single side band:	4

1.4 Transmission Bandwidth of SSB-SC	4

1.5 Ring Modulator:	4

2. Procedure and Discussion	6

Part One: SSB with Residual Carrier (SSBRC) Modulation	6

Part Two: SSB with Suppressed Carrier (SSBSC) Modulation	8

Part Three: Spectrum of the SSBRC	9

Part Four: Spectrum of the SSBSC	10

Part Five: SSBRC demodulation	10

Part Six: SSBRC demodulation	11

Part Seven: The Ring Modulator	12

Part Eight: Spectrum at the output of the ring modulator	14

3. Questions:	15

4.Conclusion:	16

5. References	17

# Abstract:

In this experiment, we have dealt with single side band (SSB) and Ring Modulator .we have  reviewed  the theoretical part of single side band modulation and demodulation  techniques.

# 1.Theory

## 1.1Single Sideband Modulation

In this type of modulation, only one of the two sidebands of DSB-SC is retained while the other sideband is suppressed. This mean that B.W of the SSB signal is one half that of DSB-SC. The saving in the bandwidth comes at the expense of increasing modulation complexity.

The time-domain representation of a SSB signal is

## Generation of SSB:1.2

Band pass filter with appropriate B.W and center frequency is used to pass the desired side band only.

## 1.3 Spectrum of Single side band:

## 1.4 Transmission Bandwidth of SSB-SC

Since we are transmitting the frequencies only in the range (fc+ fm) or ( fc – fm), the transmission bandwidth for the SSB-SC will be :

Bandwidth B = (fc+fm ) – fm= fm Hz

Or                 B= fc –  ( fc – fm) =fm Hz

This is exactly half the bandwidth of the DSB-SC modulated waves .

## 1.5 Ring Modulator:

.

Let c(t)>> m(t). Here the carrier c(t) control the behavior of the diodes .

During the positive half cycle of c(t), c(t) > 0, and D1 and D2 are ON while D3 and D4 are OFF.

y(t) = m(t)

During the negative half cycle of  c(t), c(t) < 0 and D3 and D4 are ON while and D1 and D2 are OFF

y(t) = -m(t)

So m(t) is multiplied by +1 during the +ve half cycle of c(t) and m(t) multiplied by -1 during the -ve half cycle of c(t). Mathematically, y(t) behaves as if multiplied by the switching function gp(t) where  gp(t) is the square periodic function with period Tc=   , where fc the period of c(t). By expanding gp(t) in a Fourier series, we get

# 2. Procedure and Discussion

## Part One: SSB with Residual Carrier (SSBRC) Modulation

The function generator was set to sinusoidal of frequency 2KHz and Amplitude of 2V.

Figure.1 SSB with Residual Carrier (SSBRC) Modulation Connection

The toggle switch was set to carrier on. The output signal of the channel filter CH2 and the modulating signal sM(t) was displayed of the function generator on Sensor - CASSY 2 - Starter and the signals was sketched.

Figure.2 Modulated and Demodulated Signal for SSBRC Modulation

CASSY Lab 2 example was loaded Diagram 4.1-1.labx.The measurement was started by pressing F9.

The AF signal was shift to the upper envelope curve of the Am signal and the fm frequency varied and the amplitude Am of the modulating signal were varied.

We were observed that the envelope is not the message signal.

The un-attenuated carrier amplitude AC at the input of the channel filter was measured.

Figure.3  Fc value before band pass filter

The amplitude ARC of the attenuated carrier at the output of the channel filter was measured.

Figure.4 Fc after band pass filter

The ratio k = ARC/AC was calculated.

K=0.46/2.17

=0.21

The carrier suppression t in dB was determined .

m=Am/Ac

=2/2.17

=0.92

=  20log(0.92/2*0.21)

=6.8 db

m(modulation index)= 0.92 < 1 there is no over modulation (NO DISTORTION).

## Part Two: SSB with Suppressed Carrier (SSBSC) Modulation

The function generator was set to sinusoidal of frequency 2KHz and Amplitude of 2V.

Figure.5 SSB with Suppressed Carrier (SSBSC) Modulation Connection

The toggle switch was set to carrier OFF. The CASSY Lab 2 example Diagram 4.1-2.labx was loaded. The measurement was started by pressing F9.

Figure.5 SSB with Suppressed Carrier (SSBSC) Modulation

## Part Three: Spectrum of the SSBRC

The experiment setup was kept as in part one, the toggle switch was set to the CARRIER ON position.

The modulating signal was fed  into the input LP filter of the CF transmitter.

Figure.6  Spectrum of the SSBRC with carrier on

The SSB spectrum in the range of. 15 kHz up to 25 kHz , we can see the lower side band at frequency Fc-Fm (20-2) = 18KHz but the upper side band was filtered.

The transmission bandwidth of the AM signal based on the measurements = Fm(2KHZ)

## Part Four: Spectrum of the SSBSC

The experiment setup was kept as in part one, the toggle switch was set to the CARRIER OFF position.

Figure.7 spectrum of SSBSC with carrier off

Carrier OFF  and the transmission band width = Fm (2KHz)

## Part Five: SSBRC demodulation

The experiment was set up  as specified below in figure 8 . The settings of the Sensor-CASSY 2 was started. The CASSY Lab 2 example Diagram 4.1-1.labx was loaded. The function generator was set  to: sinusoidal, AM = 2 V and fM = 2 kHz.

Figure.8 SSBRC demodulation

The CASSY Lab 2 example Diagram 4.3-1.labx was loaded

The measurement was started  by pressing F9.

The phase between the original carrier of the transmitter and the auxiliary carrier of the receiver was adjusted.

We observed that there is no change(modulation and demodulation used the same carrier )

Figure.9 SSBRC demodulation and phase adjusted between original carrier of the transmitter and the auxiliary carrier of the receiver.

SSB RC and SSB SC have the same bandwidth which is equal Fm,but the SSB SC have a power efficiency more than SSB RC.

## Part Six: SSBRC demodulation

The connecting lead between the CF transmitter and the demodulator was removed.

For the demodulation the recovered auxiliary carrier was used  from the PLL circuit to recover the carrier.

For this , the bridging plug between CARRIER RECOVERY and the auxiliary carrier input of D2 was connected.

Figure.10 SSBRC demodulation

The toggle switch was set  to CARRIER OFF. The CASSY Lab 2 example Diagram 4.3-2.labx was loaded.  The measurement was started  by pressing F9.

Figure.11 SSB_RC demodulation

There was a phase shift in the output since we had two different carriers .

Part Seven: The Ring Modulator
 The experiment was set up as specified in the following figure.

Figure.12 ring modulator connection

Carrier OFF*

The modulating AF signal and the output signal of the modulator was displayed on Sensor CASSY 2 - Starter.

The signal was sketched  in Diagram 5.1-1.

The toggle switch was set  to CARRIER OFF.

The bridging plug was used  to feed the square-wave carrier into the RF input of the modulator.

**We can use the square wave carrier to get a high order of harmonics .

A sinusoidal signal was fed with fM = 2 kHz and AM = 2 V  as the modulating signal into the AF input of the modulator.

The CASSY Lab 2 example Diagram 5.1-1.labx was  loaded.

The measurement was started by pressing F9

Figure.12 ring modulator carrier off

Carrier ON*

The experiment was repeated with CARRIER ON.

The  CASSY Lab 2 example Diagram 5.1-2.labx was loaded

The measurement was started by pressing F9.

Figure.13 ring modulator carrier on

## Part Eight: Spectrum at the output of the ring modulator

The toggle switch was set to CARRIER OFF

The CASSY Lab 2 example Diagram 5.1-3.labxwas loaded

The measurement was started by pressing F9.

Figure.14 Spectrum at the output of the ring modulator with carrier off

Figure.15 Spectrum at the output of the ring modulator with carrier on

We have noticed that we have sinc  harmonics on multiple odd numbers of  Fc .

# 3. Questions:

Q.1. In radio links the carrier is normally attenuated to 5%...10%. What advantages does this have compared to transmission with 100% carrier amplitude ? why isn’t the carrier completely suppressed ?

Advantages of attenuation  the carrier amplitude to 5% .. 10% , so that power in the side band will not lost. ( we don’t want to send a huge power in carrier , because carrier don’t have any information).

We used carrier (pilot) to recover the signal in the receiver side (demodulation operation)

Q.2. Which methods of carrier suppression are there?

Using pilot method

The carrier wasn’t completely suppressed in order to use it in the demodulation with carrier recovery circuit(PLL).

Q.3 which demodulation method is used for AM with suppressed carrier?

Coherent demodulation .

# 4.Conclusion:

in this experiment we learned about SSB – RC , SSB-SC and ring modulation , we saw the modulated and the demodulated signal in the time domain and frequency domain , and notice the effect of the carrier,  and use the band bass filter to get a SSB signal (lower or higher ) from DSB,  and we recognize the difference between the sinusoidal carrier and square carrier , and how we can use the square wave to get a sinc harmonic and high order of fundamental carrier frequency.

At the end, we achieved the aim of the experiment. Although we faced some problem in the experiment because we turn off  the  kit so we didn't get an output. We also face some mistakes when we connect the circuit.

# 5. References

[1] http://electronicspost.com/single-sideband-suppressed-carrier-ssb-sc-modulation/

https://en.wikipedia.org/wiki/Ring_modulation[2]

[3]  Principles of Communication Systems; lecture note,  by Dr. Wa’el Hashlamoun.