Electrical and Computer Systems Engineering Department

Communications Lab

ENEE 4103

PreLab for EXP. No. 3

Angle Modulation - Part one

Student Name: Aseel Awad           Student ID: 1130776

Instructor: Wael Hashlamoun        Section.No. 3

Assistant:Eng. Ahmad Faza           Date: 18/10/2016

A. Find the message signal:

In FM modulation, S(t)is:

Since we have S(t)= cos(2π17t+4sin(2πt)

2πKf∫m(t)=4sin(2πt)

M(t) = 4Kfcos(2πt)

B. Plotting S(t) -1<t<1:

C. Differentiate s(t) with respect to t and plot ds(t)/dt for -1 ≤ t ≤ 1. Notice how this operation transforms an FM waveform into an AM waveform.

The differentiation of s(t) is: (2*17*pi+8*pi.*cos(2*pi.*t)).*(sin(2*pi*17.*t+4*sin(2*pi.*t))).

D. Apply ds(t)/dt to an ideal envelope detector, subtract the dc term and show that the detector’s output is linearly proportional to m(t).

If we apply the derivative to an envelope detector. The output will be:
St_Diff_ED=abs(2*pi*17+8*pi*Cos(2*pi*t)) then after the envelop detector we apply the signal to DC blocking capacitor. So the output will be 8*pi*Cos(2*pi*t).

The envelope with DC shift

Y(t)= abs(2*pi*17+8*pi*Cos(2*pi*t))

The envelope output after subtract the dc term

Y(t)= abs(8*pi*Cos(2*pi*t)).