ENEE4103 : communication Lab

Sara sameeh #1130501

Dr. wael Hashlamoun

Teacher assistant: Ahmad faza’

Report for experiment 3&4

Angle Modulation

Section #3

Table of Contents

Abstract....................................................................................................................... 3

Theory ....................................................................................................................... 4

Procedure and Discussion ........................................................................................... 9

2.1. Angle Modulation – Part 1

Part One: Dynamic Response of FM .................................................................................... 9 Part Two: The Characteristic of FM Modulator .................................................................... 10 Part Three: Determination of the Frequency Deviation ....................................................... 11 Part Four: Measurements in the Frequency Domain of FM .................................................. 11 Part Five: Determining the Carrier Zero Crossings by Varying the Frequency Deviation ………13         Part Six: Determine the carrier zero crossings by varying the modulated frequency 𝐟𝐦. ...... 15 Part Seven: FM Spectrum for Square Wave Modulation. ..................................................... 16   2.2. Angle Modulation – Part 2

Part One: FM demodulation for loop filter T2 .................................................................. 17 Part Two: FM demodulation for loop filter T1 ................................................................... 19 Part Three: FM pre-emphasis ........................................................................................... 20 Part Four: Dynamic response of phase modulation (PM): .................................................. 23 Part Five: Characteristic of the PM modulator . ................................................................. 24 Part Six:. The PM spectrum .............................................................................................. 25     Part Seven: PM demodulation for loop filter T2……………………………………....26

Part Eight:. PM demodulation for loop filter T1  ............................................................... 27

Questions ................................................................................................................. 28

Conclusion ............................................................................................................... 30

References ............................................................................................................... 31

1.Abstract:

in this experiment, we will introduce to Angle modulation with its two types, we will generate and plot FM modulated signal (with different type of input) for example sinusoidal and square waves, we will learn how to generate FM signal & how to demodulate it, in addition to that, we will learn about PM modulation, what is the advantages to use angler modulation instead of AM modulation.

2. Theory:

Angle modulation: it’s a technique that based on altering the phase (angle) of a carrier signal to transmit data, the angle of the sine-wave carrier is varied by a modulating wave, there are two types of angle modulation, FM modulation & PM modulation.

2.1 .1 Frequency modulation: encoding the data in a carrier wave changing the instantaneous frequency of the wave.

In FM radio broadcasting of an audio signal representing voice or music , the instantaneous frequency deviation , the difference between the frequency of the frequency of the carrier and its center frequency is proportional to the modulating signal, we used FM for radio broadcasting as we said before , als in telemetry , radar, and seismic prospecting.

The instantaneous frequency is given by this relation:

The frequency modulation is given by this relation:

In general :

2.1.2Modulation index β:

Depends on β , there is two type of FM , NBFM & WBFM.

1. NBFM : narrowband FM , when  β << 1

1. WBFM : narrowband FM , when  β >> 1

Note: fm is the highest frequency component present in the modulating signal m ( t ).

2.1.3 Carson’s Rule: Carson's bandwidth rule defines the approximate bandwidth requirements of communications system components for a carrier signal that is frequency modulated by a continuous or broad spectrum of frequencies rather than a single frequency.

2.1.4 Generation of FM signal:

Two way to generate FM signals:

- Direct: by directly feeding the message into the input of a VCO.

- indirect  : the message signal is integrated to generate a phase-modulated signal. This is used to modulate a crystal-controlled oscillator, and the result is passed through a frequency multiplier to give an FM signal.

2.1.5 demodulation of FM signal : there is many way to demodulate the FM signal such as the following ways:

- slope FM detector

- Ratio detector

- Foster-Seeley FM detector

- Quadrature FM demodulator

- Coincidence FM demodulator

- PLL, Phase locked loop FM demodulator:   FM demodulators using phase locked loops, PLLs cna provide high levels of performance. They do not require a costly transformer and can easily be incorporated within FM radio ICs. Read more about the PLL FM Detector

### 2.1.6 Bessel functions

For the case of a carrier modulated by a single sine wave, the resulting frequency spectrum can be calculated using Bessel functions of the first kind, as a function of the sideband number and the modulation index. The carrier and sideband amplitudes are illustrated for different modulation indices of FM signals. For particular values of the modulation index, the carrier amplitude becomes zero and all the signal power is in the sidebands.[4]

Note: there is 98% of the power lies in the band width.

2.2.1 PM modulation: Phase modulation a modulation pattern that encodes information as variations in the instantaneous phase of a carrier wave.PM changes the phase angle of the complex envelope in direct proportion to the message signal.

Suppose that the signal to be sent (called the modulating or message signal) is m(t){\displaystyle m(t)} and the carrier  is:

Modulated signal is:

2.2.2 Modulation index for PM signal:

this quantity indicates by how much the modulated variable varies around its unmodulated level. It relates to the variations in the phase of the carrier signal.

Note that bandwidth of PM signal is:

Where h is modulation index .

3.Procedure:

3.1 Angle modulation part 1: FM

3.1 .1 Part One: Dynamic response of FM

Figure1 FM Modulation

We connected as shown in the figure1 , we set the carrier frequency to 20KHZ , we feed the fm modulator with a sinusoidal signal with following characteristic( Amplitude = 20 Vpp and frequency = 1KHZ, then we load the cassy lab  Lab2 example FM_TD.labx , and we get the following result :

Figure2 FM modulated signal in time domain

3.1.2 Part Two: The Characteristic of the FM Modulator

We use the same connection in previouspart, then we load the FM.FFT.labx example in cassy lab, and we get the following:

Figure3 FM modulation signal in frequency domain for 1KHZ

Notice that the frequency is changing between fc+nfm and fc-nfm (21 and 19 for example)

We changed the input signal (modulated signal) from sinusoidal to dc signal with value -10 volt, the result was the following:

Figure4 FM modulated signal in frequency domain with dc signal as input

Note : fc = 1.055 as shown in figure4.

Then we changed the value of the dc between (-10 t0 10) and measured the frequency of the modulated signal, by using MATLAB, we get the following:

Figure 5 relation between VCO and carrier frequency

By computing the slope, we can get the coefficient of FM modulator (Kfm)

Slope = (20.758-19.373)/(10--10) = 0.06925khz = 69.2HZ.

So kfm = 69.2

Where KFM =fVCO /U1, which indicate to the minimum change in an input voltage that produces a change in the output frequency.

3.1.3 Part Three: Determination of the frequency deviation

The maximum frequency deviation depends of the coefficient of the FM modulator and the amplitude of the modulating signal in the following relation:

Kfm from previous part = 69.2 -> frequency deviation = 69.2 * 20/2 = 692

3.1.4 Part Four: Measurements in the frequency domain of FM

We keep the connects like part one, Then we removed the bridging plug from the function generator to the FM modulator, then we load the CASSY Lab 2 example FM_FFT.lab, we set the carrier frequency to 20khz , then we reconnect the function generator to the FM modulator with the bridging plug again, then we feed the modulating signal (sinusoidal signal with Am = 20 vpp & fm = 300hz ) into the input of the FM modulator

The result was the following:

Figure 6 FFT (frequency domain) for 300hz

Then we keep the connects the same, and change the frequency from 300hz to 200hz , and the result was the following:

Figure7 FFT(frequency domain) for 200hz

In the previous two figures, we notice that frequency ranges between (fc +nfm) & (fc-nfm)    also there is many small pulses with small amplitudes, that’s because of the Bessel function which has the following relation:

3.1.5 Part Five: Determining the carrier zero crossings by varying the frequency deviation ΔFFM.

First of all , we feed the FM modulator with a sinusoidal signal from the generator with( am = 0, fm =100hz) , then we load  FM_FFT.labx on cassy lab, and start the measurement, the result was :

Figure8 FFT _ when dc= -10

Then we start to increasing the amplitude of the input signal , and notice the decaying of the carrier , when carrier disappeared we measure the amplitude value .

The carrier disappeared when:

Am = 7

Figure 9 carrier disappeared when am=7

When am = 15.9

Figure 10 carrier disappeared when am=15.9

Then using this relation, we measure the modulation index.

When am =7 -> Modulation index = 3.5*69.2/100 = 2.422

When am = 15.9 ->Modulation index = 7.95*69.2/100 = 5.50

3.1.6 Part Six: Determine the carrier zero crossings by varying the modulating frequency fM.

In the previous part, we keep the frequency fixed, and change the amplitude, in this part, we will change the frequency, and keep the amplitude fixed.

First of all , we feed the sinusoidal signal with am=10v and fm = 3khz to the FM modulator, then we load FM_FFT.labx , then we start to decreasing the frequency slowly, and notice when the carrier disappear.

The carrier disappeared when:

Fm= 124.7

Figure 11 carrier disappears when fm=124.7hz

Fm= 287hz

Figure 12 carrier disappears when fm=287hz

Using these frequencies to calculating modulation index using following formula:

When fm = 124.7 -> 69.2*10/124.7=5.54

When fm = 287 -> 69.2*10/287=2.41

We notice that these modulation index is nearly the same one in the previous part, which means that we have two way to calculate modulation index:

1.  Fixing amplitude and keep decreasing frequency (of message) until carrier disappear

1.  Fixing frequency and keep increasing amplitude (of message) until carrier disappear

3.1.7 Part Seven: FM spectrum for square wave modulation

In this part we feed the FM modulator with a square wave signal with am=20 vpp , duty cycle 50% and fm=200hz , then we load FM_FFT.labx, then we start measure and the result was :

Figure13 FM modulated signal with square wave input

3.2 Angle Modulation – Part 2

3.2.1 Part One: FM demodulation for loop filter τ2

FM demodulation for loop filter τ2

3.2.1.1Time response of the FM modulating system

In this part , we connected as shown in the above figure, then we set the loop filter of the FM modulator to  t2 , the we feed a sinusoidal signal with am = 10vpp and fm=500 to the FM modulator ,we load FM_TDEM.labx on cassy lab,& finaly we measure the input and the output of the FM modulator, the result was:

Figure14 m(t) & s(t) in time domain

Note that the red is the message signal , and the black one is the output s(t)

3.2.1.2 Transfer characteristic of the FM modulating system

In this part , we use a sinusoidal modulating signal with am=2vpp and fm = 500hz,then

we load FM_FTT_DEM.labx on CASSY lab , we start the measure , and the result was the

following:

Figure15 Transfer characteristic of the FM modulating signal at f= 500HZ

notice from the figure that ad = 1.45

then we start increasing the frequency from 500hz to 5000hz and observe the different values of ad , & by using MATLAB , we draw the relation as following:

Figure16 frequency vs AD relation

This relation represent low pass filter.

3.2.2 Part Two: FM demodulation for loop filter τ1

We keep the connect as in the previous part , the only difference that we change the loop filter of the FM modulator to t1, then we start measure, the result was the following:

Figure17 m(t) & s(t) in time domain

Notice that in this part , the original signal and the output signal are different , It’s shown form the above figure that signal is chopped, because the voltage of the signal is higher than the voltage that is supplied by the VCO.

Then we start to change the frequency from 500Hz to 5000Hz and observing the change in AD as following.

Figure17 frequency vs AD relation

Notice that the above figure represent High pass filter

3.2.3 Part Three: FM preemphasis

3.2.3.1 Time characteristic of the input signals

In this part we connected as shown in the following figure:

the loop filter of the FM demodulator was sited to τ2. •the function generator was set to a sinusoidal modulating signal sM(t) with AM = 5 Vpp and initially fM = 500 Hz.

The input of the pre-emphasis stage was displayed on channel A of the Sensor-CASSY2, and the CASSY Lab 2 example FM_preDemm.labx was loaded, the result was the following:

Figure 18 Time characteristic of the input signals , at f= 500Hz

The blue one when f= 500hz , the red one when f=1000, the black one when f=2000.

Then , The measurements were repeated sequentially for another two values of the frequency of the modulating signal: 1000 Hz and 2000 Hz., the result was the following:

Figure 19 Time characteristic of the input signals, at f= 1000Hz & f =2000Hz

3.2.3.2 Time characteristic of the output signals

In this part , we repeated the previous part , but we changed the modulating signal (AM=5vpp and fm = 500hz), the loop filter of the FM modulator was set to t2, the FM_PreDem.labx was loaded , then the  MEASUER WAS STARTED :

Figure 20 Time characteristic of the output signals, at f= 500Hz

The blue one when f= 500hz , the red one when f=1000, the black one when f=2000.

Then , The measurements were repeated sequentially for another two values of the frequency of the modulating signal: 1000 Hz and 2000 Hz., the result was the following:

Figure 21 Time characteristic of the output signals, at f= 1000Hz & f =2000Hz

3.2.4 Part Four: Dynamic response of Phase Modulation (PM)

In this part , we connected as shown in the above figure , the carrier frequency was set to 20khz , the PM modulator was fed by a modulating signal from function generator with (am=2pp and fm=1000hz), the PM_TDscope.labx example was loaded on CASSY lab, the result was :

Figure 22 m(t) and PM modulated signal

3.2.5 Part Five: Characteristic of the PM modulator

The carrier frequency set to 20khz, the function generator was set to DC , the PM_TD.labx example was loaded , after the measure the result was :

Figure 23 m(t) and shifted signal(output)at dc=1

In the above figure , the dc = 1 , we used vertical line and difference line as shown, delta t  ->

Then we calculated the phase shift using rule :

2*pi*fc*delta t = 1.256

Then we change the dc value to -1 :

Figure 24 m(t) and shifted signal(output)at dc=-1

Delta t ->

2*pi*fc*delta t = 0.628

3.2.6 Part Six: The PM Spectrum:

In this part , the PM_FFT.labx was loaded , the carrier frequency was set to 20kHz, the Pm modulator was fed by a sinusoidal signal from function generator with am=2 and fm = 300hz , the result was :

Figure 25 PM spectrum at f= 300Hz

The previous part was repeated , and the fm was changed to 200hz , the result was:

Figure 26 PM spectrum at f=200Hz

3.2.7 Part Seven: PM demodulation for loop filter τ2

We connected as shown in the above figure , the toggle switch of the PM demodulator  was set to t2,the PM_TDem.labx was loaded on CASSY lab2, after the measuer the result was :

Figure 27 PM demodulation

Part Eight: PM demodulation for loop filter τ1

The previous part was repeated, the only difference that we changed the toggle switch of the PM demodulater to t1, the result:

Figure 28 PM demodulation

4. Questions

Q.1. which carrier parameters can be used for angle modulation?

In FM modulation we use the frequency of the carrier, meanwhile in PM modulation, we use the phase of the carrier.

Q.2. which angle modulation method is least affected by noise? Why?

As for the relation between FM and PM the FM modulation is better no noise immunity as its bandwidth takes a big role in this so Noise is better suppressed in FM systems as compared to PM Systems, because the information of the message signal is transmitted by the variations of the instantaneous frequency of a sinusoidal carrier wave, and its amplitude is maintained constant. Therefore, any variations of the carrier amplitude at the receiver input must result from noise or inference, FM receivers normally have limiters that remove AM noise, further improving SNR. FM with wider deviation can achieve even greater improvements.

Q.3. in FM with η = 1.0 how high are the amplitudes of the 1st sideline compared to AM for m = 100%? Given that the amplitude of the unmodulated carrier is AC = 1 V.

Using Bessel function  ->

According to Bessel function for n=1 amplitudes of the 1st sideline=0.44 , meanwhile the amplitude in am = 1 .

Q.4. What does pre-emphasis mean?

Used in sender side , to decrease the noise , In processing electronic audio signals, pre-emphasis refers to a system process designed to increase (within a frequency band) the magnitude of some (usually higher) frequencies with respect to the magnitude of other (usually lower) frequencies in order to improve the overall signal-to-noise ratio by minimizing the adverse effects of such phenomena as attenuation distortion or saturation of recording media in subsequent parts of the system. The mirror operation is called de-emphasis, and the system as a whole is called emphasis.

Q.5. When is de-emphasis used in FM demodulation? How does it function?

De-emphasis is performed at the receiver side after pre-emphasis has been used at the sender side. It de-emphasizes the high-frequency components, so as to restore the original power distribution of the message. In such a process, the high-frequency components of the noise at the discriminator output are also reduced, thereby effectively increasing the signal-to-noise ratio of the system.

Q.6 Assuming in FM that the spectrum for a harmonic signal A1 = 5 V and fM1 = 1000 Hz is known. Can conclusions be drawn regarding the spectrum, which is produced for a different harmonic modulation signal, with, for example A1 = 3 V and fM1 = 100 Hz? Compare this with AM!

No , the reason for that is AM modulation is linear , not like FM modulation which is hard and its spectrum depend on Bessel function.

5. conclusion:

In this experiment , we learnt many new things , angle modulation in two types FM modulation and PM modulation , how to modulate and demodulate these signal , and how to find the modulation index in the two cases , in FM and PM , we notice what is the difference between AM modulation and FM modulation , FM is nonlinear modulation like AM , also there is a big advantage of using FM instead of AM , the power efficiency is better in FM than AM , we understand how the phase locked loop work to return the original message .

6. References

[1]Principles of Communication Systems; lecture note,  by Dr. Wa’el Hashlamoun.

[2]https://en.wikipedia.org/wiki/Emphasis_(telecommunications)#Pre-emphasis

[3]https://en.wikipedia.org/wiki/Carson_bandwidth_rule

[4]https://en.wikipedia.org/wiki/Frequency_modulation#Modulation

[5]http://www.radio-electronics.com/info/rf-technology-design/fm-reception/fm- demodulation-detection-overview.php

[7]http://www.sengpielaudio.com/calculator-timedelayphase.htm