# Abstract

The aim of this experiment is to understand the concept of Pulse Amplitude Modulation (PAM) which is a sampling process to maintain continuous amplitude representation of the message signal. Then, studying the effect of duty cycle and sampling frequency on the spectrum of a PAM signal that reach us to understand Nyquist theory. Finally, to study how we can multiplex (combine) data from several digital bit streams into one high-speed digital stream for transmission over a digital system Time- division Multiplexing.

Table of Contents

Abstract	1

1. Introduction	3

1.1 Pulse Train	3

1.2 Pulse Amplitude Modulation	3

1.3 Sampling, Nyquist Theory and Aliasing	4

1.4 Duty Cycle	6

1.5 Spectrum of PAM waveform	6

1.6 Sampling Generation; Sample and Hold Circuit	7

1.7 Time Division Multiplexing	7

2. Procedure and Discussion	8

Part One: Time and Frequency Characteristics of Pulse Train	8

Part two: Time Characteristics of Pulse Amplitude Modulation (PAM)	14

Part Three: Spectra of Pulse amplitude modulation (PAM)	18

Part Four: Subsampling in the Frequency Domain	21

Part Five: Subsampling in the Time Domain	21

Part Six: PAM time multiplex	22

3. Conclusion	25

4. References	26

# 1. Introduction

Modulation is the process of varying one or more properties of a periodic waveform, called the carrier signal, with a modulating signal that typically contains information to be transmitted [1]. In this experiment we will introduce the concept; Pulse Train and we will talk about another modulation technique called: The Pulse Amplitude modulator (PAM).

## 1.1 Pulse Train

A pulse wave or pulse train is a kind of non-sinusoidal waveform that is similar to a square wave, but does not have the symmetrical shape associated with a perfect square wave. It is a term common to synthesizer programming, and is a typical waveform available on many synthesizers. The exact shape of the wave is determined by the duty cycle of the oscillator. In many synthesizers, the duty cycle can be modulated (sometimes called pulse-width modulation) for a more dynamic timbre. The pulse wave is also known as the rectangular wave, the periodic version of the rectangular function. [2]

Figure 1.1: Pulse Train

## 1.2 Pulse Amplitude Modulation

In pulse amplitude modulation, the amplitude of regular interval of periodic pulses or electromagnetic pulses is varied in proposition to the sample of modulating signal or message signal. This is an analog type of modulation. In the pulse amplitude modulation, the message signal is sampled at regular periodic or time intervals and this each sample is made proportional to the magnitude of the message signal. These sample pulses can be transmitted directly using wired media or we can use a carrier signal for transmitting through wireless. There are two types of sampling techniques for transmitting messages using pulse amplitude modulation, they are

- FLAT TOP PAM: The amplitude of each pulse is directly proportional to instantaneous modulating signal amplitude at the time of pulse occurrence and then keeps the amplitude of the pulse for the rest of the half cycle.

- Natural PAM: The amplitude of each pulse is directly proportional to the instantaneous modulating signal amplitude at the time of pulse occurrence and then follows the amplitude of the modulating signal for the rest of the half cycle.

Flat top PAM is the best for transmission because we can easily remove the noise and we can also easily recognize the noise. When we compare the difference between the flat top PAM and natural PAM, flat top PAM principle of sampling uses sample and hold circuit. In natural principle of sampling, noise interference is minimum. But in flat top PAM noise interference maximum. Flat top PAM and natural PAM are practical and sampling rate satisfies the sampling criteria. [3]

Figure 1.2: Pulse Amplitude Modulation Technique

## 1.3 Sampling, Nyquist Theory and Aliasing

Sampling is a process of converting a signal (for example, a function of continuous time and/or space) into a numeric sequence (a function of discrete time and/or space). Shannon's version of the theorem states:

If a function x(t) contains no frequencies higher than B hertz, it is completely determined by giving its ordinates at a series of points spaced 1/(2B) seconds apart.

A sufficient sample-rate is therefore 2B samples/second, or anything larger. Equivalently, for a given sample rate fs, perfect reconstruction is guaranteed possible for a bandlimit B < fs/2.

When the bandlimit is too high (or there is no bandlimit), the reconstruction exhibits imperfections known as aliasing. Aliasing is an effect that causes different signals to become indistinguishable from each other during sampling. Aliasing is characterized by the altering of output compared to the original signal because resampling or interpolation resulted in a lower resolution in images, a slower frame rate in terms of video or a lower wave resolution in audio. Anti- aliasing filters can be used to correct this problem.

Modern statements of the theorem are sometimes careful to explicitly state that x(t) must contain no sinusoidal component at exactly frequency B, or that B must be strictly less than ½ the sample rate. The two thresholds, 2B and fs/2 are respectively called the Nyquist rate and Nyquist frequency, then  at the Nyquist rate the original waveform can be reconstructed by passing the sampled values through a low pass filter which smoothens out or interpolates the signal between sampled values.

Sampling is therefore a process of multiplying a constant amplitude impulse train with the input signal. It is an amplitude modulation process, where the pulse train acts as the carrier. Since the amplitude of the pulses is modulated [4]

Figure 1.3: PAM Modulation and Demodulation

Figure 1.4: Aliasing Effect When Fs<2B

## 1.4 Duty Cycle

A duty cycle is the percentage of one period in which a signal is active. A period is the time it takes for a signal to complete an on-and-off cycle. As a formula, a duty cycle may be expressed as:

Where   is the duty cycle,   is the time the signal is active, and   is the total period of the signal. Thus, a 60% duty cycle means the signal is on 60% of the time but off 40% of the time. The "on time" for a 60% duty cycle could be a fraction of a second, a day, or even a week, depending on the length of the period.[5]

Figure 1.5: Pulse Train at 0%, 20% and 70% Duty Cycle

## 1.5 Spectrum of PAM waveform

The spectrum for a naturally sampled PAM signal is:

Where W ( f ) = F[w(t)] is the spectrum of the original unsampled waveform.

The spectrum of PAM signal with natural sampling is a function of the spectrum of the analog input waveform. [7]

Figure 1.7: Spectrum of PAM Signal

## 1.6 Sampling Generation; Sample and Hold Circuit

In electronics, a sample and hold (S/H) circuit is an analog device that samples (captures, grabs) the voltage of a continuously varying analog signal and holds (locks, freezes) its value at a constant level for a specified minimum period of time. Sample and hold circuits and related peak detectors are the elementary analog memory devices. They are typically used in analog-to-digital converters to eliminate variations in input signal that can corrupt the conversion process.

A typical sample and hold circuit stores electric charge in a capacitor and contains at least one FET (field effect transistor) switch and at least one operational amplifier.[1] To sample the input signal the switch connects the capacitor to the output of a buffer amplifier. The buffer amplifier charges or discharges the capacitor so that the voltage across the capacitor is practically equal, or proportional to, input voltage. In hold mode the switch disconnects the capacitor from the buffer. The capacitor is invariably discharged by its own leakage currents and useful load currents, which makes the circuit inherently volatile, but the loss of voltage (voltage drop) within a specified hold time remains within an acceptable error margin. [8]

Figure 1.8: Sample and Hold Circuit

## 1.7 Time Division Multiplexing

Time-division multiplexing (TDM) is a method of putting multiple data streams in a single signal by separating the signal into many segments, each having a very short duration. Each individual data stream is reassembled at the receiving end based on the timing.

The circuit that combines signals at the source (transmitting) end of a communications link is known as a multiplexer. It accepts the input from each individual end user, breaks each signal into segments, and assigns the segments to the composite signal in a rotating, repeating sequence. The composite signal thus contains data from multiple senders. At the other end of the long-distance cable, the individual signals are separated out by means of a circuit called a demultiplexer, and routed to the proper end users. A two-way communications circuit requires a multiplexer/demultiplexer at each end of the long-distance, high-bandwidth cable.[9]

Figure 1.8: Time Division Multiplexing (TDM)

# 2. Procedure and Discussion

## Part One: Time and Frequency Characteristics of Pulse Train

At this part we used the function generator to select a pulse train input with fp = 1kHz and Ap = 5 V and duty cycle 10%, then we loaded the CASSY Lab 2 example PulseTime.labx to study the time characteristics of pulse train and we got the following result:

Figure 2.1.1: Time Domain for Pulse Train 10% Duty cycle

We can see from the previous that the period of the signal is 1 ms and the signal is high until 0.1 ms and low for the rest of the period, that indicates the meaning of  10% duty cycle.

Then, we loaded the CASSY Lab 2 example PulseFFT.labx to study the frequency characteristics of the pulse train, and we got the following result:

Figure 2.1.2: FFT for Pulse Train 10% Duty cycle

We can see from the previous result that the spectrum of a pulse train is a sinc function with zero crossing at the N multiples of fp/duty cycle, which is in our case 10 KHz. So we can notice the zeros at 10K,20K and so on the spectrum. We can also see that there is 9 spectrums between the zero crossings.

Then we repeated the measurement of the spectra and time characteristics for the same pulse frequency fP = 1 kHz and pulse amplitude AP for different duty cycles

- 20% duty cycle

Time Domain:

Figure 2.1.3: Time Domain for Pulse Train 20% Duty cycle

Frequency Spectrum:

Figure 2.1.4: FFT for Pulse Train 20% Duty cycle

zero crossing at N multiples of fp/0.2 = 5 KHz, so we can see these crossings at 5k,10k,15k,..

- 30% duty cycle

Time Domain:

Figure 2.1.5: Time Domain for Pulse Train 30% Duty cycle

Frequency Spectrum:

Figure 2.1.6: FFT Pulse Train 30% Duty cycle

zero crossing at N multiples of fp/0.3 = 3.3333KHz, so the zero crossings are at  3.3k, 6.6k, 10k,... but since the frequencies 3.3k and 6.6 are not multiples of the input frequency 1k, we will not have zero crossing on it. However, 10k is an integer multiple of fp, so we have a zero crossing at it.

- 40% duty cycle

Time Domain:

Figure 2.1.7: Time Domain for Pulse Train 40% Duty cycle

Frequency Spectrum:

Figure 2.1.8: FFT for Pulse Train 40% Duty cycle

Zero crossing at N multiples of fp/0.4 = 2.5 KHz, so the zero crossings are at  2.5k, 5k, 7.5k,10k,...

The frequencies 2.5k and 7.5 does not have zero crossing value since they are not integer's multiples of fp while 5k and 10 are integer's multiples of fp so they have zero crossing.

- 50% duty cycle

Time Domain:

Figure 2.1.9: Time Domain for Pulse Train 50% Duty cycle

Frequency Spectrum

Figure 2.1.10: FFT for Pulse Train 50% Duty cycle

Zero crossing at N multiples of fp/0.5 = 2KHz, so the zero crossings are at  2k, 4k, 6k,8k,...

All of these frequencies are multiples of fp, so we can see zero crossing every 2kHz.

- 90% duty cycle

Time Domain:

Figure 2.1.11: Time Domain for Pulse Train 90% Duty cycle

Frequency Spectrum:

Figure 2.1.12: FFT for Pulse Train 90% Duty cycle

Zero crossing at N multiples of fp/0.9 = 1.1 KHz, so the zero crossings are at  1.1 k, 2.2 k, 3.3k,10k,...

10 K is the Nth multiple of 1.1k and it is an multiple of fp so we can see a zero crossing value at 10K.

- Why do pulse trains require large transmission bandwidths?

Since the Spectrum of the pulse train is a sinc function that has an infinite bandwidth.

## Part two: Time Characteristics of Pulse Amplitude Modulation (PAM)

At this part, we first establish the connections, then we adjust the sampling frequency to 5000Khz by connecting the CASSY output UB1 to the clock generator of PAM modulation and load the CASSY Lab 2 example pulse frequency5000.labx. We got the following result

Figure 2.2.1: Adjusting the sampling frequency to 5kHz.

- The effect of CH1 filter in PAM Modulation

First, we set the input from the function generator to sine,500Hz and A=10v Vpp

Then we display the time domain of for the input signal before the filter and at the output of this filter. We got the following result

Figure 2.2.2: Input Signal Before and after anti-aliasing Filter

We can see from the previous result that we have only a phase shift for the input. This is because the cut of frequency of this low pass filter is 3.4kHz and the input frequency is 500Hz so the input frequency will pass through this filter. This filter is used as anti-aliasing filter which cuts frequencies higher that a specific frequency to avoid aliasing.

- Display the time characteristic of the PAM.

We took the first signal to be the input PAM Modulator CH1 and the second signal Output PAM2. We got the following result:

Figure 2.2.3: Input and output at the PAM2 Modulator

Then, we took the second signal from output PAM1 and we got the following result

Figure 2.2.4: Input and output at the PAM1 Modulator

- Measure the modulating signal sM(t) and the demodulated signal sD(t) as a function of the duty cycle.

To adjust the duty cycle we connect the first signal to the clock generator then we loaded the CASSY Lab 2 example DutyCycle.labx

- 50% duty cycle

Figure 2.2.5: Adjust duty cycle to 50%

Then we connect the first signal to the input PAM Modulator CH1 and the second signal Output of channel filter CH1 at PAM demodulator. After that, we loaded the CASSY Lab 2 example PAMModDem.labx and we got the following result

Figure 2.2.6: Input Signal and Demodulated PAM Signal at 50% duty cycle

- 30% Duty Cycle

Figure 2.2.7: Adjust duty cycle to 30%

Figure 2.2.8: Input Signal and Demodulated PAM Signal at 30% duty cycle

It appears that the output of the demodulation was the same as the original signal but with phase shift due to the filters and the other components. When we varied the duty cycle of the clock generator to become lower, it helps us to have more signals in Time Division Multiplexing since the ON period will be lesser.

## Part Three: Spectra of Pulse amplitude modulation (PAM)

- The PAM1 spectrum as a function of the frequency of the modulating signal

We  set the sampling frequency to 5Khz and the input signal from the function generator to sine,500Hz and A=10 Vpp. Then we connected the CASSY UA1 to the output PAM1 at PAM modulator and CASSY UB1 to Output of the clock generator. After that we loaded th CASSY Lab 2 example PAMFFT.labs and we got the following result after measurement

Figure 2.3.1: Spectrum of PAM1 when Fc=500Hz

Figure 2.3.2: Spectrum of PAM1 when Fc=1000Hz

From the Previous results, we can conclude that the PAM spectrum deltas appears at the N multiples of  Fp+Fc and Fp-Fc, which is in the first case at 5k+0.5k and 5k-0.5k and in the second case 5k+1k and 5k-1k. However, since the duty cycle was set to 50%, the zero crossing will appear at n multiples of 10K, which explains why we do not see spectrums at these multiples. This result comes from multiply the sinc function with the frequency spectrum for the message signal (Deltas at 0.5 KHz). Moreover, since the sampling frequency is larger than twice of the message frequency in both cases, we did not see any aliasing in the frequency spectrums.

- The PAM1-Spectrum as a function of the duty cycle.

First, we set the function generator to Sine, 1000 Hz, A = 10 Vpp. Then we connected CASSY UA1 to output PAM1 at PAM modulator and CASSY UB1 to clock generator G. Then, we Loaded the CASSY Lab 2 example DutyCycle.labs to set duty cycle to 30%. Finally, we loaded PAMFFT.labs. to start measurement and we got the following result:

Figure 2.3.3: Spectrum of PAM1 when Fc=1000Hz and Duty Cycle 30%

- The PAM2 spectrum as a function of the frequency of the modulating signal.

We set All fP = 5000 Hz , the function generator to Sine, 500 Hz, A = 10 Vpp, CASSY UA1 to output PAM2 at PAM modulator, CASSY UB1 to output of the clock generator then we loaded the CASSY Lab 2 example PAMFFT.labs and got the following results:

Figure 2.3.4: Spectrum of PAM2 when Fc=500Hz

Figure 2.3.5: Spectrum of PAM2 when Fc=1000Hz

Figure 2.3.6: Spectrum of PAM2 when Fc=2000Hz

## Part Four: Subsampling in the Frequency Domain

We set the Function generator: Sine, 3000 Hz, A = 5 Vpp, CASSY UA1 to output PAM1 at the PAM modulator, CASSY UB1 to Clock generator, fP = 5000 Hz and the duty cycle to 20%. Then For display  the spectrum we loaded the CASSY Lab 2 example PAMFFT.labs and we got the following result

Figure 2.4.1: Spectrum of PAM1 when Fc=3000Hz; Aliasing Effect

In this case, Aliasing effect appears since the sampling frequency is 5KHz which is not at least twice the maximum frequency of the message signal (3KHz). Each delta in the spectrum of the pulse train will be shifted by ± 3KHz. They will intersect with each other and aliasing will occur. For example at we have 2 spectrums around 5k which are at 5k+3k (8k) and 5k-3k (2k) and we have 2 spectrums around 10k which are at 10k+3k(13) and 10k-3k(7k) so we can notice the intersect between spectrums at 8k and 7k.

## Part Five: Subsampling in the Time Domain

We Display the modulating signal sM(t) and the demodulated signal sD(t) at subsampling after loading the CASSY Lab 2 example PAMTime.labs

Figure 2.5.1: Input of PAM1 modulator and output of demodulator when Fc=3000Hz; Aliasing Effect

Since we had aliasing effect, the demodulation circuit cannot reconstruct the origin message signal before modulation, so we saw that the message and the demodulated signal are not the same.

## Part Six: PAM time multiplex

We made the following connections

Figure 2.6.1: TDM Connection

- Display the time characteristic of the time multiplex signal.

We set the sampling frequency fP = 5000 Hz, duty cycle maximal, function generator 1: Triangle, fM1 = 200 Hz, A = 5 Vpp, function generator 2: Sine, fM2 = 300 Hz, A = 10 Vpp, CASSY UA1 Input PAM modulator channel CH1, CASSY UB1 Output PAM modulator PAM1 and we loaded the CASSY Lab 2 example PAMTDMInput.labx. The result after taken measurement:

Figure 2.6.2: Modulated signal for TDM vs Sine input

Figure 2.6.3: Modulated signal for TDM vs Triangular input

We can see that the modulated signal is combination between the sine and the triangular signals, this is done by using TDM; two pulse trains when one is on the other is off.

Figure 2.6.4: TDM for Transmit Two Channels; 50% duty cycle for each clock generator

- PAM demodulator time shift Δt left/middle

We connected CASSY UA1 Output PAM demodulator channel CH1, CASSY UB1 Output PAM demodulator channel CH2. Then we loaded the CASSY Lab 2 example PAMTDMOutput1.labx and we got the following result:

Figure 2.6.5: PAM Demodulated Signals from CH1 and CH2

The demodulation technique in PAM is a demultiplexing circuit that contains a switching function s(t) has a period T. It is aligned under the samples from the desired channel. The switch is closed during the time the samples from the desired channel are at its input. Consequently, at the switch output appear only the samples of the desired channel. From these the message can be reconstructed. When changing this switching rate (Δt), the demodulated output will have distortion since it is not as the sampling frequency.

Figure 2.6.5: PAM Demodulated Signals from CH1 and CH2 with High Distortion

# 3. Conclusion

In this experiment, we studied the time and frequency characteristics for the pulse train which is used for PAM modulation. We also studied the two types of PAM modulation (Natural and Top-flat) which were PAM1 and PAM2 in our settings . Then, we studied time and frequency characteristics for PAM modulation and we saw the effect of changing the input frequency and the duty cycle on these domains. We also understood the concept of Nyquist Theory and how to choose a suitable sampling frequency to avoid aliasing and be able to reconstruct the message signal without any distortion.

The second part in this experiment was the concept of Time Division Multiplexing (TDM)  that is used to transmit many signals at the same bandwidth after multiplexing them then the receiver uses a demultiplexer circuit to demodulate the signals.

# 4. References

[1] http://en.wikipedia.org/wiki/Modulation access date 11/6/2016 at 8:20 PM

[2] https://en.wikipedia.org/wiki/Pulse_wave access date 11/6/2016 at 8:24 PM

[3] http://www.electronicshub.org/modulation-and-different-types-of-modulation/#2_Pulse_Amplitude_Modulation_PAM access 11/6/2016 at 8:40 PM

[4] https://en.wikipedia.org/wiki/Nyquist%E2%80%93Shannon_sampling_theorem access 11/6/2016 at 8:55 PM

[5] https://en.wikipedia.org/wiki/Duty_cycle access 11/6/2016 at 9:08 PM

[7] http://s3.amazonaws.com/cramster-resource/104613_Pulse%20amplitude%20modulation(Natural%20Sampling,noise%20performance,Spectrum%20of%20PAM%20signal).pdf  access 11/6/2016 at 9:30 PM

[8] https://en.wikipedia.org/wiki/Sample_and_hold access 11/6/2016 at 9:40 PM

[9] http://whatis.techtarget.com/definition/time-division-multiplexing-TDM access 11/6/2016 at 9:50 PM