Faculty of Engineering and Technology

Department of Electrical and Computer Engineering

ENEE4103 , COMMUNICATIONS LABORATORY, First Semester, 2016-2017

Experiments  #6 + #7: Pulse Code Modulation (PCM)

Report__

Student Name : Sharehan Badwan

Student Id : 1130167

Instructor: Dr. Wael Hashlamoun

Teaching Assistant: Eng. Ahmad Faza’

Section: 3

Date :30/11/2016

1.Abstract:

These two parts of  PCM experiment aim to study quantization and  learn in details

how the quantization convert the signal from analog to digital with different types                    of quantization , how to make  sampling and  encoding .

It’s also aims to understand the different between Linear and non-Linear

Quantization, and use Compressor and Expander in non-linear mode to show there

characteristic. Also, to see the  relation between the number of  bits and resolution

of  quantizer.

Also , we are going to understand the quantization noise and DBCM .

Contents

2.Introduction:	5

3.Procedeure And Discussion:	6

A. Pulse Code Modulation (PCM) – Part 1:	6

3.A.1 Part One: Linear Quantization:	6

3.A.2 Part Two: Non-linear Quantization	7

3.A.3 Part Three: Compressor / Expander characteristic	8

3.A.4 Part Four: Resolution of Quantizer	9

3.A.5 Part Five: Encoding	10

B. Pulse Code Modulation (PCM) – Part 2:	11

Part One: PCM Transmission	11

Part Two: Quantization Noise	12

Part Three: Quantization Noise Variations	15

Part Four: Difference Pulse Code Modulation (DPCM)	19

4.Conclusion :	26

5.References:	27

# 2.Introduction:

## 2.1 Pulse code modulation[1] :

Pulse code modulation (PCM) is a digital representation of an analog signal that takes samples of the amplitude of the analog signal at regular intervals. The sampled analog data is changed to, and then represented by, binary data.

Figure 2.1 block diagram of PCM

## 2.2 Quantization[2]:

Quantization  is a process of converting a range of input values into a smaller set of output values that closely approximates the original data.

If  bits are used to represent the reconstruction values, then there are  levels.

## 2.3 linear Quantization (Uniform)[2] :

In this type , the input signal is distributed uniformly on the defined levels . In this type , all sample sizes are equal to each other.

## 2.4 non-linear quantization(non-uniform)[2]:

In this type , the input signal is distributed non-uniformly on the defined levels . In this type , sample sizes are different.

## 2.5 Resolution:

The number of levels used to send the signal controls the resolution of the sent message since the sample size is given by  X / 2^n , then when increasing n ( number of bits )  , the sample size becomes less , so more levels and more points are taken to represent smaller periods and samples  .

## 2.6 Quantization noise:

When an ADC converts a continuous signal into a discrete digital representation, there is a range of input values that produces the same output, Which is the mid point of the step size (Q) . The difference between input and output is called the quantization error. Therefore, the quantization error can be between ±Q/2.

## 2.7 Encoding:

Encoding is a procedure comes after quantization in the process of converting the analog signal to digital signal , in this operation the levels resulted from quantization are encoded , which means they are set to a certain voltages resulting digital Values .

## 2.8 DPCM[3]:

(DPCM) is a procedure of converting an analog into a digital signal in which an analog signal is sampled and then the difference between the actual sample value and its predicted value (predicted value is based on previous sample or samples) is quantized and then encoded forming a digital value.

Figure 2.8 block diagram of DPCM

# 3.Procedeure And Discussion:

## A. Pulse Code Modulation (PCM) – Part 1:

### 3.A.1 Part One: Linear Quantization:

In this part, the component was assembled as shown in figure 3.A.1

Figure3.A.1  part 1 setup

The PCM modulator and PCM demodulator mode was set to linear mode in each of them .All bits was activated by button SELECT and ON/OFF .At the PCM modulator , the potentiometer for DC voltage was turned slowly in the range [-10,10]. And the cassy lab 2 example Quant.labx  was loaded.The measurements were started by pressing F9.The potentiometer was turned to the right. This was produced an input voltage at the PCM-Modulators (736 101) which is slowly rising from –10 V to +10 V. This input voltage was displays as voltage UA1. The output voltage (after quantization) at the PCM Demodulator (736 111) was displayed as voltage UB1.

The quantization characteristic was recorded as shown in Figure 3.A.2 which shows

the relation between the input voltage of the modulator and the output voltage

of the demodulator.

Figure 3.A.2 quantization characteristic when all bits activated

As shown in the previous figure we noticed that the step size ∆ is too small since we

make the quantization using 8 bits so we have 2^8 levels.

We used high resolution ( 8bits ) such that the step size  =  ( 10 - -10 ) / 2^8  which is

very small sample size . so 2^8 points are quantized through the interval (-10v,10v)

as shown in figure 3.A.3.

Figure 3.A.3 clear snapshot of figure 3.A.2

### 3.A.2 Part Two: Non-linear Quantization

The MODE of the PCM modulator and PCM demodulator was set to non-linear quantization and the measurements were repeated as the previous part.

Figure 3.A.4 Non-Linear Quantization

As we can see from figure 3.A.4, the relation also a linear between the

input and output voltage , since we used a non-linear quantization type

for the both modulation and demodulation

At zero the characteristic is linear, since it took more data points around

zero since compressor has high slope around zero.

### 3.A.3 Part Three: Compressor / Expander characteristic

In order to plot the Compressor / Expander characteristic ,

only one device is operated in the nonlinear mode, while the other device runs in the

linear mode.

First, we set the mode of PCM modulator to Non-linear and the mode of PCM

demodulator to linear.

After that , we get this characteristic as shown in figure 3.A.5.

Figure 3.A.5 Expander characteristic

When the Sender non-linear ,we noticed that we have expander characteristics.

This is used when the message is weak in sender side so we need to amplify it.

Second, we set the mode of PCM modulator to linear and the mode of PCM

demodulator to Non- linear.

After that , we get this characteristic as shown in figure 3.A.6.

Figure 3.A.6 compressor characteristic

When the Receiver non-linear ,we noticed that we have compressor characteristics.

Since we have expansion characteristics in sender side , in receiver side we should compress the message.

### 3.A.4 Part Four: Resolution of Quantizer

In  Part one, the resolution used in quantization is 8bits . In this part , we  reduced  it to 5bits . We did that by Switching off the three LSB .

Figure 3.A.7 (8bits VS 5bits resolution)

When we used 8 bits resolution better than 5 bits resolution .

8 bits gives 2^8 levels and the step size was small ,here we have a better chance to recover the message but when we used 5 bits resolution was reduced so we have 2^5 levels and the step size was increased compared with 8 bits and the chance to recover the message reduced too.

Zero has two representations, -0 and +0.

### 3.A.5 Part Five: Encoding

In this part ,we connected  the components as required in the lab manual. Then, we set the mode of the PCM modulator and PCM demodulator to: PCM, linear quantization. After that, we activate all bits and load the CASSY Lab 2 example Code.labx. We set the potentiometer to maximum left and then we change the voltage from -10 to 10 volts to observe the different output values and the bits corresponding to the input voltage

The binary representation of the different DC input voltages shown in figure 3.A.8.

| DC input voltage | Binary representation |
| --- | --- |
| -9.91 | 01111111 |
| -8.0 | 01100111 |
| -6.01 | 01001110 |
| -4.0 | 00110100 |
| -2.0 | 00011010 |
| 0 | 10000001 |
| 2.0 | 10011001 |
| 4.0 | 10110011 |
| 6.0 | 11001100 |
| 8.0 | 11100110 |
| 10.0 | 11111111 |

Figure 3.A.8 encoding

We noticed that when the DC voltage is positive the most significant bit (MSB)is 1 but when the DC voltage is negative , the MSB is 0.

So the MSB determine the sign and the upper bits are LSB and the lower one are HSB.

## B. Pulse Code Modulation (PCM) – Part 2:

### Part One: PCM Transmission

We assembled the components as shown in figure 3.B.1.

Figure 3.B.1 connection setup

First of all, the components were connected as required in the lab manual.

And  the mode of the PCM modulator and PCM demodulator were set to: PCM, linear quantization and  all bits were activated, we adjusted the sampling frequency to be at the max. Also, we used  two generators like this:

* Function generator 1: Sine, fM1 = 300 Hz, A = 10 Vpp.

* Function generator 2: Triangle, fM2 = 200 Hz, A = 5 Vpp.

we connected the CASSY UA1 with input PAM modulator CH1 and the CASSY UB1 with output PAM Demodulator CH1. After that we Load the CASSY Lab2 PCMTrans.labx.

Figure 3.B.2 shows the Transmission of sine signal.

Figure 3.B.2 transmission of sine signal

Figure 3.B.3 shows the transmission of triangle signal.

Figure3.B.3 transmission of triangle signal

We noticed  from figures(3.B.2 and 3.B.3) the distortion that happens due to the quantization process .

### Part Two: Quantization Noise

We assembled the components as shown in figure 3.B.4.

Figure 3.B.4 connection setup

We connected the CASSY UA1 with input PAM modulator CH2 and the CASSY UB1 with output PCM Demodulator CH1. And the both channels (CH1 and CH2) were connected of the PAM Modulator with the function generator (Triangle, 30 Hz, 12 Vpp). Then the mode of the PCM modulator and PCM demodulator were set  to: PCM, linear quantization, and all bits were activated. After that we load the CASSY Lab 2 example QNoise.labx.

Figure 3.B.5 shows the output.

Figure 3.B.5 Qnoise(8 bits , 30Hz ,triangle)

Figure 3.B.5 shows the Qnoise which is the difference between the original message(triangle) and the quantized signal. When the input > output the difference is positive so the quantization noise or error is positive but when the output > input  ,the quantization error is negative.

The same measurements were  done with resolution of  5bits as shown in figure 3.B.6.

Figure 3.B.6 Qnoise(5bits , 30 Hz ,triangle)

It’s clear that the Quantization noise has increased since the resolution has decreased so error will increased too.

For the same triangle signal but the frequency of the signal set to be 300Hz and note the output as shown in figure 3.B.7.

Figure 3.B.7 Qnoise(5 bits , 300Hz ,triangle)

We note that  when increasing the frequency of the signal , the noise curve value will increase too, from the figure above it reaches about 3v while when the frequency is 30Hz it doesn’t exceed 0.5v  . since when increased frequency the number of sampled points in each period is less.

### Part Three: Quantization Noise Variations

In this part we make the same measurements as the same of part2 but for the sinusoidal signal.

Figure3.B.8 Qnoise(8bits , 30Hz , Sinosoidal)

We note that the Qnoise is sinusoidal wave but with small amplitude which is not exceed 0.5v ; it is sinusoidal since we have two periodical signals(with phase shift) and when we subtract two periodical signal the output will be periodical signal (sinosoidal).

When we changed the frequency from 30Hz to 300Hz we get the following as shown in figure 3.B.9.

Figure 3.B.9 Qnoise(8bits , 300Hz ,sinosoidal)

We note that the Qnoise increased , it was approximately reached 4v.So error is  big.

Then , we changed the resolution to 5 bits and frequency 30Hz and we get the following.

Figure3.B.10 Qnoise(5bits , 30Hz , sinosoidal)

In figure 3.B.11 we saw the Qnoise when we changed the frequency to 300 Hz.

Figure 3.B.11 Qnoise (5bits , 300Hz , sinosoidal)

Qnoise increased due  to reduce number of bits and increased the frequency.

Then , we repeated the same steps of part2 but for non-linear quantization.

For 8bits resolution and frequency of 30Hz we get the following as shown in figure 3.B.12

Figure 3.B12 Qnoise non-linear (8bits , 30Hz , sinosoidal)

We notice a roughness at the peak , this is because more emphasis ( more levels ) on the lower amplitudes by the quantizer .

For 8bits resolution and frequency of 300Hz we get the following as shown in figure 3.B.13

Figure 3.B.13 Qnoise non-linear (8bits , 300Hz , sinosoidal)

For 5bits resolution and frequency of 30Hz we get the following as shown in figure 3.B.14

Figure 3.B.14 Qnoise non-linear (5bits , 30Hz , sinosoidal)

We notice that more levels at the lower amplitudes compare to the higher one , making the lower amplitudes more noticeable as the distortion become less , and the higher amplitudes becomes distorted .

For 5bits resolution and frequency of 300Hz we get the following as shown in figure 3.B.15

Figure 3.B.15 Qnoise non-linear (5bits , 300Hz , sinosoidal)

### Part Four: Difference Pulse Code Modulation (DPCM)

We connected the components as shown in figure 3.B.16.

Figure 3.B.16 connection setup

Then, We switched the power supply ON , and the both of channels (CH1 and CH2) were connected with the function generator(Triangle, 30 Hz, 12 Vpp ) .This avoids time gaps at the output of the PCM demodulator.

The PCM modulator and PCM demodulator modes were set to: PCM, linear

quantization and all bits were activated.

DPCM is a redundancy reducing method. In the predictor the difference to the

previous value is transmitted. At the start of the transmission it is important that the

predictors in the PCM modulator and in the PCM demodulator start from the same prediction value. During switch-on the prediction value is initialized with 0. But since the two systems cannot be switched on simultaneously, the following switch-on sequence has to be adhered to:

1. Connect the PAM input of the PCM modulator to 0 V.

2. Switch the PCM modulator to DPCM mode.

3. Switch the PCM demodulator to the DPCM mode.

4. Disconnect the PAM input of the PCM modulator from 0 V.

5. Drop the amplitude of the modulation signal to 0 V (on the function generator).

6. Feed the modulation signal into the PAM input of the PCM modulator and reset to the

7. desired amplitude

Step 5 has to be performed every time before selecting the ACTIVE BITS.Afterwards

the signal amplitude can be enhanced again.

Then the CASSY Lab 2 example DPCM.labx was loaded. and

The channel UA1 of the CASSY was connected with the input signal of the PAM

modulator .connect with channel UB1 of the CASSY record successively the

following signals:

o PAM input

o Predictor of the DPCM modulator

o Output of the DPCM modulator

o Input of the DPCM demodulator

o Predictor of the DPCM demodulator

o PAM output of the DPCM demodulator

When resolution is 8 bits

First, we connected UB1 to PAM input and we get the following.

Figure 3.B.17 PAM input 8 bits

We noticed that the reconstructed version is almost the same as the original one since there is no noise .

Then, we connected UB1 to Predictor of the DPCM modulator and we get the

following.

Figure 3.B.18 Predictor of the DPCM modulator 8 bits

We noticed that the reconstructed version is almost the same as the original one since

there is no noise.

.

Then, we connected UB1 to the Output of the DPCM modulator  and we get the

following.

Figure 3.B.19 Output of the DPCM modulator 8 bits

Then, we connected UB1 to the Input of the DPCM demodulator and we get the

following.

Figure 3.B.20 Input of the DPCM demodulator 8 bits

We noticed that the amplitude of the input of  DPCM demodulator is very small which is make sense since the sent signals are the difference between the predicted and original which is small value .

Then, we connected UB1 to the Predictor of the DPCM demodulator and we get the

following.

Figure 3.B.21 the Predictor of the DPCM demodulator 8 bits

From figure 3.B.21 we noticed the difference between the original and predicted signal due to the initial state of the demodulator which  is different from that of modulator which makes that shift in the values .

Then, we connected UB1 to the PAM output of the DPCM demodulator and we get

the following.

Figure 3.B.22  PAM output of the DPCM demodulator

Sum of error and predicated sample value = original samples values.

Finally we deactivate the following bits  to get 4 bits resolution.

First, we connected UB1 to PAM input and we get the following.

Figure 3.B.23 PAM input 4 bits

Then , we connected UB1 to the predictor of the DPCM modulator

and we get the following.

Figure 3.B.24 Predictor of the DPCM modulator 4 bits

Then, we connected UB1 to the Output of the DPCM modulator  and we get the

following.

Figure 3.B.25 Output of the DPCM modulator  4 bits

Then, we connected UB1 to the Input of the DPCM demodulator and we get the

following.

Figure 3.B.26 Input of the DPCM demodulator ZOOMED

Then, we connected UB1 to the Predictor of the DPCM demodulator and we get the

following.

Figure 3.B.27 Predictor of the DPCM demodulator

Here , we have phase shift since we have a problem with initialization.

Then, we connected UB1 to the PAM output of the DPCM demodulator and we get

the following.

Figure3.B.28 output of the DPCM demodulator 4 bits

After we get all of the output when we use 4 bit resolution we noticed that there is no

big difference between using 8 bits or 4 bit resolution , since we use DPCM to

reconstruct the signal.

# 4.Conclusion :

In this experiment , we understood the process of Quantization and sampling and how PAM work. We dealt with two types of quantization which are linear and non-linear .

We saw the Qnoise (Quantization noise) and determine the factors that change the noise such resolution while we use a few number of bits resolution will be decreased and the Qnoise increased .However, when the signal frequency increased , the Qnoise increased too.

We used DPCM to recover the signals , and saw how could we use less number of

bits when we use DPCM since it was sent the difference between sample not the

samples itself. So this is reduce the data rate .

# 5.References:

[1] http://www.slideshare.net/vishnudharan11/pulse-code-modulation-pcm

[2] https://analogquantized.wordpress.com

[3]http://slideplayer.com/slide/5259460/