ENEE4103 : communication Lab

Sara sameeh #1130501

Dr. wael Hashlamoun

Teacher assistant: Ahmad faza’

Report for experiments 8&9

Delta Modulation

Section #3

1. Abstract:

in those two experiments we dealt with delta modulation , which is a conversion technique between analog and digital signals, so we can deal easily with digital signals, in addition to that , we understood the difference between LDM and DCDM techniques, also we saw the two type of quantization error which are slope overload and granular noise.

Table of contents :

Abstract …………………….………………………………………..2

Theory ………………….……………………………………………..4

Procedure …………………..………………………………………..7

Conclusion ………………….………………………………………..35

References ………………….………………………………………..36

2. Theory:

2.1 Delta modulation:

A delta modulation (DM or Δ-modulation) is an analog-to-digital and digital-to-analog signal conversion technique used for transmission of voice information where quality is not of primary importance. DM is the simplest form of differential pulse-code modulation (DPCM) where the difference between successive samples are encoded into n-bit data streams. In delta modulation, the transmitted data are reduced to a 1-bit data stream.

2.2 LDM modulation :

Linear DM is a simple type of predictive quantizing system and is essentially a one digit differential pulse code modulation system.

The basic linear DM system consists merely of a two level quantizer and a feedback path containing a single integrator.

The pulse or step is of positive polarity if the error signal or quantize input is positive, and of negative polarity if the error signal is negative. The sequence of binary pulses produced by the quantize is transmitted via the digital channel to the decoder where a replica of the original input signal is reconstructed. The decoder consists of an integrator identical to that in the encoder, and a low pass filter having the same bandwidth as the input signal.

2.3 DCDM modulation:

DCDM is a simple type of adaptive system and is essentially a 3 digit differential pulse code modulation system.

Adaptive delta modulation or [continuously variable slope delta modulation] (CVSD) is a modification of DM in which the step size is not fixed. Rather, when several consecutive bits have the same direction value, the encoder and decoder assume that slope overload is occurring, and the step size becomes progressively larger.

Otherwise, the step size becomes gradually smaller over time. ADM reduces slope error, at the expense of increasing quantizing error.This error can be reduced by using a low-pass filter. ADM provides robust performance in the presence of bit errors meaning error detection and correction are not typically used in an ADM radio design, this allows for a reduction in host processor workload (allowing a low-cost processor to be used)

2.4  Slope Overload Distortions

In delta modulation, the rate of rise and fall of input signal is very high at some time instants. This rapid change of input signal cannot be achieved by staircase signal generated by The  predictor. The step size . is too small for the predictor to follow the rapid changes in input signal. Hence there is large difference between the actual signal and predicted signal. This difference introduces the distortion. It is called slope overload distortion.

2.5 granular noise :

A problem with delta modulation is that the output signal must always either increase by a step, or decrease by a step, and cannot stay at a single value. This means that if the input signal is level, the output signal could potentially be oscillatory. That is, the output signal would appear to be a wave, because it would go up and down regularly. This phenomena is called Granular Noise.

When used in ADCs (Analog to Digital Converters), this problem can be solved by internally adding additional bit(s) of resolution that correspond to the value of Δ. This way, the LSBs (Least significant bits) that were added can be ignored in the final conversion result.

3.Procedure:

Delta modulation – part1

Part One: Prediction signals in Linear delta modulation (LDM)

The components was connected as the previous figure shown , the clock frequency was set to 100 KHZ , the modulating signal with 1v  peek to peek , with 100 hz was set , the bridging plug was set to LDM , the output was the following:

Figure1 LDM modulating signal with prediction signal with Fcl= 100KHZ

Figure2 LDM modulating signal with prediction signal with Fcl= 10KHZ

Notice that the prediction is better when we increased the clock frequency ,that’s mean when we increasing the clock frequency , we take more samples in time , more reading in time , the result of that is better prediction.

Part Two: Prediction signals in Digital Controlled Delta Modulation (DCDM):

The components was connected as the previous figure shown , the clock frequency was set to 100 KHZ , the modulating signal with 1v  peek to peek , with 100 hz was set , the bridging plug was set to DCDM , the output was the following:

Figure3 DCDM modulating signal with prediction signal with Fcl= 100KHZ

Figure4 DCDM modulating signal with prediction signal with FCL= 10KHZ

We notice in this part, the same thing that we conclude in the previous part, when we increasing the clock frequency, the prediction will become better.

Also there is a difference between LDM & DCDM , LDM have a fixed delta , and it’s using one reading for prediction , in the other hand , DCDM have a dynamic delta , and it’s using three reading for prediction , which make DCDM better.

Part Three: Pulse height in LDM

In this part , the clock frequency set to 50KHZ , the modulating signal with  100 HZ and 7 peek to peek , was feed as input , the bridging plug was set to LDM, the input A was connected to the input signal , and the input B was connected to the output of the PAM modulator , the PulseAmplitude.labs example was loaded to CASSY Lab.

Figure5 setting clock frequency to 50KHZ

Figure6 input signal with PAM modulator output LDM

Figure5 show how we use the FFT to set clock frequency to 50KHZ, figure6 show how the delta is fixed in LDM modulation,  when the signal is higher than prediction , the delta is positive , around 2 , when the signal is lower than prediction , the delta is negative, around      -2.

Part Four: Pulse height in DCDM

In this part , the clock frequency set to 50KHZ , the modulating signal with  100 HZ and 7 peek to peek , was feed as input , the bridging plug was set to DCDM, the input A was connected to the input signal , and the input B was connected to the output of the PAM modulator , the PulseAmplitude.labs example was loaded to CASSY Lab, the output was the following:

Figure7 input signal with PAM modulator output DCDM

This part show how the delta in DCDM modulation is dynamic, it goes with signal , which mean that the DCDM is better in prediction than LDM.

Part Five: Output signals of the LDM modulator:

In this part , first we connected the components as shown in the previous figure , the clock generator was set to 30 KHZ , a sine signal with 100HZ , 4 peek to peek  was feed as modulating signal , the bridging plug was set to LDM , input A was connected to modulating signal , input B was connected to bipolar RZ , the LDM_10KHZ .lab was loaded to CASSY LAB, and the result was the following:

Figure8 input signal with bipolar RZ LDM

Notice in this part, when we use

Part Six: Output signals of the DCDM modulator:

In this part, the components was connected as shown in the previous figure , the clock generator was set to 30 KHZ , a sine signal with 100HZ , 4 peek to peek  was feed as modulating signal , the bridging plug was set to LDM , input A was connected to modulating signal , input B was connected to bipolar RZ , the LDM_10KHZ .lab was loaded to CASSY LAB, and the result was the following:

Figure9 input signal with bipolar RZ DCDM

Part Seven: Clock and DM signals in RZ format:

In this part , the component was connected as shown in the previous figure , the clock frequency was set to 10KHZ , a sine wave with 200HZ, 2 peek to peek was feed as a modulating signal , the Bridging plug was set to LDM , input A was connected to clock generator , input B was connected to bipolar RZ , the clock.labs example was loaded to CASSY LAB , the output was the following:

Figure10 clock generator with DM output (bipolar)

Notice in the previous figure, it’s the bipolar type, return to zero RZ.(positive value +5 to half of period , then go back to zero , negative value -5 to half of period , then go back to zero.

Part Eight: Clock and DM signals in NRZ format:

In this part , the component was connected as shown in the previous figure , the clock frequency was set to 10KHZ , a sine wave with 200HZ, 2 peek to peek was feed as a modulating signal , the Bridging plug was set to LDM , input A was connected to clock generator , input B was connected to RZ/NRZ converter , the clock.labs example was loaded to CASSY LAB , the output was the following:

Figure11 clock generator with RZ/NRZ converter

Notice in the previous figure, it’s the unipolar type, NRZ non return to zero.(positive value +5 to half of period , negative value -5 to half of period.

Part Nine: Granular noise in LDM

In this part , the components was connected as shown in the previous figure , the clock frequency was set to 10KHZ(minimum frequency) , a square signal with 50% duty cycle , 200HZ , 2 v peek to peek was feed as modulating signal , the bridging plug was set to LDM , input A was connected to modulating signal , input B was connected to prediction signal X , the example Granularnoise.lab was loaded to CASSY LAB , the clock frequency was changed to 30KHZ & 100KHZ, the output was the following:

Figure12 clock Granular noise in LDM  10KHZ

Figure13 clock Granular noise in LDM  30KHZ

Figure14 clock Granular noise in LDM  100KHZ

Notice in the previous figures (12-14) , the Granular noise at the square wave , the ripple on it , when we increasing the frequency , the Granular effect will reduce.

Figure15 ground input  LDM modulation with 10KHZ

Figure16 ground input  LDM modulation with 30KHZ

Figure17 ground input  LDM modulation with 10KHZ

Notice in the previous figures (15-17) , the noise is less when we rise frequency , also we can’t get a zero (as ground ) , exactly , because each time we need to add delta or to difference a delta.

Part Ten: Granular noise in DCDM

In this part , the components was connected as shown in the previous figure , the clock frequency was set to 10KHZ(minimum frequency) , a square signal with 50% duty cycle , 200HZ , 2 v peek to peek was feed as modulating signal , the bridging plug was set to DCDM , input A was connected to modulating signal , input B was connected to prediction signal X , the example Granularnoise.lab was loaded to CASSY LAB , the clock frequency was changed to 30KHZ & 100KHZ, the output was the following:

Figure17 clock Granular noise in DCDM 10KHZ

Figure18 clock Granular noise in DCDM 100KHZ

Notice in the previous figures (17-18),that the Granular noise is decreased when we increasing the frequency.

Figure19 ground input  DCDM modulation with 30KHZ

We note that we can’t get a zero with prediction , even that we have dc (zero) ground from the input , that’s because predictor need to add delta or subtract delta , so we can’t get exactly to zero, also we note that DCDM is closer to zero than DCDM , because DCDM use three reading from D-FF.

Note : no slope over error , just  granular noise in this case

Part Eleven: Slope overload in LDM

In this part , the components was connected as shown in the above figure , a sine with 100HZ , 7 peek to peek was feed as modulating signal , the bridging plug was set to LDM , input A was connected to modulating signal , input B was connected to prediction signal X ,  the slopeOver example was loaded on CASSY LAB , the function generator was feed by square wave signal which have 50%, 100HZ & 4 peek to peek , the frequency of the square wave signal was increased to 300HZ , the result was the following:

Figure20 Slope Over LDM modulation with Sine input

Notice in the previous figure the slope over error.

Figure21 Slope Over LDM modulation with Square input 100HZ

Figure22 Slope Over LDM modulation with Square input 300HZ

Notice in the previous figures (21-22) the slope over error with input of square wave , when we changed the frequency from 100HZ to 300Hz , it doesn’t affect the error ,in the other hand Granular noise decrease when we increase the frequency.

Part Twelve: Slope overload in DCDM:

In this part , the components was connected as shown in the above figure , a sine with 100HZ , 7 peek to peek was feed as modulating signal , the bridging plug was set to DCDM , input A was connected to modulating signal , input B was connected to prediction signal X ,  the slopeOver example was loaded on CASSY LAB , the function generator was feed by square wave signal which have 50%, 100HZ & 4 peek to peek , the frequency of the square wave signal was increased to 300HZ , the result was the following:

Figure23 Slope Over DCDM modulation with Square input 100HZ

Figure24 Slope Over DCDM modulation with Square input 300HZ

Notice in the previous figures (23-24), the slope over error in DCDM modulation,

Frequency didn’t affect the slope over error as it affect the granular noise , also the slope over reach the peek of the signal , but in LDM it didn’t reach the peek.

Delta Modulation – Part 2

Part One: Dynamic of LDM and DCDM

A) Dynamic of LDM

In this part , the components was connected as shown in the previous figure, the clock frequency was set to 100KHZ(maximum ) , a sine signal with 100HZ was feed as modulating signal , the bridging plug was set to LDM , input A was connected to modulating signal , input B was connected to prediction signal X , the slopeOver.lab example was loaded on CASSY LAB , we kept changing the amplitude so until X demonstrates the beginnings of slope overload (recognizable by the exponential distortion(,  the frequency of the message signal  was changed , and the result was the following:

Figure25 modulating signal and prediction signal LDM at 300HZ

Figure26 Distortion

The amplitude was changed for each frequency until we reach the maximum amplitude without distortion, and the result was the following:

| Frequency | Maximum amplitude without distortion |
| --- | --- |
| 200 | 2 |
| 300 | 1.4 |
| 400 | 1.3 |
| 500 | 0.6 |
| 1000 | 0.5 |
| 2000 | 0.2 |
| 2500 | 0.2 |
| 3000 | 0.2 |

As you see in the above table , when we increasing the frequency , the amplitude decrease.

To calculate D:

D = 20log (0.2/2)

B) Dynamic of DCDM :

the measurement was the same as in the previous part , the only difference that the bridging plug was set to DCDM , the result was the following:

Figure26 modulating signal and prediction signal DCDM at 500HZ

The amplitude was changed for each frequency until we reach the maximum amplitude without distortion, and the result was the following:

| Frequency | Maximum amplitude without distortion |
| --- | --- |
| 200 | 10.2 |
| 300 | 7.7 |
| 400 | 6.1 |
| 500 | 4.6 |
| 1000 | 1.7 |
| 2000 | 0.9 |
| 2500 | 0.4 |
| 3000 | 0.5 |

As you see in the above table , when we increasing the frequency , the amplitude decrease.

To calculate D:

D = 20log (0.5/10.2)

Part Two: Coding and companding

A) LDM: Coding in the DM modulator

In this part , the components was connected as shown in the above figure , the clock frequency was set to 10KHZ(minimum) , a sine with 100HZ , 4 volt peek to peek  was feed as modulating signal , the bridging plug was set to LDM , input A was connected input of the D-FF , input B was connected to bipolar /RZ , the example LDM_Code.labs was loaded to CASSY LAB , the result was the following:

Figure27  Input of DFF with bipolar RZ

Notice from the previous figure , that the first is the comparator result , and the second is the coding of the comparison in bipolar form.

B) DCDM: Companding in the DM modulator

In this part , the components was connected as shown in the above figure , the clock frequency was set to 50KHZ(minimum) , a sine with 100HZ , 15 volt peek to peek  was feed as modulating signal , the bridging plug was set to DCDM , input A was connected prediction signal  , input B was connected to compressor , the example DCDM_Compand.labs was loaded to CASSY LAB , the result was the following:

Figure28  prediction signal with compressor

Notice from the previous figure , the difference between the signal and the predicted , if signal is greater than predicted , it send 1, if signal is less than predicted it send zero.

C) DCDM: Compressor & DM signals

In this part , the components was connected as shown in the above figure , the clock frequency was set to 50KHZ(minimum) , a sine with 100HZ , 15 volt peek to peek  was feed as modulating signal , the bridging plug was set to DCDM , input A was connected DCDM  , input B was connected to compressor , the example DCDM_Compand.labs was loaded to CASSY LAB , the result was the following:

Figure29

From the previous figure , one of them is the sequence of delta in DCDM , and the other one is the compressor , which is sequence of zeros or sequence of ones.

D) DCDM: Recovery of the predicted PAM signals in DM modulator and DM demodulator

In this part , the components was connected as shown in the above figure , the clock frequency was set to 50KHZ , a sine with 100HZ , 15 volt peek to peek  was feed as modulating signal , the bridging plug was set to DCDM in both modulator & demodulator ,input A was connected DCDM (modulator) , input B DCDM (demodulator) , the example DCDM_Compand.labs was loaded to CASSY LAB , the result was the following:

Figure30

The previous figure show the DCDM (modulated) & DCDM (demodulated ), and you can see that they are the same , the recover was success.

Part Three: Demodulation

A) LDM demodulation

In this part , the components was connected as shown in the above figure , the clock frequency was set to 100KHZ(maximum) , a triangular with 300HZ , 4 volt peek to peek  was feed as modulating signal , the bridging plug was set to LDM(for both modulator & demodulator) , input A was connected modulating signal  , input B was connected to demodulated signal , the example DM_dem.labs was loaded to CASSY LAB, the switch was changed to square wave with 10% , the result was the following:

Figure 31 triangular modulated and demodulated signal

Figure 32 square wave modulated and demodulated signal LDM

Notice from the previous figures (31-32) , that there is distortion in the demodulated signal ,

B) DCDM demodulation:

The same connection as the previous part, the only difference that we changed the plug to DCDM, the result was the following:

Figure 33 square wave modulated and demodulated signal DCDM

4.Conclusion:

In those two experiments we learnt about various techniques of delta modulation which are LDM and DCDM (advantages and disadvantages, differences between them according to principles of operation and components of the system).we concluded that if we want accuracy , we should reduce the step size & increase the sampling frequency. And if we want to reduce the slope overload effect we should use DCDM.

5. References:

https://en.wikipedia.org/wiki/Delta_modulation#Adaptive_delta_modulation

https://en.wikipedia.org/wiki/Delta_modulation

https://en.wikibooks.org/wiki/Analog_and_Digital_Conversion/Delta_Modulation

http://www.eng.auburn.edu/~roppeth/courses/TIMS-manuals-r5/TIMS%20Experiment%20Manuals/Student_Text/Vol-D1/D1-13.pdf

https://www.google.ps/url?sa=t&rct=j&q=&esrc=s&source=web&cd=2&cad=rja&uact=8&ved=0ahUKEwjqhfD7yPTQAhXE2CwKHYDGDq0QFggkMAE&url=http%3A%2F%2Farchives.njit.edu%2Fvol01%2Fetd%2F1960s%2F1967%2Fnjit-etd1967-004%2Fnjit-etd1967-004.pdf&usg=AFQjCNHGF3S2fpyeeTrP36IXdkpwF_DOqA&sig2=6yRlQK6sSPI_qvFP_RY79A&bvm=bv.141320020,d.bGg

https://www.scribd.com/doc/126021244/Slope-Overload-Distortion