Electrical and Computer Systems Engineering Department

Communications Lab

ENEE 4103

Report for EXP. No. 10

Noise in Amplitude Modulation

Student Name: Aseel Awad           Student ID: 1130776

Instructor: Wael Hashlamoun        Section.No. 3

Assistant:Eng. Ahmad Fazaa          Date: 12/21/2016

# Abstract

The aim of the experiment is to be introduced to the noise in amplitude modulation. Moreover, getting to know new terms related to noise analysis which are Signal to Noise Ratio (SNR) and Noise Figure. Also, comparing the effect of the noise on DSB-SC modulation and SSB modulation.

Table of Contents

Abstract	2

1. Introduction	4

1.1 Additive White Gaussian Noise	4

1.2 Signal-To-Noise Ratio	5

1.2.1 Pre-detection SNR	5

1.2.2 Post detection SNR	6

1.2.3 Noise Figure	6

2. Procedure and Discussion	7

Part One: Power of Modulated and Demodulated in DSB-SC and SSB Modulation in Time Domain	7

Part two: Power of Modulated and Demodulated Noise in SSB and DSB-SC in Frequency Domain.	10

3. Conclusion	14

4. References	15

# 1. Introduction

In communication systems, noise is an error or undesired random disturbance of a useful information signal in a communication channel. The noise is a summation of unwanted or disturbing energy from natural and sometimes man-made sources [1].

## 1.1 Additive White Gaussian Noise

The noise analysis of communication systems is often based on an idealized noise process called white noise. The power spectral density of white noise is independent of frequency. White noise is analogous to the term white light in the sense that all frequency components are present in equal amounts. We denote the power spectral density of a white noise process W(t) as [2]

Sw(f) = N02

Where the factor 0.5 has been included to indicate that half the power is associated with positive frequencies and half with negative frequencies.

Figure 1: Characteristic of White noise (Power spectral density & Auto-Correlation function)

Since the autocorrelation function is the inverse Fourier transform of the power spectral density, it follows that the autocorrelation of white noise is given by

Additive white Gaussian noise (AWGN) is a basic noise model used to imitate the effect of many random processes that occur in nature. The modifiers denote specific characteristics:

Additive because it is added to any noise that might be essential to the information system. White refers to the idea that it has uniform power across the frequency band for the information system. It is an analogy to the color white which has uniform emissions at all frequencies in the visible spectrum. Gaussian because it has a normal distribution in the time domain with an average time domain value of zero.

## 1.2 Signal-To-Noise Ratio

Signal-To-Noise ratio (SNR) is the measure of quality for analog systems. It is the relation between the power in the original signal and the power in the noise.

The received signal in many communication systems can be modeled as the sum of the desired signal s(t), and a narrowband noise signal n(t) as shown by

X(t) = s(t)+n(t)

Signal-to-noise ratio is defined as the ratio of the power of a signal (meaningful information) and the power of noise (unwanted signal)

The signal-to-noise ratio is clearly measured at the receiver, but there are several points in the receiver where the measurement may be carried out. In fact, measurements at particular points in the receiver have their own particular importance and value.

Figure 2: High-level block diagram of a communications receiver.

For instance, if the signal-to-noise ratio is measured at the front-end of the receiver, then it is usually a measure of the quality of the transmission link and the receiver front-end. But if the signal-to-noise ratio is measured at the output of the receiver, it is a measure of the quality of the recovered information-bearing signal.

### 1.2.1 Pre-detection SNR

At the input to the demodulator: x(t)=s(t)+n(t)

Pre-detection Signal to Noise Ratio (SNR) at the demodulator input (after bandpass filter):

### 1.2.2 Post detection SNR

The output of the demodulator: recovered message with noise. The bandwidth of the output signal is usually the bandwidth of the message signal m(t) , denoted as W. And this is the SNR perceived by the users .

### 1.2.3 Noise Figure

The Noise Figure (NF) is a measure of the degradation of the SNR [3]

F = SNRi/ SNRo

The ideal value of this ratio is 1 and system with maximum NF has the best efficiency since the power received  after the demodulator is close to the power  transmitted before the demodulation (No power loss in transmitter).

# 2. Procedure and Discussion

## Part One: Power of Modulated and Demodulated in DSB-SC and SSB Modulation in Time Domain

In order to be able to find the signal-to-noise ratio for each of DSB-SC & SSB modulation, the concept of superposition technique was used . For each of the modulation techniques, first the power of the modulated signal before the demodulation and without any noise was founded and then the power of the noise after entering it to the modulation and before the demodulation was founded . This was done in order to find the SNR before demodulation. After that, the same thing was done for the noise and the modulated signal before demodulation but this time after the demodulation in order to find the SNR after demodulation.

The first thing was making a simple DSB-SC modulation without any input noise.

The following components were assembled as follow

Figure 2.1 : DSB-SC Modulation and Demodulation Connection

Then, the the CARRIER switch was set to OFF,  a sinusoidal signal with 5 Vpp and f = 2 kHz was set  on the function generator, the controller “NOISE LEVEL” was adjusted to the far left limit, Am modulation type was set to DSB-SC (without passing it to the filter), the Sensor-CASSY 2 inputs were connected as follows:

o Input A → the modulated signal

o Input B → the demodulated signal (tapped at the output of the CF receiver).

Then modulated and demodulated signals  were displayed on the monitor and the result was as follows:

Figure 2.2 : DSB-SC Modulation and Demodulation Signals

From the previous result, the power of the demodulated signal can be determined as Ac2/2. But there was an amplification in the demodulating system by 2.5 . So the maximum amplitude should be divided by 2.5   = (4.69/2.5)2 /2 = 1.76 Watt

To find the power of the demodulated signal when Ac modulation is SSB, a jumper was connected before the filter and the output of modulated signal was taken after the filter. The output of modulated and demodulated signals was as follows:

Figure 2.3 : SSB Modulation and Demodulation Signals

From the previous result, the power of the demodulated signal  in SSB can be determined as Ac2/2  = (1.39/2.5 )2 /2 = 0.15 Watt

The Next Step was to determine the power for the modulated signals in SSB and DSB-SC. Since the DSB-SC modulated signal will not be a sinosoidal signal , it will be hard to find its power in time domain. While in SSB, the modulated signal is sinosoidal signal. Then the power for modulated SSB signal can be calculated as Ac2/2   and the power for modulated DSB-SC is twice the power in modulated SSB signal.

The following figure shows the different between SSB and DSB-SC modulated signals:

Figure 2.4 : SSB and DSB-SC Modulation Signals

From the previous result, the power of the modulated signal  in SSB can be determined as Ac2/2  = (1 v)2 /2 = 0.5 Watt. Thus the power of the modulated signal  in DSB-SC equals to 1 Watt.

| DSB-CS |  |  | SSB |  |
| --- | --- | --- | --- | --- |
|  | Before Receiver |  | After Receiver | Before Receiver | After Receiver |
| Power in modulated signal | 1 | 1.76 |  | 0.5 | 0.15 |

## Part two: Power of Modulated and Demodulated Noise in SSB and DSB-SC in Frequency Domain.

The next step was doing the second part of the superposition which was applying only a noise signal. The value of the noise that was adjusted to -30db. In order to find the noise power before demodulation in DSB-SC, the FFT of the noise signal  was taken and the maximum amplitude of the noise in the bandwidth range of the modulated signal was found. In our case, the input signal was sinusoidal with 2kHz so in DSB-SC, the bandwidth we are interested in as 18KHz to 22KHz since the system  uses a  20KHz carrier. Since the power can be found in the frequency domain by the area under the curve of the squared function, then it can be founded in our case by taking the maximum amplitude in the bandwidth we are interested in and then square it and multiply it with the bandwidth. The following figure represents the FFT of the noise signal with -30db before demodulation.

Figure 2.5 : FFT of the Noise Signal at -30 db.

The maximum amplitude was 0.003V. So the power of the noise signal before demodulation in DSB-SC will be (0.003)2 * (4 K) =  0.036 Watt and the power of the noise signal before demodulation in SSB will be (0.003)2 * (2 K ) =  0.018 Watt .

The last step was finding the noise signal power after passing it to the demodulation part and finding the power. Bandwidth after demodulation will be 3.4 KHz since there is a LPF at the end of the demodulation with cut-off frequency of 3.4 KHz. In DSB-SC demodulation, the noise signal will not be passed to the BPF which exists at the beginning of the demodulator.

The following figure represents the FFT of the noise signal with -30db after DSB-SC demodulation

Figure 2.6 : FFT of the Noise Signal at -30 db after Demodulation in DSB-SC.

Figure 2.7 : FFT of the Noise Signal at -30 db after Demodulation in DSB-SC, Zoomed Output.

The maximum amplitude of the DSB-SC demodulated noise was 0.0045 so the power will be (0.0045)2 * (3.4K) = 0.06885 Watt.

For the SSB modulation, the maximum amplitude of the noise signal after demodulation was 0.004  so the power will be (0.004)2 * (3.4K) = 0.0544 Watt.

Figure 2.8 : FFT of the Noise Signal at -30 db after Demodulation in SSB .

Figure 2.9 : FFT of the Noise Signal at -30 db after Demodulation in SSB, Zoomed output .

The following table summarizes all the previous values we got from applying only noise signal with -30db value.

| DSB-CS |  |  | SSB |  |
| --- | --- | --- | --- | --- |
|  | Before Receiver |  | After Receiver | Before Receiver | After Receiver |
| Power in noise | 0.036 | 0.06885 |  | 0.018 | 0.0544 |

Now,  SNR before demodulation and after demodulation and the ratio between them can be calculated . So SNRPre-detection = P(modulated signal before demodulation)P(noise signal before demodulation)  and it can be used as a measure of the quality of the transmission link & the receiver front-end. While SNRPost-detection = P(modulated signal after demodulation)P(noise signal afterdemodulation)  can be used as a measure of the quality of the recovered information-bearing signal.

| DSB-CS |  |  | SSB |  |
| --- | --- | --- | --- | --- |
| -30db noise | Before Receiver |  | After Receiver | Before Receiver | After Receiver |
| Power in message signal | 1 | 1.76 |  | 0.5 | 0.15 |
| Power in noise | 0.036 | 0.06885 |  | 0.018 | 0.0544 |
| SNRPre-detection | 27.78 |  |  | 27.78 |  |
| SNRPost-detection | 25.56 |  |  | 2.76 |  |
| Noise Figure (N) | 0.92 |  |  | 0.1 |  |

# 3. Conclusion

In this experiment, we learned how a real communication system differs from the previous systems we analyzed before in previous experiments in which that this system takes into consideration the noise cause when transmitting the message signal using the channel and the effect of this noise on the received signal.

Moreover, we became familiar with SNR concept which can be used as a measure of the quality of either the transmission link or the quality of the recovered information. So we became able to know how to calculate these ratios and how to find the power of the signal and the noise.

Finally, we compared the effect of the noise on two different techniques of modulation which are DSB-SC and SSB modulation.

# 4. References

- https://en.wikipedia.org/wiki/Noise_(electronics) access date 12/19/2016 at 10:30 pm

- Introduction to Analog and Digital Communications, Second Edition, By Simon Haykin & Michael Moher

- http://rfic.eecs.berkeley.edu/142/pdf/lect14_noise.pdf  access date 12/19/2016 at 11:10 pm